# Scriptwriter Name: Lorri Marek

# SciEd Collection: Immunology

# Lab Name and JoVE ID #: 10496 ELISA Assays:  Indirect, sandwich, and competitive

- Indirect ELISA

  - The first experiment will demonstrate how to use indirect ELISA to determine the presence of anti-influenza virus antibodies in serum obtained from influenza-infected mice (1-WIDE).  To begin, add 50 L of purified antigen – in this case, 2mg/mL of purified A/PR/8 Influenza A virus – to each well of a 96-well ELISA plate (2-MED).

    - Talent approaches bench containing equipment for the experiment, wearing the appropriate PPE, including a lab coat and gloves. Authors: It is critical to wear the appropriate PPE on your filming day. This video is meant to be an education tool and we should be presenting ‘golden standard’ safety practices.

    - Talent adds 50 L of purified A/PR/8 Influenza A virus to the wells in a 96-well plate. Video editor: Feel free to cut this shot whenever it suits the VO. The talent did the whole plate since it was required for the protocol.

  - Next, cover the plate with an adhesive cover (1-MED-over-the-shoulder) and incubate it overnight at 4 °C to allow the antigen to bind to the plate (2-WIDE-TXT).

    - Talent covers the plate with an adhesive plate cover.  Note: This shot is slated as 2.2.1 and 2.2.2 (also one of the takes can be used for 3.6.2) but I do not see the talent putting the plate in the fridge in the footage. That comes in a separate shot.

    - Talent places the plate inside the refrigerator. TEXT: 4 °C, O/N

  - The following day, remove the coating solution by flicking the plate over a sink (1-MED).

    - Talent flicks the plate over the sink to remove the excess liquid.

  - Next, block the remaining protein-binding sites in the coated wells by adding 200 L of a blocking buffer – here 5% donkey serum in 1X PBS – to each well (1-MED-over-the-shoulder).  Leave the plate to incubate for at least 2 hours at room temperature (2-MED-TXT).

    - Talent pipets 200 L of blocking buffer to each well in the plate.

    - Talent sets a timer for 2 hours, leaving the plate on the bench.  TEXT:  2 h, RT; Alternatively, O/N, 4 °C

  - Following the incubation, remove the blocking buffer (1-MED), and then wash the plate by adding 200 L of 1X PBS containing 1% Tween-20 (2-MED-over-the-shoulder).  Flick the plate over the sink once more to remove the wash (3-MED).

    - Talent removes the blocking buffer by flicking the plate into a sink. Use one of the takes from 3.5.1.

    - Talent pipets 200 L of 1X PBS containing 1% Tween-20 to the wells.

    - Talent flicks the plate to remove the wash. Use one of the takes from 3.5.1.

  - Then, prepare the test samples by adding 460 L PBS to a fresh tube, and then adding 40 L of serum to make a 1:12.5 dilution (1-MED). Then, add 300 l of PBS to a second tube, and then add 100 L of the first dilution (2-CU).  Continue this serial dilution range until obtaining a final sample with the dilution of 1:204,800 (3-MED-TXT).

    - Talent pipets the 40 L of serum into 460 L of PBS.

    - Talent pipets 100 L of the first dilution into a second tube already containing 300 l of PBS. Video editor: This was mis-slated as 2.6.1

    - Talent completes the series of dilutions. Videographer: The tubes in the rack should be in the shot TEXT: 7 1:4-dilutions

  - Add the serially-diluted serum samples, in triplicate to the wells (1-CU-TXT).  Cover the plate with an adhesive cover (2-MED-over-the-shoulder) and incubate at room temperature for an hour (3-MED-TXT).

    - Talent adds diluted serum samples in triplicate to the wells. TEXT: In this example, triplicate wells run across the plate and the dilutions run down the plate.

    - Talent covers the plate with adhesive cover.  Video editor: 2.7.2 and 2.7.3 were filmed in one shot and slated accordingly.

    - Talent sets the timer for 1 hour.  TEXT:  1 h, RT

  - Next, remove the samples by flicking the plate into the sink (1-CU) and then wash the plate by adding 200 L of 1X PBS containing 1% Tween-20 (2-MED-over-the-shoulder).  Once again, flick the plate to remove the wash (3-MED).

    - Talent remove the samples by flicking the plate into a sink. Use one of the takes from 3.5.1

    - Talent pipets 200 L of 1X PBS containing 1% Tween-20 to the wells. See multiple takes from 2.5.2.

    - Talent flicks the plate to remove the wash. Use one of the takes from 3.5.1.

  - Now, add 100 L of an enzyme-conjugated secondary antibody, which in this experiment is a horseradish peroxidase, or HRP, – conjugated donkey anti-mouse secondary, to each well (1-MED).  Incubate the plate for 1 hour at room temperature (2-MED-TXT)… and flick the plate to remove any excess liquid.

    - Talent pipets 100 L of the diluted secondary antibody to each well.

    - Talent sets a timer for 1 hour.  TEXT:  1 h, RT

    - Added shot: Video editor – Use one of the takes from 3.5.1.

  - After incubating with the secondary antibody, remove the secondary antibody by flicking the plate into the sink, wash the plate with 1X PBS containing 1% Tween-20 (1-MED), and then, apply 100 L of the indicator substrate at a concentration of 1 mg/mL to each well (2-CU). Incubate the plate with the substrate for 5-10 minutes at room temperature (3-MED).  In this example, the colorless 3,3’,5,5’-tetramethylbenzidine – or TMB – substrate turns a blue color when AP is present (4-ECU).

    - Talent washes the plate with 1X PBS containing 1% Tween-20. See multiple takes from 2.5.2.

    - Talent pipets 100 L of substrate to each well.

    - Talent sets the timer for 10 minutes.

    - Show detail of the plate to capture the change in color in the wells.

  - After 10 minutes, stop the enzymatic reaction by adding 100 L of 2N Sulfuric acid (1-MED).  The samples will turn a yellow color.

    - Talent adds 100 L 2 N H2SO4.

  - Within 30 minutes of adding the stop solution, insert the plate into a microplate reader (1-WIDE) and read the plate at 405 nm to determine the absorbance of the wells (2-MED).

    - Talent inserts the plate into a microplate reader.

    - (Video editor: Use this shot for 5.4.1. in results) Talent sets the machine to the appropriate wavelength and hits start.

- Sandwich ELISA

  - To begin the sandwich ELISA, the plate must be coated with purified capture antibody (1-WIDE).  To do this, add 100 L of the capture antibody, at a concentration within the 1-10 g/mL range, to each well of a 96-well ELISA plate (2-MED-over-the-shoulder).

    - Talent approaches the bench wearing appropriate PPE, including a lab coat and gloves, carrying reagents needed for the experiment.

    - Talent pipets 100 L of the capture antibody to each well.

  - Next, cover the plate with an adhesive plate cover (1-CU) and then incubate the plate overnight at 4 °C (2-WIDE-TXT).

    - Talent covers the plate with an adhesive plate cover.

    - Talent walks to a refrigerator and places the plate inside.  TEXT:  4oC, O/N Video editor: Use one of the takes from 2.2.2.

  - After the incubation, remove the coating solution by flicking the plate over a sink (1-MED).

    - Talent flicks the plate over the sink to remove the solution. Video editor: Use one of the takes from 2.3.1.

  - Now, block the remaining protein-binding sites in the coated wells by adding 200 L of 5% nonfat dry milk to the wells (1-MED-over-the-shoulder).  Incubate the plate at room temperature for at least 2 hours (2-CU-TXT).

    - Talent pipets 200 L of blocking buffer to the wells.

    - Zoom in on the talent setting a timer for 2 hours.  TEXT:  2 h, RT; Alternatively, O/N, 4 °C

  - Next, remove the blocking buffer (1-MED), and then wash the wells with 1X PBS containing 1% Tween-20 (2-MED-over-the shoulder).   Remove the wash by flicking the plate over the sink (3-MED).

    - Talent flicks the plate over a sink to remove the blocking buffer.

    - Talent pipets 1X PBS containing 1% Tween-20 into the wells. See multiple takes from 2.5.2.

    - Talent flicks the plate over the sink to remove the wash. Video editor: Use one of the takes from 3.5.1.

  - Now, add 100 L of the test sample to the wells (1-MED).  Seal the plate with an adhesive cover (2-MED-over-the-shoulder) and then incubate it at room temperature for 2 hours (3-CU-TXT).

    - Talent pipets 100 L of samples to the wells.

    - Talent seals the plate with an adhesive cover. Video editor: Use footage from 2.2.1. There were several takes to choose from. Alternatively, you can use footage from the combined shot of 3.6.2 and 3.6.3.

    - Zoom in on the talent setting a timer for 2 hours.  TEXT:  2 h, RT; Alternatively, O/N, 4 °C  Video editor: See combined shot of 3.6.2 and 3.6.3.

  - After incubation, remove the samples by flicking the plate over the sink and then wash the wells with 200 L 1X PBS containing 1% Tween-20 (1-MED).  Flick the plate over the sink to remove the wash (2-MED), and then add 100 L of enzyme-conjugated detection antibody to the wells (3-MED-over-the-shoulder).  Seal the plate with an adhesive cover (4-MED). Leave the plate to incubate at room temperature for 2 hours (5-CU-TXT).

    - Talent pipets 200 L 1X PBS containing 1% Tween-20 to the wells. See multiple takes from 2.5.2.

    - Talent flicks the plate over the sink. Use one of the takes from 3.5.1.

    - Talent adds 100 L of the detection antibody to the wells.

    - Talent seals the plate with an adhesive cover. Video editor: Use one of the takes from 3.2.1. Alternatively, use combined shot of 3.7.4 and 3.7.5.

    - Talent sets the timer for 2 hours with the plate on the bench in the background.  TEXT:  2 h, RT Video editor: See combined shot of 3.7.4 and 3.7.5.

  - After the incubation, remove the unbound detection antibody by flicking the plate over a sink (1-MED) and wash the wells with 200 L 1X PBS containing 1% Tween-20 (2-MED-over-the-shoulder).

    - Talent flicks the plate over the sink. Video editor: Use any of the shots of the talent flicking the plate over the sink.

    - Talent adds 200 L 1X PBS containing 1% Tween-20 to the plate. Video editor: Use one of the takes from 2.5.2.

  - Next, add 100 L of the indicator substrate at a concentration of 1 mg/mL (1-MED) and incubate the plate for 5-10 minutes at room temperature (2-MED).

    - Talent adds 100 L of substrate to the wells.

    - Talent sets the timer for 10 minutes.

  - After 10 minutes, stop the enzymatic reaction by adding 100 L 2N sulfuric acid to the wells (1-MED-over-the-shoulder) and then read the plate within 30 minutes of adding the stop solution in a microplate reader (2-WIDE).

    - Talent adds 100 L of 2N H2SO4 to the plate.

    - Talent places the plate in a microplate reader. Videographer – please get two takes – one longer take of the sample running for a little while as the talent watches – this will be used in results

- Competitive ELISA

  - To perform a competitive ELISA, first coat the wells of a 96-well ELISA plate with 100 L of purified antigen at a concentration of 1-10 g/mL (1-MED).  Cover the plate with an adhesive plate cover (2-CU) and then incubate overnight at 4 °C (3-WIDE-TXT).

    - Talent pipets 100 L of purified antigen solution into the wells.

    - Talent wraps the plate with an adhesive plate cover. Video editor: Use one of the takes from 3.2.1.

    - Show the talent approaching a cold room carrying the plate.  TEXT:  4 °C, O/N Video editor: Use one of the takes from 2.2.2.

  - Following this, remove the unbound antigen solution from the wells by flicking the plate over a sink (1-MED).

    - Talent flicks the plate over a sink. Video editor: Use one of the takes of 2.3.1.

  - Next, block the remaining protein-binding sites in the coated wells by adding 200 L of blocking buffer to each well, which can be either 5% non-fat dry milk in PBS (1-MED-over-the-shoulder).  Incubate the plate for at least 2 hours at room temperature (2-CU-TXT).

    - Talent pipets 200 L of 5% non-fat dry milk into the wells. Video editor: You can also use one of the takes from 3.4.1.

    - Talent sets a timer for 2 hours.  TEXT:  2 h, RT; Alternatively, O/N, 4 °C Video editor: Use takes from 3.4.2.

  - While blocking the wells, prepare the antigen-antibody mixture in a 1.5 mL tube by adding 150 L of sample antigen to 150 L of primary antibody for each well in the assay (1-MED-TXT). Incubate this mixture for 1 hour at 37 °C (2-MED-TXT).

    - Talent mixes together the sample antigen with the primary antibody in a 1.5 mL tube. TEXT: In this example, half of the plate is used.

    - Talent places the tubes in a 37oC incubator and sets a timer for 1 hour.  TEXT:  1 h, 37 °C

  - Now, remove the blocking buffer from the wells by flicking the plate over a sink (1-MED).  Then, wash the wells with 1X PBS containing Tween-20 (2-MED-over-the-shoulder) …and then add 100 L of the sample antigen-primary antibody mixture (3-MED).  Leave the plate to incubate at 37 °C for 1 hour (4-WIDE).

    - Talent flicks the plate over a sink to remove the blocking buffer. Use one of the takes from 3.5.1.

    - Talent washes the wells with 1X PBS containing Tween-20. See multiple takes from 2.5.2.

    - Talent pipets 100 L of the sample antigen-antibody mixture to the wells. Video editor: Use take 2.

    - Talent places the plate in an incubator set to 37oC and sets a timer.  TEXT: 1 h, 37 °C

  - Next, remove the sample mixture by flicking the plate over a sink (1-MED), and then wash the wells with 1X PBS containing 1% Tween-20 to remove any unbound antibody (2-MED-over-the-shoulder).

    - Talent flicks the plate over a sink. Use one of the takes from 3.5.1.

    - Talent washes the wells with 1X PBS containing 1% Tween-20. See multiple takes from 2.5.2.

  - Add 100 L of an enzyme conjugated secondary antibody, which in this case is AP-conjugated, to each well (1-MED), and incubate the plate for 1 hour at 37 °C (2-WIDE-TXT).

    - Talent pipets 100 L of the enzyme conjugated secondary antibody to the wells.

    - Talent places the plate in a 37C incubator and sets the timer.  TEXT:  1 h, 37 °C Video editor: Use takes from 4.7.2.

  - Following this, wash the plate with 1X PBS containing 1% Tween-20 (1-MED), and then add 100 L of the substrate solution to each well (2-MED-over-the-shoulder).  Wait for 5-10 minutes (3-CU).

    - Talent washes the plate with 1X PBS containing 1% Tween-20. See multiple takes from 2.5.2.

    - Talent adds 100 L of the substrate solution to each well.

    - Talent sets the timer for 10 minutes.

  - After 10 minutes, stop the enzymatic reaction by adding 100 L 2N sulfuric acid (1-MED) and then measure the absorbance in a microplate reader within 30 minutes of adding the stop solution (2-WIDE).

    - Talent adds 100 mL 2N H2SO4.

    - Talent walks the plate over to a microplate reader.

- Results

  - For the semi-quantitative indirect ELISA assay, the presence of influenza A virus antibodies in serially diluted samples of serum from influenza A-infected mice was determined by reading the absorbance of each well at 405 nm in a plate reader (1-MED-over-the-shoulder).   This raw data is exported to a spreadsheet for calculation purposes (2-MED).

    - Talent removes the plate from the plate reader.

    - Talent sits down at the computer and opens an Excel spreadsheet.

  - In this experiment, the serially diluted serum samples, which range from 1:12.5 to 1:204,800, were repeated in triplicate (1-LM).  To analyze the data, the mean absorbance value is therefore calculated for each set of triplicates by adding all the values for each dilution and dividing the sum by 3 (2-LM).

    - See storyboard: Author Figure 1, highlight the sample rows

    - See storyboard: Author Figure 1, show the equation and highlight the mean absorbance values

  - Once the mean for each set of triplicates is determined, the mean OD450 readings are plotted (1-LM) against the serial dilutions (2-LM).  The OD readings decrease as the serum is diluted, indicating that less antibodies are found in the more diluted samples (3-LM).

    - See storyboard: Author Figure 2, highlight the Y- axis

    - See storyboard: Author Figure 2, highlight the X-axis

    - See storyboard: Author Figure 2, highlight the decreasing plotted black line

  - In the quantitative sandwich ELISA, dilutions of a known standard - in this case, recombinant human TNF - were added to a 96-well plate and read along with the unknown samples (1-MED).

    - Use longer take from 3.10.2 (see note to videographer) – Talent places plate into reader and starts Video editor: There is a shot labeled 5.4.1 that should be used here.

  - To create the standard curve, the mean absorbance value for each set of readings of the known concentrations was calculated (1-LM).  Then, the mean absorbance value is plotted on the y-axis (2-LM) against the known protein concentrations on the x-axis (3-LM). A best fit curve is added through the points in the graph (4-LM).

    - See storyboard: Author Figure 3, highlight the mean absorbance rows.

    - See storyboard: Author Figure 4, highlight the y-axis.

    - See storyboard: Author Figure 4, highlight the x-axis.

    - See storyboard: Author Figure 4, highlight the best fit curve.

  - Once the standard curve is generated, the amount of TNFprotein in the test sample can be determined by first calculating the mean absorbance value for the test sample (1-LM).  In this example, the test samples gave OD450 readings of 0.636 and 0.681 (2-LM).   Adding those values and dividing the sum by 2 gives an average of 0.659 (3-LM).

    - See storyboard: Show author figure 3 and the equation for the mean.

    - See storyboard: Highlight the test sample OD450 readings

    - See storyboard: Input those values into the calculation of the mean equation

  - From the y-axis on the standard curve graph, extend a horizontal line from this absorbance value to the standard curve (1-LM).  At the point of intersection, extend a vertical line to the X-axis and read the corresponding concentration, which in this test sample corresponds to a TNF concentration of 38.72 pg/mL (2-LM-TXT).

    - See storyboard: Author Figure 4, draw the dotted line from the y-axis to the standard curve.

    - See storyboard: Author Figure 4, draw the dotted line from the point of intersection on the standard curve to the x-axis and flash in the concentration text.  TEXT:  OD450 0.6585 = 38.72 pg/mL