# SciEd: Immunology

# Lab: 10502 (Immunoprecipitation)

# Scriptwriter Name: Disharee Nath

- Video Title: Concepts

  - Immunoprecipitation or IP is a widely used technique to isolate a protein of interest from a cell or tissue lysate, or a body fluid (1.1.1), for protein characterization, or to investigate protein-protein interactions (1.1.2).

    - See Storyboard

    - See Storyboard

  - The process begins with an antibody which has a high affinity and specificity for the target protein (1.2.1). This antibody is mixed with the sample, allowing antibody-target complexes to form. Any protein bound to the target protein also gets indirectly attached to the antibody in the process. (1.2.2)

    - See Storyboard

    - See Storyboard

  - Next, the solution is incubated with agarose beads conjugated to a bacterial protein which has a strong affinity for the constant region of antibodies (1.3.1). The bacterial protein binds to the antibody, and connects the antibody-target complexes to the beads (1.3.2).

    - See Storyboard

    - See Storyboard

  - Then, the solution is centrifuged to precipitate the beads, thereby extracting the entire complex containing the binding antibody, the target protein and any interacting proteins. (1.4.1)

    - See Storyboard

  - Finally, the bound proteins are extracted from the beads and released from each other, and are used for further analysis by techniques such as Western Blotting. (1.5.1)

    - See Storyboard

  - Several variations of different parts of this technique are commonly used, like pre-clearing, using peptide tags or magnetic beads, or analyzing other non-protein binding partners. (1.6.1)

    - See Storyboard

  - IP can be preceded by a ‘pre-clearing’ step to remove non-specific antibody-binding proteins in the sample and minimize background (1.7.1). This involves first incubating the sample with a mixed pool of antibodies, allowing them to bind to these proteins (1.7.2), and then using agarose beads to precipitate the complexes. The sample is then ready to proceed to the actual IP. (1.7.3)

    - See Storyboard

    - See Storyboard

    - See Storyboard

  - Peptide tags are useful if a specific antibody is not available for IP. Here, the target protein can be genetically modified to contain a peptide epitope ‘tag’, and an antibody against the tag is able to pull out the protein of interest. (1.8.1)

    - See Storyboard

  - Magnetic beads are often used instead of agarose to precipitate the target (1.9.1). After binding to the antibody-target complex, the sample tube is placed in a strong magnetic field, which extracts the beads from the solution. This eliminates the need for centrifugation and improves speed and convenience. (1.9.2)

    - See Storyboard

    - See Storyboard

  - Immunoprecipitation is also used for studying DNA or RNA binding proteins, and are known as chromatin immunoprecipitation and RNA immunoprecipitation, respectively. (1.10.1)

    - See Storyboard

  - These variations are useful for troubleshooting and adapting the method for different experimental applications. (1.11.1)

    - See Storyboard

  - In this video, you will observe how to pre-clear a cell lysate and perform immunoprecipitation to extract a protein of interest, followed by Western Blot analysis to validate the experiment. (1.12.1)

    - Selected shots from protocol, TBD