SciEd: Current Research

Project ID: 20018

Title:  Two-Photon Microscopy: A Method for Detailed Light Imaging

Scriptwriter Name: Timothy Masiello

| Protocol Project ID: 58788 http://www.jove.com/video/58788?access=jtzkv33q |  |
| --- | --- |
| Asset | Timecode |
| Clip | 3:12 (VO: “Thirty minutes before…”) – 6:14 (VO: “...dot XLS file.”) |

- Title TEXT: “Two-Photon Microscopy: A Method for Detailed Light Imaging”

1.1	With a prepared sample on the microscope, find the area of interest using increasing magnification in brightfield mode.

  - Next, activate fluorescence mode to confirm your staining protocols were effective and your targets are visible.

  - At this point the microscope should be covered with a large cloth to prevent any outside light from reaching the sample.

  - Now, activate multi-photon mode. This changes the excitation from using one visible photon to two near-infrared [pronunciation] photons.

  - Individually, near-infrared photons are lower energy than visible photons. However, two near-infrared photons can combine to produce an equivalent excitation signal to the one visible photon.

  - In addition, when faced with deeper tissues, near-infrared photons penetrate them more efficiently.

  - During microscopy, you can capture still images or take video over an extended period ranging from minutes to hours.

  - Finally, you have many options for image processing. These can include adjusting contrast and brightness or zooming and cropping specific regions of the image.

  - In the example protocol, we will use two-photon microscopy to analyze chemotaxis [pronunciation] , or the attraction of microglia [pronounced: micro-glia (second, short “I” option)] cells towards biochemical agents, in brain tissue slices.

- Title TEXT: “Two-photon Imaging of Microglial Chemotaxis in Brain Slices”