Encyclopedia of Experiments

Project ID: 20067

Project Name:  Light-Sheet Microscopy

Scriptwriter Name: Emanuela Zaharieva

| Protocol Project ID: 59043 |  |
| --- | --- |
| Asset | Timecode |
| Clip | 5:26 (VO: "For a light-sheet fluorescent...") - 7:13 (VO: "... to capture the images.") |

- Overview Title TEXT: “Light Sheet Imaging: A Method for Single Plain Illumination Microscopy”

  - Mount the sample onto the sample holder and place the holder inside the microscope chamber.

  - Turn on the laser from the software. The microscope system transforms the laser into an ultra-thin horizontal plane of illumination referred to as a light sheet.

  - The detection objective is placed perpendicular to the light sheet, decoupling illumination from detection.

  - Thus, the intersection of both illumination and detection paths defines the focal plane from which the fluorescent signal is acquired.

  - Move the focal plane up, to define the top of the sample, and down, to define the bottom.

  - Start the scan to image the entire sample volume in a series of optical sections, or a Z stack [pronounced: ‘Z’ stack].

  - Once acquisition is completed, save the files.

  - In the example protocol we will see light sheet fluorescent microscopy and the software setup used to image a mouse lung lobe.

- Protocol Title TEXT: “Whole Mouse Lung Lobe Imaging by Light Sheet Fluorescent Microscopy”