Collection: Encyclopedia of Experiments

Project ID: 20111

Project Name:  Chemical Drying for SEM

Scriptwriter Name:  Kelley J. Donaghy

| Protocol Project ID: 58761 http://www.jove.com/video/58761?access=uh8edeu2 |  |
| --- | --- |
| Asset | Timecode |
| Clip | 3:56 (VO: “To prepare…”) – 6:46 (VO: “...for at least three hours.”) |

- Overview Title TEXT:  Chemical Drying: Biological Sample Preparation for Scanning Electron Microscopy (SEM)

  - To begin, start by dehydrating and drying a fixed sample to reduce sample distortion caused by the removal of gases and liquid water at the reduced operating pressure of conventional scanning electron microscopes – SEM’s.

  - To dehydrate the sample, wash it with increasing concentrations of ethanol [pronunciation] in water until one hundred percent ethanol is reached.

  - Then, repeat this process with a chemical drying agent in ethanol until the sample is washed with pure drying agent.

  - Transfer the sample and drying agent to an evaporating dish, cover it with minimal additional drying agent, and let it dry in a fume hood overnight.

  - Next, mount the prepared sample on the top of a microscope stub using carbon adhesive tape.

  - Finally, apply silver paint to the edges of the stub, then from the edge of the stub to each of the samples, to reduce the build-up of excess electrons, which could result in undesirable image artifacts.

  - In this experiment, fruit flies - Drosophila melanogaster [pronunciation] - are prepared for SEM analysis using the drying agent– HMDS.

- Protocol Title TEXT:  Chemical Drying of Drosophila melanogaster Using Hexamethyldisilazane (HMDS)

Sources:

- Scanning Electron Microscopes https://en.wikipedia.org/wiki/Scanning_electron_microscope, last accessed, 10/2/19.

- Biological Sample Preparation for SEM.  James Cook University, Australia, https://research.jcu.edu.au/archive/enabling/aac/cairns-aac/biological-sample-preparation-for-the-sem, last accessed 10/2/2019.

- Fischer, E. R., Hansen, B. T., Nair, V., Hoyt, F. H., Dorward, D. W.  “Scanning Electron Microscopy,” Curr. Protoc. Microbiol, 2012.  https://doi.org/10.1002/9780471729259.mc02b02s25.  Last accessed at:  https://www.ncbi.nlm.nih.gov/pmc/articles/PMC3352184/ on 10/2/2019.

- Das Murtey, M., Ramasamy, P., “Sample Preparations for Scanning Electron Microscopy – Life Sciences,”  https://www.intechopen.com/books/modern-electron-microscopy-in-physical-and-life-sciences/sample-preparations-for-scanning-electron-microscopy-life-sciences Last accessed, 10/2/2019.