Fly Aspirator Tube: A Simple Tool to Transfer Live Drosophila

OVERVIEW

Source: Barradale, F., et al. Quantification of Drosophila Grooming Behavior. J. Vis. Exp. (2017).

This video describes one method to build a fly aspirator, which can be used to transfer insects between enclosures without anesthesia.  The aspirator in the example protocol was prepared for an experiment to quantify individual grooming behaviors.

PROTOCOL

- Prepare an aspirator for moving live Drosophila from a culture vial to the grooming chamber. Aspirators allow transfer of conscious animals to the behavioral chambers to ensure that anesthesia does not affect subsequent behavioral observation.

  - Using scissors cut 1.5 feet of tygon tubing ID ⅛", OD ¼". Cut at least 1 inch off of the tip of a 1 mL disposable micropipette tip. Hold a 1 cm square piece of mesh (opening 0.196 inch) over the cut tip.
NOTE: Most 1mL tips have a gradation line where the tip sits in the package box, typically cut to that line.

  - Snugly place a fresh 1 mL micropipette tip over the mesh/cut tip. Observe the mesh form a tight barrier between the two tips. The inside of the new tip creates a holding chamber for flies.

  - Cut the extreme tip of the new micropipette tip to widen the opening enough to allow passage of a single fly. The hole will roughly match the opening of the grooming chamber (approximately 1.5-2 mm in diameter).

  - Snugly fit the nested tips onto the end of the tygon tubing. Push the tips into the tube to ensure a tight fit to allow for vacuum pressure.

  - On the other end of the tubing, cut the tip of a 200 μL micropipette tip and fit the narrow cut end into the tubing. This is the "mouth" side of the aspirator.

MATERIALS

[INSERT MATERIALS TABLE HERE]