## F1FO ATPase vesicle preparation and recordings for ATP synthase activity study.

Authors:

Silvio Sacchetti, Kambiz N. Alavian, Emma Lazrove, Elizabeth A. Jonas.

Authors: institution(s)/affiliation(s) for each author:

Silvio Sacchetti

Department of Internal Medicine

Yale University, New Haven, CT (USA)

silvio.sacchetti@yale.edu

Kambiz N. Alavian

Department of Internal Medicine

Yale University, New Haven, CT (USA)

Kambiz N. Alavian@yale.edu

Emma Lazrove

Department of Internal Medicine

Yale University, New Haven, CT (USA)

emma.lazrove@yale.edu

Elizabeth A. Jonas

Department of Internal Medicine

Yale University, New Haven, CT (USA)

Elizabeth.jonas@yale.edu

Corresponding author:

Silvio Sacchetti

Keywords:

F1FO ATPase, mitochondria, patch clamp electrophysiology, submitochondrial vesicles, Bcl-xL.

Short Abstract:

A method to isolate submitochondrial vesicles enriched in F1FO ATP synthase complexes from rat brain is described. These vesicles allow the study of the activity of  F1FO ATPase complex and its modulation using the technique of patch clamp recording.

Long Abstract:

## Mitochondria are involved in many important cellular functions including metabolism, survival1, development and, calcium signaling2. Two of the most important mitochondrial functions are related to the efficient production of ATP, the energy currency of the cell, by oxidative phosphorylation, and the mediation of signals for programmed cell death3.

## The enzyme primarily responsible for the production of ATP is the F1FO-ATP synthase, also called ATP synthase4-5. In recent years, the role of mitochondria in apoptotic and necrotic cell death has received considerable attention. In apoptotic cell death, BCL-2 family proteins such as Bax enter the mitochondrial outer membrane, oligomerize and permeabilize the outer membrane, releasing pro-apoptotic factors into the cytosol6.  In classic necrotic cell death, such as that produced by ischemia or excitotoxicity in neurons, a large, unregulated increase in matrix calcium contributes to the opening of an inner membrane pore, the mitochondrial permeability transition pore or mPTP. This depolarizes the inner membrane and causes osmotic shifts, contributing to outer membrane rupture, release of pro-apoptotic factors, and metabolic dysfunciton.  Many proteins including Bcl-xL7 interact with F1FO ATP synthase, modulatating its function.  Bcl-xL interacts directly with the beta subunit of F1FO ATP synthase, and this interaction decreases a leak conductance within the F1FO ATPase complex, increasing the net transport of H+ by F1FO during F1FO ATPase activity8 and thereby increasing mitochondrial efficiency. To study the activity and modulation of the ATP synthase, we isolated from rodent brain submitochondrial veiscles (SMVs) containing F1FO ATPase. The SMVs retain the structural and functional integrity of the F1FO ATPase. Here, we describe a method that we have used successfully for the isolation of SMVs from rat brain and the patch clamp technique to analyze channel activity (ion leak conductance) of the SMVs.

Protocol Text:

1) Brain Mitochondrial isolation (adapted from Brown M.R et al.9)

## 1.1) Sacrifice the rat according to established protocols.

## 1.2)  Cut the head of the animal by decapitation, cut the skin and expose the skull.

## 1.3) Open the skull gently by cutting with a scissor or rongeur. Remove the brain.

## 1.4) Mince finely the brain without cerebellum in Isolation Buffer (see table 1) and transfer it to a 5 ml glass/teflon homogenizer (see equipement list).

## 1.5) Homogenize tissue gently 10 times (no bubbles), approximately 5 min.

## 1.6) Centrifuge sample at 1500 ×g for 10 min at 4°C in a bench-top centrifuge.

## 1.7) Save the supernatant (mitochondrial and synaptosomes) and discard the pellet (nuclear material). Centrifuge at 16000 ×g for 10 min at 4°C in a bench-top centrifuge.

## 1.8) Discard supernatant and re-suspend pellet in 500 μl of Isolation Buffer. Disrupt synaptosomes with a cell disruption vessel (see equipment list). Apply a pressure of 1200 psi for 10 minutes, followed by rapid decompression.

## 1.9) Layer the mixture onto Ficoll gradients (see table 2), place in SW-50.1 rotor and centrifuge at 126500 ×g for 20 min at 4°C in an ultracentrifuge (see equipment list). The pellet is the purified mitochondria, the layer between the different densities of Ficoll is undistrupted synaptosomes.

## 1.10) Wash pellet by centrifuging in Isolation Buffer at 16000 ×g for 10 min at 4°C in a bench-top centrifuge.

2) Submitochondrial vesicles (SMV) isolation (adapted from Chan et al.10)

## 2.1) Re-suspend mitochondria in 200 μl Isolation Buffer combined with an equal volume of 1% digitonin and allow to sit on ice for 15 min.

## 2.2) Add more Isolation Buffer and centrifuge at 16000 ×g for 10 min at 4°C in a bench-top centrifuge. Do this twice.

## 2.3) Re-suspend pellet in 200 μl Isolation Buffer and add 2 μl of 10% Lubrol PX (C12E9). Mix and allow to sit on ice for 15 min.

## 2.4) Layer mixture onto Isolation Buffer, place in SW-50.1 rotor and centrifuge at 182000 ×g at 4°C for 1 hour.

## 2.5) Wash final pellet by centrifuging in a desk-top centrifuge in Isolation Buffer at 16000 ×g for 10 min at 4°C.

3) Electrophysiological recording

## 3.1) A typical electrophysiology rig includes an amplifier, a PC computer equipped with a Digidata 1440A analog-to-digital converter interface in conjunction with pClamp10.0 software, manipulators, a microscope, a vibration isolation table, Faraday Cage.

## 3.2) Borosilicate glass capillary tubes are inserted into a Flaming/Brown Micropipette Puller Model P-87. A pipette-puller program is optimized to generate pipettes with resistances between 80 to 100MΩ.

## 3.2) SMVs are placed in a physiological intracellular solution (ATP is added at the appropriate time during the recording) (table 3). Patch clamp pipettes are filled with the same solution (no ATP). Recordings are made by forming a giga-ohm seal onto SMVs at room temperature. Vesicles are visualized by phase-contrast microscopy with a Nikon or Zeiss inverted microscope.

## 3.3) The membrane potential is maintained at voltages ranging from –100 mV to + 100 mV for periods of 10 seconds. Recordings are filtered at 5 kHz using the amplifier circuitry.

## 3.4) Data are analyzed with Clampfit 10.0 software for example to determine frequency and amplitude of single channel events. Voltage dependence is determined by plotting a current voltage relationship.

Represetative Results

An example of a brain-derived submitochondrial vesicle patch recording is shown in Figure 1. The recording demonstrates channel activity modulated by ATP. The control (CTL) recording (left) shows multi-conductance channel activity that was decreased upon addition of ATP to the bath.

Figure 1. Representative patch clamp recording before and after addition of 1 mM ATP. Holding potential is + 70mV. The dashed line represents 0 pA.

Discussion

This method enables the isolation of pure mitochondria at the end of step 1 and submitochondrial vesicles (SMVs) after step 2. SMVs purified by this method are essentially free of contamination by other subcellular organelles and retain their structural and functional integrity prior to freezing. After freezing and thawing, isolated mitochondria or SMVs are used for recordings and biochemical assays, but not for respiratory studies.

Attention should be given to several technical details before starting the isolation. It is crucial to use clean tools and keep them on ice. All procedures should be performed on ice and all centrifuges set to 4°C. Preparing the Ficoll gradient is critical for a successful experiment, in the centrifuge tubes you must have a perfect separation between the two concentrations of Ficoll otherwise is more difficult to obtain pure mitochondria.

Mitochondria or SMVs are aliquoted and stored at -80o C and they are stable for recordings for several months, however, it is better not to re-freeze the mitochondrial sample after thawing. 	Vesicles purified by this method are useful to study the ATP synthase, using different approaches such as patch clamp recording, biochemical techniques and other functional studies. With this preparation one can analyze the effects of drugs or small molecules on F1FO ATPase activity and structure.

Table of specific reagents and equipment:

## Table 1)  Isolation Buffer

|  | ## Final Concentration |
| --- | --- |
| ## Sucrose | ## 250 Mm |
| ## Hepes | ## 20 Mm |
| ## EDTA | ## 1 Mm |
| ## BSA | ## 0,5% |

## Table 2)  Ficoll Stock

| ## Ficoll | ## 22 ml 20% |
| --- | --- |
| ## Sucrose | ## 12 ml 1 M |
| ## EDTA | ## 18.75 μl 0,1 M |
| ## Tris-HCl | ## 375 μl 1 M (ph 7.4) |

| ## Top Layer 7.5% of Ficoll |  |
| --- | --- |
| ## Ficoll stock | ## 10 ml |
| ## Isolation Buffer | ## 6 ml |

| ## Bottom Layer 10% of Ficoll |  |
| --- | --- |
| ## Ficoll stock | ## 15.5 ml |
| ## Isolation Buffer | ## 2.5 ml |

Table 3)  Internal Solution

| ## KCl | ## 120 mM |
| --- | --- |
| ## NaCl | ## 8 mM |
| ## EGTA | ## 0.5 mM |
| ## Hepes | ## 10 mM |
| ## pH | ## 7.3 |

| Name | Company | Catalogue number |
| --- | --- | --- |
| Potter-Elvehjem Tissue Grinder with PTFE Pestle | Krackeler Scientific, Inc. | 1-7725T-5 |
| Eppendorf Centrifuge 5424 | Eppendorf | 5424 000.410 |
| 4639 Cell Disruption Vessel | Parr Instrument Company | 4639 |
| Ficoll | Sigma-Aldrich | F5415 |
| Polycarbonate centrifuge tubes | Beckman Coulter | P20314 |
| SW-50.1 rotor | Beckman Coulter |  |
| L8-70M Ultracentrifuge | Beckman Coulter |  |
| Digitonin | Sigma-Aldrich | D5628 |
| Lubrol PX (C12E9) | Calbiochem | 205534 |
| Axopatch 200B | Axon Instruments |  |
| Digidata 1440A | Molecular Device |  |
| pClamp10.0 | Molecular Device |  |
| Manipulator | Sutter Instrument |  |
| Borosilicate glass capillary | World Precision Instruments | 1308325 |
| Flaming/Brown Micropipette Puller Model P-87 | Sutter Instrument |  |

## References

## 1. Cheng, W.C., Berman, S.B., Ivanovska, I., Jonas, E.A., Lee, S.J., Chen, Y., Kaczmarek, L.K., Pineda, F. & Hardwick, J.M. Mitochondrial factors with dual roles in death and survival. Oncogene 25, 4697-4705 (2006).

## 2.  Duchen, M.R., et al. Mitochondria and calcium in health and disease. Cell Calcium 44, 1-5 (2008)

## 3. Lemasters, J.J. Modulation of mitochondrial membrane permeability in pathogenesis, autophagy and control of metabolism. J Gastroenterol Hepatol 22 Suppl 1, S31-37 (2007).

## 4. Cox GB, Jans DA, Fimmel AL, Gibson F, Hatch L. Hypothesis. The mechanism of ATP synthase. Conformational change by rotation of the beta-subunit. Biochim Biophys Acta 768, 201-208 (1984).

## 5. Cox GB, Fimmel AL, Gibson F, Hatch L. The mechanism of ATP synthase: a reassessment of the functions of the b and a subunits. Biochim Biophys Acta 849, 62-69 (1986).

6.  Cory S, Huang DC, Adams JM. The Bcl-2 family: roles in cell survival and oncogenesis. Oncogene 22, 8590-8607 (2003).

## 7.  Vander Heiden MG, Thompson CB. Bcl-2 proteins: regulators of apoptosis or of mitochondrial homeostasis? Nat Cell Biol 1, E209-216 (1999).

## 8.  Alavian KN, Li H, Collis L, Bonanni L, Zeng L, Sacchetti S, Lazrove E, Nabili P, Flaherty B, Graham M, Chen Y, Messerli SM, Mariggio MA, Rahner C, McNay E, Shore GC, Smith PJS, Hardwick JM and Jonas EA.  Bcl-xL regulates metabolic efficiency of neurons through interaction with the mitochondrial F1FO ATP synthase. Nat Cell Biol. 2011 Sep 18;13(10):1224-33.

## 9. Brown MR, Sullivan PG, Dorenbos KA, Modafferi EA, Geddes JW, Steward O. Nitrogen disruption of synaptoneurosomes: an alternative method to isolate brain mitochondria. Journal of Neuroscience Methods 137 (2004) 299–303

## 10. Chan TL, Greenawalt JW, Pedersen PL. Biochemical and ultrastructural properties of a mitochondrial inner membrane fraction deficient in outer membrane and matrix activities. J Cell Biol. 1970 May;45(2):291-305.