FIBS-enabled non-invasive metabolic profiling

Authors:

Alireza Behjousiar, Karen M Polizzi, Cleo Kontoravdi

Authors: institution(s)/affiliation(s) for each author:

Alireza Behjousiar

Centre for Process Systems Engineering, Department of Chemical Engineering and Chemical Technology

Centre for Synthetic Biology and Innovation, Division of Molecular Biosciences

Imperial College London

a.behjousiar09@imperial.ac.uk

Karen M Polizzi

Centre for Synthetic Biology and Innovation, Division of Molecular Biosciences

Imperial College London

k.polizzi@imperial.ac.uk

Cleo Kontoravdi

Centre for Process Systems Engineering, Department of Chemical Engineering and Chemical Technology

Imperial College London

cleo.kontoravdi@imperial.ac.uk

Corresponding author:  Cleo Kontoravdi

Keywords:   metabolite monitoring, in vivo biosensors, in situ monitoring, mammalian cell culture, bioprocess engineering, medium formulation

Short Abstract: (50 words maximum)

A description of how to calibrate Förster Resonance Energy Transfer integrated biological sensors (FIBS) for in situ metabolic profiling is presented.  The FIBS can be used to estimate intracellular metabolite concentrations non-invasively aiding in the development of metabolic models and high throughput screening of bioprocess conditions.

Long Abstract: (150 words minimum, 400 words maximum)

In the era of computational biology, new high throughput experimental systems are necessary in order to populate and refine models so that they can be validated for predictive purposes.  Ideally such systems would be low volume, which precludes sampling and destructive analyses when time course data are to be obtained.  What is needed is an in situ monitoring tool which can report the necessary information in real-time and non-invasively.

For metabolite monitoring, which also has applications in bioprocess development, medium formulation, and cell line engineering, various methods are available for concentration measurements through the use of enzymatic (e.g. [1]), chemical (e.g. [2]), or binding protein-based assays (e.g. [3]). An interestingoption is the use of fluorescent, protein-based in vivo biological sensors as reporters of the intracellular concentration of key metabolites.  In ultra-low volume assays, fluorescence is a convenient tool as miniaturization actually improves the signal-to-noise ratio [4] and protein-based sensors can be genetically encoded meaning that no exogenous reagents are necessary for metabolite analysis.  One particular class of in vivo biosensors which has found applications in metabolite quantification is based on Fӧrster Resonance Energy Transfer (FRET) between two fluorescent proteins connected by a ligand binding domain (Figure 1).  FRET integrated biological sensors (FIBS) are constitutively produced within the cell line and their spectral characteristics change based on the concentration of metabolite within the cell.  FIBS have fast response times making them ideal for taking measurements for dynamic models. They have previously been used to monitor single (e.g. [5-7]) and multiple [8] metabolites, and provide data on spatiotemporal distribution [9].

In this paper we describe the method for constructing Chinese hamster ovary (CHO) cell lines that constitutively express a FIBS for a metabolite, in this case glucose and glutamine, and calibrating the FIBS in vivo in batch cell culture in order to enable future quantification of intracellular metabolite concentration. We further present data from fed-batch CHO cell cultures, where glucose- or glutamine-rich feed was supplemented, and demonstrate that the FIBS was able in each case to detect the resulting change in the intracellular concentration.  Using the fluorescent signal from the FIBS and the previously constructed calibration curve we were able to accurately determine the intracellular concentration as confirmed by an independent enzymatic assay.

Protocol Text:

1. Cell line revival and maintenance

1.1) Revive CHO cells in 9 mL CD-CHO medium supplemented with 8mM L-Glutamine and 10ml/L 100X hypoxanthine/thymidine supplement (complete growth medium).

1.2) Centrifuge at 800 rpm for 5 minutes.

1.3) Discard the supernatant and resuspend the cells in 10 mL of fresh complete growth medium.

1.4) Remove a 1 mL sample and determine the viable cell concentration by light microscopy using the trypan blue dye exclusion method in an improved Neubauer haemacytometer.

1.5) Initiate a culture at a seeding density of 3x105 cells mL-1 in Erlenmeyer 125 flasks.

1.6) Maintain the culture in an incubator at 37oC, a humidified atmosphere of 5% CO2, on an orbital shaking platform rotating at 120 rpm.

1.7) Subculture every 3-4 days in the complete growth medium at a seeding density of 2x105 cells mL-1.

2. Transfection of the cell line with the plasmid containing the biosensor gene

2.1) Prepare plasmid DNA using a large-scale plasmid purification kit (Maxi prep)

2.2) Maintain cells at the appropriate conditions (37°C, 5% CO2) 12‒24 hours prior to transfection to ensure the cells are actively dividing at the time of transfection.

2.3) Count the cells using the Trypan blue dye exclusion method, then prepare a working volume of 20mL of cell culture in an Erlenmeyer 125 flask at a concentration of 1x106 cells mL-1.

2.4) Use the TransIT-PRO® transfection kit as per the manufacturer’s instructions. The plasmid DNA to transfection reagents ratio will be dependent on the cell line and vector. It is recommended that this is optimized by testing a range of ratios within the range specified by the manufacturer prior to conducting the final transfections. Also maintain at least one negative control culture containing the cells but no DNA.

2.8) Incubate for 4 days in static mode and then transfer to a shaking platform rotating at 125 rpm.

2.9) Add the appropriate antibiotic for plasmid selection to a suitable concentration as determined by a kill curve. In our study, zeocin was added to a final concentration of 400µg mL-1 for selection of transfected cells.

2.10) Change media every three days (adding antibiotic each time) until cells in control well have died.

2.11) Use the most confluent wells to progress to shaking cultures.

2.12) Establish a cell bank by freezing the cells in cryogenic vials containing 107viable cells in 1 mL of freeze mix, which consists of 92.5% growth medium and 7.5% dimethyl sulfoxide.

3. Batch and fed-batch cell growth curve

3.1) Follow instructions for cell maintenance above to establish triplicate cell cultures of the transfected CHO cells in Erlenmeyer 250 flasks with a working volume of 50 mL.

3.2) Maintaining the cultures in a humidified cell incubator at 37oC, with 5% CO2, on an orbital shaking platform rotating at 125 rpm, remove 4.1 mL samples from each growing culture at 24 h intervals.

3.3) Use 100 μL of the sample to determine the viable cell concentration and cell viability with the Trypan blue dye exclusion method.

3.4) Repeat until the viable cell concentration is reduced to zero.

3.5) Repeat for the fed-batch cell cultures supplementing glucose or glutamine on day 6 to bring the concentrations of these metabolites to 36 mM and 4 mM, respectively. It is recommended that additional control cultures are also maintained, which are to be fed with the same volume (12 mL in this case) of pure water instead of nutrients.

4. FRET ratio measurements

4.1) Take 2 mL of the daily samples removed in step 3.2 above and centrifuge at 800 rpm for 5 minutes at 4°C.

4.2) Discard the supernatant and re-suspended the cell pellet in 2 mL of ice cold phosphate buffered saline (PBS). Transfer this into a 6 well plate and add a blank sample containing no cells to one well.

4.3) Take blue and yellow fluorescence measurements immediately using a fluorescence plate reader at an excitation wavelength of 430/35 nm and emission wavelengths of 465/35 nm (blue) and 520/10 nm (yellow).

4.3) Calculate the FRET ratios as the amount of yellow fluorescence detected divided by the amount of blue fluorescence detected after correction for the background fluorescence of the sample containing no cells.

5. Metabolite assays

5.1) Take 2 mL of the daily samples removed in step 3.2 above and centrifuge at 800 rpm for 5 minutes at 4 °C. Remove the supernatant.

5.2) Re-suspend the cell pellet in 3 mL ice cold PBS and centrifuge again at 800 rpm for 5 minutes.

5.3) Remove the supernatant and re-suspend the cell pellet in 3 mL ice cold PBS as above. Sonicate the sample 5 times for 3 minutes each at a pulse of 15 seconds on and 15 seconds off. At this point, the cell extracts may be frozen if desired.

5.4) Use the Amplex® Red glucose/glucose oxidase assay as per the manufacturer’s instructions using a 1:2 dilution of the cell extracts to determine the intracellular glucose concentration. The standards used by the authors ranged between 0 and 100 mM. Using the readings for the standards, construct the standard curve and find the linear equation of best fit using Microsoft Excel (in our case: y = 1310.51x - 723.43 where y is the absorbance reading and x is the glucose concentration). Based on readings for samples, calculate the glucose concentrations using the standard curve and taking the dilution into account. The intracellular glucose concentration can then be calculated using this equation:

[1]

For CHO cells, the single cell volume was calculated using a cell diameter of 12 μM [10].

5.5) Use the EnzyChromTM Glutamine Assay Kit as per the manufacturer’s instructions using a 1:2 dilution of the cell extracts to determine the intracellular glutamine concentration. The standards used by the authors ranged between 0 and 2 mM. As in 5.4 above, construct the calibration curve (in our case y = 203.1x + 17.1 where y is the absorbance reading and x is the glucose concentration) and use it to calculate the glutamine concentrations using the standard curve and taking the dilution into account. The intracellular glutamine concentration can be calculated using equation 1 above.  Note that because the EnzyChromTM kit is a coupled enzymatic assay it is necessary to add an extra set of controls in order to be able to correct for the presence of intracellular glutamate.

Representative Results: An overview of our methodology is presented in Figure 1. In the work presented herein, we transfected CHO cells with the FIBS vector, creating two separate stable cell lines constitutively expressing the glucose and glutamine sensors, respectively. The glucose sensor is based on the Apo-On principle, whereas the glutamine sensor is based on the Apo-Off principle, as illustrated in Figure 2. Batch overgrow cultures of each cell line were maintained taking daily samples for the in vivo calibration of each FIBS. The viable cell growth profile of the two cell lines is shown in Figure 3 and the corresponding FRET measurements and intracellular metabolite concentrations determined by enzymatic assays are presented in Table 2. FRET measurements before day 4 of the cell culture are not reliable because of low cell numbers. Similarly after day 8 the high degree of cell lysis results in a high amount of light scattering. We therefore only used data for days 4 to 8 to construct our calibration curves for the glucose and glutamine FIBS, the equations for which are shown in Table 2.

Following this and to validate our findings, we performed fed-batch overgrow cultures of the two cell lines, a feed containing high substrate concentration (glucose in the case of the glucose FIBS and glutamine in the case of the glutamine FIBS) was supplemented on day 6 of the cell culture to elevate the extracellular concentrations of the substrate. In the case of the glucose-fed culture the extracellular glucose concentration was brought back up to 36 mM, while for the glutamine-fed culture, the glutamine concentration was increased to 4 mM. Figure 4 shows representative results from this study. Figures 4A and 4C depict the FRET ratios on each day, which clearly respond to feeding by reversing the trend they followed up to day 6. Specifically, in Figure 4A, the FRET ratio for the glucose FIBS CHO cell line decreases on day 7 in response to glucose addition, since the glucose FIBS follows the APO-On configuration. Similarly, in Figure 4B, the FRET ratio for the glutamine FIBS CHO cell line decreases in response to glutamine addition, in line with the principle of Apo-Off sensors.

We finally used these FRET measurements to calculate the corresponding intracellular glucose and glutamine concentrations based on the aforementioned calibration curves and the results are shown in Figures 4B for glucose and 4D for glutamine. They are compared to the actual intracellular concentrations of these substrates as determined with the Amplex® Red and EnzyChromTM enzymatic assays and show little deviation from the experimentally obtained values.

Discussion: The FIBS enable in vivo and in situ quantitation of key molecules, in our case growth-limiting nutrients, removing uncertainties arising from quenching and extraction methods. Our findings suggest that the FIBS signal is best correlated with the intracellular concentrations between 1 and 7 mM for glucose and 0.5 and 2 mM for glutamine. These ranges correspond to the exponential, stationary, and early decline phases of the CHO cell culture, which are the most important in terms of elucidating metabolic requirements in a bioprocessing context. Although this methodology can only monitor a limited number of molecules at a time, it can provide useful information for cell line and process development, or even data acquisition for the validation of mathematical models.

A key benefit of the use of FIBS is the ability to obtain online measurements potentially in a continuous fashion. The method therefore lends itself for high-throughput applications which can reduce manual handling and expedite development efforts. With the increased availability of microreactor environments with built-in functionality for fluorescence detection, such as the Biolector system (m2p-labs, Germany), we envisage our FIBS-enabled metabolite profiling platform being used for cell screening, media and process optimization, and process scale-up.

Disclosures:   The authors have nothing to disclose

Acknowledgments:   AB is funded by the BBSRC Targeted Priority Studentships programme.  Both CK and KP are supported by RCUK Fellowships in Biopharmaceutical Processing.  CK also wishes to thank Lonza Biologics for their financial support.  The Centre for Synthetic Biology and Innovation is generously supported by the EPSRC.

References:

- Moser, I., Jobst, G., & Urban, G.A.  Biosensor arrays for simultaneous measurement of glucose, lactate, glutamate, and glutamine. Biosensors & Bioelectronics. 17:297-302 (2002).

- Billingsley K., Balaconis M. K., et al.  Fluorescent nano-optodes for glucose detection.  Analytical Chemistry. 82(9): 3707-3713 (2010).

- Dattelbaum J. D. & Lakowicz, J. R. Optical Determination of Glutamine Using a Genetically Engineered Protein.  Analytical Biochemistry. 291: 88-95 (2001).

- Kfouri, M., Marinov, O., et al. Toward a miniaturized wireless fluorescence-based diagnostic imaging system. IEEE Journal of Selected Topics in Quantum Electronics 14 (1): 226 - 234 (2008).

- Bermejo, C., Haerizadeh, F., et al. Dynamic analysis of cytosolic glucose and ATP levels in yeast using optical sensors. Biochemical Journal 432: 399-406 (2010).

- Yang, H. Y., Bogner, M., et al. H(+)-independent glutamine transport in plant root tips.  Plos One. 5(1) (2010).

- Fehr, M., Takanaga, H., et al. Evidence for high-cavacity bidirectional glucose transport across the endoplasmic reticulum membrane by genetically encoded fluorescence resonance energy transfer nanosensors. Molecular and Cellular Biology. 25(24): 11102-11112 (2005).

- Ai, H. W., Hazelwood, K. L., et al. Fluorescent protein FRET pairs for ratiometric imaging of dual biosensors. Nature Methods. 5(5): 401-403 (2008).

- Ouyang, M. X., Sun, J., et al. Determination of hierarchical relationship of Src and Rac at subcellular locations with FRET biosensors. Proceedings of the National Academy of Sciences of the United States of America. 105(38): 14353-14358 (2008).

- Han, Y., Liu, X. M., et al., Cultivation of recombinant Chinese hamster ovary cells grown as suspended aggregates in stirred vessels. Journal of Bioscience and Bioengineering. 102(5): 430-435 (2006).

Tables and Figures

Table 1: Specific reagents and equipment

Table 2: Results for in vivo calibration of FRET ratio for glucose and glutamine FIBS. The calibration curves are described by the following equations that can be used to make predictions: y= -1.232x + 8.034 for glucose, and y= 1.892x + 0.332 for glutamine, where y is the FRET ratio and x is the concentration in mM.

Figure 1: Overview of the methodology for FIBS calibration and subsequent non-invasive quantification of intracellular glucose and glutamine concentrations.

Figure 2: Principles of the FIBS used in this study. The glucose FIBS (left) follows the Apo-On principle, by which glucose binds to the sensing domain and generates a conformational change that separates the fluorescent proteins, thus decreasing FRET. The glutamine FIBS (right) follows the Apo-Off principle, by which glutamine binds to the sensing domains and generates a conformational shift that brings both fluorescent proteins closer together, thus increasing FRET.

Figure 3: Viable cell concentration profile for batch overgrow cultures of glucose FIBS- and glutamine FIBS-expressing CHO cell lines (n=2 biological repeats with three measurements per cell line).

Figure 4: Fed batch culture of FIBS-producing CHO cells and corresponding FRET measurements. (a) Daily measurements of FRET ratio of fed-batch CHO cell cultures expressing glucose FIBS. The arrow indicates the addition of glucose to the cell culture. (b) Actual intracellular glucose concentration profile measured using the Amplex® Red glucose assay kit against values predicted based on in vivo glucose calibration curve (Table 2). (c) Daily measurements of FRET ratio of fed-batch CHO cell cultures expressing glutamine FIBS. The arrow indicates the addition of glutamine to the cell culture. (d) Actual intracellular glutamine concentration profile measured using the EnzyChromTM glutamine assay kit against values predicted based on in vivo glutamine calibration curve (Table 2). In all cases, n=2 biological repeats with three measurements per cell line.

Table 1

| Name of the reagent | Company | Catalogue number | Comments (optional) |
| --- | --- | --- | --- |
| CHO-S Cells | Life Tecnologies | 11619-012 | Cell line will vary depending on the goals of the study |
| TransIT-PROTM Transfection Reagent | Mirus Bio | MIR 5700 | Transfections can be accomplished using any method suitable for the cell line under study |
| CD-CHO medium | Life Technologies | 10743-011 | Cell growth medium is dependent upon the cells under study |
| 100X HT Supplement | Life Technologies | 11067-030 |  |
| L-Glutamine 200 mM (100X), Liquid | Life Technologies | 25030032 |  |
| InfinitePRO 200 | Tecan | FLx800TBI | Any 96-well fluorescence plate reader that can access the required wavelengths can be substituted |
| Maxiprep Plasmid Purification Kit | Qiagen | 12163 | Any suitable kit can be substituted |
| Amplex Red glucose/glucose oxidase assay kit | Invitrogen | A22189 |  |
| EnzyChrom Glutamine Assay Kit | BioAssay systems | EOAC-100 |  |
| Improved Neubauer haemacytometer | Fisher Scientific | MNK-420-010N |  |
| Incubator | Nuaire | NU-5510E |  |

Table 2

|  | Glucose FIBS |  | Glutamine FIBS |  |
| --- | --- | --- | --- | --- |
| Day | FRET ratio | Intracellular concentration (mM) | FRET ratio | Intracellular concentration (mM) |
| 4 | 1.5 | 5.56 | 3.5 | 2.11 |
| 5 | 3.5 | 3.05 | 2.0 | 1.12 |
| 6 | 5.7 | 2.11 | 1.8 | 0.84 |
| 7 | 6.6 | 1.14 | 0.2 | 0.58 |
| 8 | 7.1 | 1.00 | 1.4 | 0.32 |