# Submission ID #: 50386

# Editor Name: Laifong Lee

# Videographer name:

# Film Date:

# Authors and Affiliations:

Sangmi Jun 1,2, Gongpu Zhao 1, Jiying Ning 1, Gregory A. Gibson 3, Simon C. Watkins 3, Peijun Zhang 1

Sangmi Jun

1Department of Structural Biology

University of Pittsburgh School of Medicine, Pittsburgh, PA 15260, USA

2Present Address: Division of Electron Microscopic Research

Korea Basic Science Institute, Daejeon, Korea (ROK)

smjun@kbsi.re.kr

Gongpu Zhao

1Department of Structural Biology

University of Pittsburgh School of Medicine, Pittsburgh, PA 15260, USA

goz1@pitt.edu

Jiying Ning

1Department of Structural Biology

University of Pittsburgh School of Medicine, Pittsburgh, PA 15260, USA

jin7@pitt.edu

Gregory A. Gibson

3Department of Cell Biology and Physiology

University of Pittsburgh School of Medicine, Pittsburgh, PA 15260, USA

gagst18@pitt.edu

Simon C. Watkins

3Department of Cell Biology and Physiology

University of Pittsburgh School of Medicine, Pittsburgh, PA 15260, USA

swatkins@pitt.edu

Peijun Zhang

1Department of Structural Biology

University of Pittsburgh School of Medicine, Pittsburgh, PA 15260, USA

pez7@pitt.edu

Title: Correlative Microscopy for 3D Structural Analysis of Dynamic Interactions

# Corresponding Author:

Peijun Zhang

pez7@pitt.edu

Tel: 1-412-383-5907; Fax: 1-412-648-8998

Authors, please fill out the brief questionnaire below.

A.  Will you require JoVE to record video microscopy, such as filming a complex dissection or microinjection technique? (Y/N) __N_______ If yes, please list make and model of your microscope: __

B.   Does your protocol include detailed, step-by-step, descriptions of software usage? (Y/N)_Y__Editor’s note: some screen capture files needed. Yes, we add screen capture files below.

C.  Which steps of your protocol will viewers benefit most from having filmed? Please list 4-6 steps_3.3, 4.2, 5.3, 5.5, and 6.2._________________________

D.  What is the single most difficult aspect of this procedure and what do you do to ensure success?  ___6.3, In a cryoEM projection image, at a low magnification, the recognition of the shape and direction of a selected cell and sometimes counting holes on the grid are necessary to locate the correlated region in the EM field. Fluorescent latex beads are added to the sample for a more robust and accurate correlation.

1. Introduction (Schematic Overview and Interview)

A. Schematic Overview (read by voice talent at JoVE):

# Procedural Narrative:

The overall goal of this procedure is to demonstrate the usage of the correlative fluorescent light and cryo-electron microscopy method by imaging dynamic, small HIV-1 particles interacting with host HeLa cells. (Intro)

This is accomplished by first plating and growing HeLa cells on the gold finder EM grid in a glass-bottom culture dish, and infecting HeLa cells with GFP-labeled HIV-1 particles. (P1, the grid emerges out of the pink petri dish)

The second step is to collect time-lapse high-speed 3D live-cell confocal images of HIV-1 infected cells and analyze the images by automated 3D particle tracking for single particle dynamics. (P2, can you add in the images one at a time, starting with image 1 and ending with all nine images on the screen?)

After vitrifying the sample, the same viral particles from the live-cell imaging data are located in cryo-fluorescent images for further high-resolution 3D cryo-electron tomography analysis. (P3)

The final step is to collect tilted cryo-EM projection series for all the identified positions containing HIV-1 particles, and reconstruct 3D tomograms with a weighted back-projection algorithm using IMOD software. (P4)

Ultimately, advanced 3D correlative live-cell fluorescence microscopy and high-resolution cryo-electron tomography are used to directly visualize dynamic, diffraction-limited viral particles and their interactions with host cells. (P5)

Video editor: Schematic Overview graphics are in ‘Graphics_illustration.pptx’

B.  Interview: (Said by you on camera. Don’t forget to smile!)

  - Sangmi Jun: Our work is aimed at developing a correlation method that permits direct visualization of early events of HIV-1 infection by combining live-cell fluorescent light microscopy, cryo-fluorescent microscopy, and cryo-electron tomography.

  - Sangmi Jun: In this manner, live-cell and cryo-fluorescent signals can be used to accurately guide the sampling in cryo-electron tomography. Furthermore, structural information obtained from cryo-electron tomography can be complemented with the dynamic functional data available through live-cell imaging of fluorescent labeled targets.

  - Gongpu Zhao: The main advantage of this technique over existing methods is that an advanced correlative high-speed 3D live-cell imaging with high-resolution 3D cryo-electron tomography approach is applied to study dynamic events that are by nature difficult to capture, such as HIV-1 and host cell interactions at early stages of infection.

# Protocol (read by voice talent at JoVE):

- HeLa cell culture on carbon-coated, gold EM finder grids

  - To prepare the grid for cell culture, glow discharge the carbon-side of a 200-mesh R2/2 Quantifoil gold EM finder grid under 25 mA for 25 seconds (TEXT: Glow discharge at 25 mA; 25 s)

Shots:

    - MED: Talent glow-discharging the gold EM finder grid.

    - LAB MEDIA: Figure2a.tiff

  - Coat the EM grid with fibronectin by floating it, carbon-side down, onto a 40 µl droplet of 50 µg/ml fibronectin solution. Leave it in a tissue culture hood under UV light for 2 hours for disinfection.

Shots:

    - CU: EM grid being floated carbon-side down, onto a 40 µl droplet of 50 µg/ml fibronectin solution.

    - MED: Talent placing the grid in the hood and switching on the UV light.

  - Plate HeLa cells onto the grid at a density of 2 x 104 cells/ml in a glass-bottom culture dish in DMEM with appropriate supplements (TEXT: 2 x 104 cells/ml in DMEM + 4.5 g/l L-glutamine and glucose, 10 % heat-inactivated fetal calf serum, 100 units/ml penicillin, 100 µg/ml streptomycin).

Shots:

    - CU: HeLa cells being plated onto the grid in a glass-bottom culture dish.

  - Incubate cells at 37 °C with 5 % CO2 for approximately 18 hours before HIV-1 infection. (TEXT: 37 °C; 5 % CO2; 18 h)

Shots:

    - MED: Multiple takes from different angles of talent putting the dish into the incubator.  Shot will be repeated later.

- HIV-1 infection and live-cell imaging (comment: Section 3 was filmed on 02-06-13, some of shots were repeated from previous session )

  - Prior to HIV infection, add a fluorescent cell tracker to the HeLa cells in the glass-bottom culture dish and incubate the dish at 37°C for 10 minutes to allow up-take of the fluorescent dye.

# Shots:

    - MED: Talent adding Red CMTPX to the cells in the glass-bottom culture dish. .(Repeat, use 02-06-13 shots)

    - Use shot from 2.4.1. (This may have been refilmed on 2-6-13, the notes say take 1,2, and 3)

  - Wash the cells with PBS and add 50 µl of pre-warmed fresh medium.

# Shots:

    - MED: Talent adding PBS to dish and then aspirating PBS. (Repeated, use 02-06-13 shots)

    - CU: 50 µl of pre-warmed fresh medium being added to cells. (Repeated, use 02-06-13 shots).

  - Place the dish into the live-cell chamber, at 37 °C, of a Swept Field Confocal scanner microscope. Since cryo-EM requires relatively thin regions of the sample, select multiple fields that contain 1-3 cells per square, with the cells between two mitosis phases when they are most spread-out and flat (show Figure 2b). Store these positions with NIS-Elements software for future imaging.

# Shots:

    - MED: Talent putting the dish into the live-cell chamber of the confocal scope.

    - MED: Talent viewing the cells through the screen.

    - LAB MEDIA: Figure2b.tiff

    - MED/over the shoulder: Talent using NIS-Elements software to store the positions.

  - Next infect the cells with VSV-G pseudo-typed HIV-1 particles that contain GFP-Vpr. Add the virus particles into the bottom well of the dish, being careful not to disturb the EM grid, since the positions for imaging have already been selected and stored.

# Shots:

    - MED: Talent about to add the virus particles to the cells.

    - CU: Match action above: Virus particles being carefully added into bottom well without disturbing EM grid.

  - Immediately after addition of virions, collect time-lapse high-speed 3D confocal images, at the previously selected positions for 20-40 minutes.

# Shots:

    - MED/over the shoulder: Talent starting image acquisition with NIS-Elements software.

- Frozen-hydrated EM sample preparation

  - Immediately after confocal live-cell imaging, place the culture dish onto an ice-cooled copper block and transfer it to the cryo-EM sample preparation room.

Shots:

    - MED: Talent placing culture dish onto an ice-cooled copper block.

    - WIDE: Talent taking the culture dish to the cryoEM sample preparation room.

  - Load the EM grid onto the specialized tweezers. Quickly blot away residual culture medium with filter paper. Immediately place onto the grid 4 µl of a 15-nm gold bead solution mixed with 0.2 μm fluorescent microspheres. The fluorescent microspheres are used to aid correlation between fluorescent and cryo-EM images (show Figure 2 e & f).

Shots:

    - CU: EM grid being loaded onto specialized tweezers.

    - CU: Residual culture medium being blotted away with filter paper.

    - CU/ECU: 4 µl of a 15-nm gold bead solution mixed with 0.2 μm fluorescent microspheres being placed onto the grid.

    - LAB MEDIA: Figure2e&f.tiff

  - Load the tweezers onto the vitrification device for plunge-freezing with the blotting parameters optimized to achieve the best results (TEXT: blot time, 7s; blot offset, 0; drain time, 1s; temperature, 22 °C; humidity, 100 %).

Shots:

    - MED: Talent loading tweezers onto the vitrification device.

    - CU: Sample being vitrified.

- Cryo-fluorescence light microscopy

  - To begin the procedure for cryo-fluorescence light microscopy, connect a dry nitrogen gas line to a sleeve placed over the objective lens to keep the lens warm and free of frost.

Shots:

    - Talent connecting a dry nitrogen gas line to a sleeve placed over the objective lens

  - Mount a homebuilt cryo-fluorescence sample stage onto an inverted fluorescence light microscope. Connect the liquid nitrogen inlet of the cryo-sample stage to the self-pressurized dewar and place the liquid nitrogen overflow protection outlet into an appropriate container.

Shots:

    - MED:Talent mounting homebuilt cryo-fluorescence sample stage onto an inverted fluorescence light microscope

    - MED: Talent connecting liquid nitrogen inlet of the cryo-sample stage to the self-pressurized dewar and then placing the liquid nitrogen overflow protection outlet into an appropriate container.

    - WID shot of 5.2.2.

  - Place the frozen-hydrated sample grid into the EM specimen cartridge on the copper block and place a pre-cooled C-clip ring on top to keep the grid in place.

Shots:

    - CU: frozen-hydrated sample grid being placed into the EM specimen cartridge on the copper block.0

    - CU: pre-cooled C-clip ring being placed on top of the grid (Videographer: please include footage of the cartridge with the copper ring correctly in place.) (Comment: So small, can not see it. HR figure available in paper) (C-clip ring was used instead)

  - Place the cartridge in the inner chamber of the cryo-stage.

Shots:

    - CU: Footage of the empty inner chamber of the cryo-stage and then the cartridge being placed into the inner chamber.

  - Search and find the same virus particles from the live-cell imaging data. Since the EM grid has an index, it is straightforward to localize the same particle on the grid in both live-cell images and cryo-florescence images.

Shots:

    - MED: Talent at the microscope, looking for the virus particles.

    - LAB MEDIA: cryo_FLM_HIV_Step5.5.pptx

  - Acquire cryo-DIC and fluorescence images at the identified positions under cryo-condition using a light microscope with a long working objective lens (TEXT: 2.7 mm-4 mm objective lens).

Shots:

    - MED/over the shoulder: talent acquiring images.

  - During the cryo-fluorescence imaging, periodically check the liquid nitrogen level in the cryo-stage and refill it with a self-pressurized dewar, as needed, to keep the sample stage below -170 C.

Shots:

    - MED: Talent refilling the liquid nitrogen in the cryo-stage with a self-pressurized dewar.

- Cryo-electron tomography

  - For cryo-electron tomography, load the sample grid into the cryo-transfer station of an electron microscope equipped with a field emission gun.

# Shots:

    - MED: Talent loading sample grid into the cryo-transfer station of an electron microscope.

    - CU: Match action above: sample being loaded into cryo-transfer station.

  - Under low-dose-search mode at a magnification of 140x, identify the regions or grid squares where cryo-fluorescence images have been acquired. Record a low magnification cryoEM image of the correlated area (show Figure 2c&d) and save the position into a stage file.

# Shots:

    - MED: Talent at the microscope searching for regions/grid squares.

    - LAB MEDIA: Figure2c&d.tiff

    - MED/over the shoulder: Talent saving the position into a stage file.

  - Under low-dose-search mode at a magnification of 3,500x, insert a 100 μm objective aperture, search, and save all the positions correlated with GFP signals into a second stage file.

# Shots:

    - MED: Talent selecting 3,500x magnification, inserting 100 μm objective aperture, and searching.

    - MED/over the shoulder: Talent saving the positions into a second stage file.

  - Set up the tomography parameters for acquiring a tilt-series, including tilting angle range, electron dose, defocus value, and tilting schemes. [TEXT: tilt range = -70 to 70; tilt intervals = 3 (<30) and 2 (>30); dose = 1-2 e-/Å2; defocus = 8µm]

# Shots:

    - MED/over the shoulders: talent setting up the tomography parameters for acquiring a tilt-series.

  - Acquire tilt-series for all the saved positions.

# Shots:

    - MED: Talent acquiring tilt-series.

- Three-dimensional reconstruction using IMOD

# Video editor: author has submitted screen capture file ‘JOVE_correlative_microscopy_Step 7_screen.mp4’

  - To begin this procedure, configure the main window such that several stages of the tomogram computation can be adjusted or followed simultaneously.

Shots:

    - MED: Talent at the computer.

    - SCREEN: Main window being configured.

  - Modify the necessary parameters and execute the specific programs that are required by each processing step (TEXT: eTomo tutorial available at http://bio3d.colorado.edu/imod/doc/etomoTutorial.html)

Shots:

    - SCREEN: Parameters being modified.

    - SCREEN: Specific programs being executed.

  - At the final stage, create a full aligned stack with a mean residual error less than 0.6, and reconstruct tomograms using a weighted back-projection algorithm in IMOD.

Shots:

    - SCREEN: A full aligned stack being created.

    - SCREEN: Tomograms being reconstructed.

- Results: observing dynamic behavior of small HIV-1 particles interacting with host HeLa cells

  - To characterize the dynamic behavior of virus particles, HeLa cells infected with HIV-1 were imaged by high-speed confocal live-cell microscopy and the particle movements were analyzed by automated 3D particle tracking (Figure 1). In this representative figure, a single green-fluorescent viral particle, indicated by the yellow arrowhead, was tracked in 3D confocal stacks with a 3-minute time interval between frames.

Shots:

    - LAB MEDIA: Figure1.tiff

  - To avoid the time lapse of several minutes that can occur between collection of the last confocal live-cell image and plunge-freezing, a cryo-fluorescence light microscopy stage (Figure 3) was designed and constructed for imaging frozen-hydrated samples, within cryoEM cartridges, on light microscopes.

Shots:

    - LAB MEDIA: Figure3.tiff

  - Panel a shows a self-pressurized dewar, filled with liquid nitrogen that is used to cool down the copper cryo-sample stage (Video editor: use the white arrow to indicate the cryo-sample stage).

Shots:

    - LAB MEDIA: Figure3.tiff

  - Panel b shows the top, inside view of the cryo-sample stage, with the inner chamber indicated by the white arrowhead. The sample grid is placed onto the EM specimen cartridge, which sits in the center of a copper block platform, indicated by the black arrow. This platform was added for use with non-Polara electron microscopes. Once loaded with the sample grid, the cartridge is transferred to the inner chamber for cryo-fluorescence light microscopy.

Shots:

    - LAB MEDIA: Figure3.tiff

  - Panels c and d show, respectively, the specimen cartridge before (c) and after (d) placing a copper ring (Video editor: use black arrowhead to indicate the copper ring) to keep the grid in place for use with a non-Polara cryo-electron microscope.

Shots:

    - LAB MEDIA: Figure3.tiff

  - The utility of the procedure for advanced correlative live-cell microscopy and cryo-electron tomography is demonstrated with direct visualization of fluorescently labeled HIV-1 particles interacting with a host HeLa cell (Figure 4). A DIC image recorded with a cryo-light microscopy stage, shown in panel a, is overlaid with a cryo-fluorescence image of GFP-tagged particles, shown in panel b. The tagged particles are colored red.

Shots:

    - LAB MEDIA: Figure4a&b.tiff

  - Panels c, d and e are low dose cryoEM images of the region containing a fluorescent particle circled in panels b and c at 140x (c), 3,500x (d) and 27,500x (e), respectively. The inset in panel e is an enlarged view recorded after acquisition of the tomographic tilt series.

Shots:

    - LAB MEDIA: Figure4cd&e.tiff

  - Panels f, g and h show three 4-nm-thick tomographic slices separated by a distance of 21 nm in the z direction. Connections between the particle and HeLa cell membrane are indicated by arrows in both the projection image in the inset in panel e, and tomographic slices in panels f and g.

Shots:

    - LAB MEDIA: Figure4fg&h.tiff

- Conclusion (said by authors on camera)

  - Sangmi Jun: We have presented a straightforward set of protocols to provide an advanced correlative approach to analyze dynamic virus-cell interactions using time-lapse confocal live-cell fluorescence imaging followed by cryo-electron tomography.

  - Gongpu Zhao: While attempting this procedure, it’s important to remember that precise and reliable correlation between the fluorescence image and the electron microscopy image is critical to successful acquisition of cryo-electron tomography data. The procedure described will be useful for numerous applications involving dynamic cellular processes in cell biology, such as cell signaling, surface receptor internalization.

# Provided Media

# 1A. Graphics_illustration.pptx

# 2.1. Figure2a.tiff

# 3.3. Figure2b.tiff

# 4.2. Figure2e&f.tiff

# 5.5. cryo_FLM_HIV_Step5.5.pptx

# 6.2. Figure2c&d.tiff

# 8.1. Figure1.tiff

# 8.2-8.5. Figure3.tiff

# 8.6. Figure4a&b.tif

# 8.7. Figure4cd&e.tif

# 8.8. Figure4fg&h.tif

# General Preparation

# It’s critical for a smooth and organized shoot that all reagents are accounted for, in advance.

Any overnight or long incubation steps should be recognized and specimens/samples be prepared in advance so that prior steps can be recorded and shooting can continue with pre-prepared specimens/samples.

# All tubes/flasks should be pre-labeled neatly before we arrive.

Ex. Luciferase assay done in 96 well plates should be labeled with negative/positive control wells and experimental samples are labeled accordingly.

You will receive more detailed preparation instructions, as well as an introduction to your videographer, closer to your filming date.