//MAIN PROGRAM

main(){

Read_ConfigurationFile()	//Determines type and number of hardware elements

Initialize_Hardware()		//Prepare GUI according hardware that is present

Process_Events()		//Execute commands triggered by controller or interface

//as in an infinite loop

Close()				//Free allocated resources

}

//INITIALIZATION PROCEDURE

initialize_hardware(){

//Some hardware requires a connection first

Connect_Joystick()		//Connect to controller and start state polling loop

Connect_Manipulators()	//Connect to manipulators and start position readout loop

Connect_PressureController()	//Connect to pressure controller and start readout loop

Connect_Amplifiers()		//Connect to amplifiers

Connect_VideoAcquisition()	//Connect to video acquisition card

//Some hardware requires setting initial values properly

Initialize_PressureController()	//Adjust settings for pressure controller operation

Initialize_Amplifiers()		//Set amplifiers to initial configuration VC, no holding

Initialize_Oscilloscopes()	//Adjust oscilloscope signal scale and position

Initialize_VideoAcquisition()	//Start video passthru operation with overlay loop

}

//PRESSURE CONTROL LOOP

PressuretTimer_Tick(){			//Executed at regular intervals

if( pressureSystemConnected AND pressureControlEnabled){

if( positiveBufferPressure < reference_value){

CLOSE allValves

OPEN positivePumpValve

WAIT 200ms

CLOSE allValves

}

if( negativeBufferPressure > -reference_value){

CLOSE allValves

OPEN negativePumpValve

WAIT 200ms

CLOSE allValves

}

for ( i = 1 : numberOfChannels ){

READ pressureValue

if ( channel[i].pressure < reference_value ){

CLOSE allValves

OPEN positiveBufferValve

OPEN channel[i].valve

WAIT 200ms

CLOSE allValves

}

}

}

}

//VIDEO DISPLAY LOOP

VideoTimer_tick(){

if( videoConnected AND passthruStarted ){

Create_graphics()

if(passthrumode & 128)	DrawText()

if(passthrumode & 64)	DrawTip()

if(passthrumode & 32)	DrawCoordinates()

if(passthrumode & 8)	DrawImage()

if(passthrumode & 4)	DrawTraces()

if(passthrumode & 2)	DrawCells()

if(passthrumode & 1)	DrawScale()

Overlay_graphics()		//draw elements overlaid on video

}

}

//JOYSTICK COMMANDS LOOP

ControllerTimer_Tick(){

joystickPreviousState = joystickState

Acquire_JoystickState()

buttonflag = 0

if( ButtonPressed(BUTTON_A)){

buttonflag = 1

if( ButtonRaised(BUTTON_R1)){

Microscope.Recede()

//Go to position [0,0,7000] in Microscope Coordinates

}

if( ButtonRaised(BUTTON_R2)){

Microscope.GoToSearchPosition()

//Go to position [0,0,2000] in Microscope Coordinates

}

if( ButtonRaised(BUTTON_L2)){

Pipette[selected].GoToSearchPosition()

//Go to position [0,0,2000] in Microscope Coordinates

}

if( ButtonRaised(BUTTON_L1)){

Pipette[selected].GoToSafePosition()

//Go to position [0,0,0] in Microscope Coordinates receding x-axis 6mm

}

}

if( ButtonPressed(BUTTON_C)){

buttonflag = 3

if( ButtonRaised(BUTTON_Z)){

//buttons C + Z

Pipette[selected].QueryPosition()

Pipette[selected].SaveFoundPosition()

Pipette[selected].SaveTipImage()

Pipette[selected].AdjustOffset()

}

if( ButtonRaised(BUTTON_Y)){

//buttons C + Y

Pipette[selected].SaveAssignedCellPosition()

}

if(ButtonRaised(BUTTON_L1)){

//Move pipette to the centre of the screen

Pipette[selected].MoveToCentreOfScreen()

}

if(ButtonRaised(BUTTON_L2)){

//Approach Pipette towards cell , microscope follows

Pipette[selected].ApproachCell()

}

if(ButtonRaised(BUTTON_R2)){

//Move microscope to active cell

Microscope.MoveToCell()

}

if(ButtonRaised(BUTTON_R1)){

//Move microscope to active pipette

Microscope.MoveToPipette(selected)

}

if(ButtonRaised(BUTTON_B)){

//Buttons C + B ->Locate tip on screen

Pipette[selected].LocateOnScreen()

}

if(ButtonRaised(BUTTON_B)){

buttonflag = 2

for(i = 1:numberOfPipettes){

Pipettes[i].Stop();

}

}

if(ButtonPressed(BUTTON_Z)){

buttonflag = 6;

if(ButtonRaised(BUTTON_START)){

Pipette[selected].TogglePressureControl();

}

if(ButtonRaised(BUTTON_X)){

Pipette[selected].ReleasePressure();

}

if(ButtonRaised(BUTTON_Y)){

Pipette[selected].ApplyNegativePressure();

}

if(ButtonRaised(BUTTON_X)){

buttonflag = 4

if(Pipette[selected].GetMode() != VC){

Pipette[selected].SetMode(VC)

Pipette[selected].SetOscilloscopeScale(0.5);

if(Pipette[selected].GetHoldingEnable() == TRUE){

Pipette[selected].SetOscilloscopeCoupling(AC)

}else{

Pipette[selected].SetOscilloscopeCoupling(DC)

}

}

}

if(ButtonPressed(BUTTON_X)){

buttonflag = 4

if(ButtonRaised(BUTTON_L1)){

Pipette[selected].ToggleSealTest()

}

if(ButtonRaised(BUTTON_L2)){

Pipette[selected].ToggleHoldingPotential()

}

if(ButtonRaised(BUTTON_R1)){

Pipette[selected].DisableSealTest()

Pipette[selected].AutoPipetteOffset();

Pipette[selected].SetOsciloscopeCoupling(DC)

}

if(ButtonRaised(BUTTON_R2)){

Pipette[selected].Zap();

}

if(POVDownPressed()){

Pipette[selected].DecreaseOffset();

}

if(POVUpPressed()){

Pipette[selected].IncreaseOffset();

}

}

if(ButtonRaised(BUTTON_Y)){

buttonflag = 5

if(Pipette[selected].GetMode() != IC){

Pipette[selected].SetMode(IC)

Pipette[selected].SetHoldingEnable(false);

Pipette[selected].SetHolding(0);

Pipette[selected].SetOscilloscopeCoupling(AC);

Pipette[selected].SetOscilloscopeScale(0.02);

}

}

if(ButtonPressed(BUTTON_Y)){

buttonflag = 5

if(POVDownPressed()){

Pipette[selected].DecreaseHoldingCurrent();

}

if(POVUpPressed()){

Pipette[selected].IncreaseHoldingCurrent();

}

if(POVRightPressed()){

Pipette[selected].IncreaseBridgeBalResist();

}

if(POVLeftPressed()){

Pipette[selected].DecreaseBridgeBalResist();

}

if(ButtonRaised(BUTTON_L1)){

Pipette[selected].ToggleSetSealTest();

}

if(ButtonRaised(BUTTON_L2)){

Pipette[selected].ToggleSlowCurrentInjEnable();

}

if(ButtonRaised(BUTTON_R1)){

Pipette[selected].AutoPipetteOffset();

}

}

if (buttonflag != 0) return

if( LeftStickLeftRaised()){

DecreaseSpeed

}

if( LeftStickRightRaised()){

IncreaseSpeed

}

if( LeftStickUpRaised() ){

selected = Selected + 1

If( selected > NumberOfPipettes){

selected = 1

}

}

if( LeftStickDownRaised() ){

selected = Selected - 1

If( selected < 1){

selected = NumberOfPipettes

}

}

if( RightStickLeftRaised()){

Microscope.MoveLeft()

}

if( RightStickRightRaised()){

Microscope.MoveRight

}

if( RightStickUpRaised() ){

Microscope.MoveUp()

}

if( RightStickDownRaised() ){

Microscope.MovveDown()

}

if( RightStickLeftReleased()){

Microscope.Stop()

}

if( RightStickRightReleased()){

Microscope.Stop()

}

if( RightStickUpReleased() ){

Microscope.Stop()

}

if( RightStickDownReleased() ){

Microscope.Stop()

}

if(ButtonReleased(BUTTON_R1)) Microscope.Stop();

if(ButtonReleased(BUTTON_R2)) Microscope.Stop();

if(ButtonReleased(BUTTON_L1)) Pipette[selected].Stop();

if(ButtonReleased(BUTTON_L2)) Pipette[selected].Stop();

if(ButtonRaised(BUTTON_L2)){

Pipette[selected].MoveUpZ();

}

if(ButtonRaised(BUTTON_L1)){

Pipette[selected].MoveDownZ();

}

if(ButtonRaised(BUTTON_R2)){

Microscope.MoveUpZ();

}

if(ButtonRaised(BUTTON_R1)){

Microscope.MoveDownZ();

}

switch(joystickState.POV){

case -1:

if(joystickPreviousState.POV != -1){

Pipette[selected].Stop();

}

break;

case POV_RIGHT:

if(joystickState.POV != POV_RIGHT){

Pipette[selected].MoveRight();

}

break;

case POV_LEFT:

if(joystickState.POV != POV_LEFT){

Pipette[selected].MoveLeft();

}

break;

case POV_UP:

if(joystickState.POV != POV_UP){

Pipette[selected].MoveUp();

}

break;

case POV_DOWN:

if(joystickState.POV != POV_DOWN){

Pipette[selected].MoveDown();

}

break;

}

}

}