MATLAB script

Location analysis

%This is the main script, which calls all the other functions.

%"locationAnalysis" is what needs to be put in the command window to get

%this to run.

%Any variables currently stored are cleared at the start of each run of the

%analysis script

clear

%PARAMETERS:

initial_threshold = 30;  %This is an intensity threshold against which the

%cell membrane image is compared.  Any pixels in the image with an

%intensity (ranging from 0-255) above this threshold are classed as

%corresponding to the cell membrane and are used as a mask

filter_threshold = 30;  %This intensity threshold determines which pixels

%are classed as corresponding to the protein being analysed.

bulk = 0;  %This defines the amount by which the width of cell membranes in

%the cell membrane mask are increased.  Setting this to "0" will not

%increase the width.  Changing this will increase the area around the cell

%membranes in which proteins are classed as being colocalised with the cell

%membrane

plot_condition = 1;  %This determines whether the system will display

%images.  Setting this to "1" will show images and setting to "0" will

%surpress images

%MAIN SCRIPT:

%Asking the user to enter the root filename of the two images to be

%analysed.  This name doesn't contain the "0000.tif" part of the name.  It

%is a "string" variable type (word), so is written in single inverted

%commas

root_filename = input('Enter filename root: ', 's');

%Displaying the current filename (as input by the user) in the command

%window

disp(['Analysing "',root_filename, '"']);

%Loading the two images for analysis.  "base_im" is the image showing the

%cell membranes and "top_im" is the image showing the proteins of interest

base_im = imread(strcat([root_filename,'0001.tif']));

top_im = imread(strcat([root_filename,'0000.tif']));

%Displaying the cell membrane image ("base_im") if the user set

%"plot_condition" to "1"

if plot_condition == 1

%Opening a new figure window

figure();

%Populating the new figure window with "base_im" and telling it to

%display in grayscale (the default colourmap is a visible colour

%spectrum)

image(base_im), colormap(gray);

end

%Calling the main function, which generates and applies the image mask.

%This takes certain input variables (listed in brackets) and output other

%ones (in square brackets).  The variables are listed in detail at the

%beginning of the function.  The function is for analysing proteins

%colocalised with the cell membrane

[cell_membrane_im filt_cell_membrane_im cell_membrane_table] = ...

cellMembraneAnalysis(base_im, top_im, initial_threshold, bulk, ...

filter_threshold);

%If the user specified "plot_condition = 1" this will execute and display a

%series of images.  In each case a new figure window is opened.

if plot_condition == 1

%The protein image with only the cell membrane region displayed

figure();

image(cell_membrane_im), colormap(gray);

%The protein image with only the cell membrane region displayed and

%thresholded, so only pixels with an intensity above "filter_threshold"

%are displayed

figure();

image(filt_cell_membrane_im), colormap(gray);

end

%ANALYSIS:

%Calculating the percentage of pixels in the cell membrane region, which

%are above "filter_threshold".

%"cell_membrane_table" contains all the pixels, which were determined

%previously to lie on a cell membrane.  Measuring the number of rows in

%this table gives the total number of pixels on the cell membrane.

cell_membrane_num_total = size(cell_membrane_table);

%Here, the number of pixels in the protein image, which lie on the cell

%membrane and have an intensity above "filter_threshold" are counted.

cell_membrane_num_populated = size(find(cell_membrane_table(:,4)==255));

%This calculates the percentage of pixels that correspond with protein

%(populated) in the total number of possible pixels (area in pixels of the

%cell membrane)

cell_membrane_percentage = (cell_membrane_num_populated(1,1)/...

cell_membrane_num_total(1,1))*100;

%The percentage is displayed in the command window

disp([num2str(cell_membrane_percentage), '% of cell membrane populated']);

%A blank line is output to the command line to make the output look tidier

disp(' ');

Make mask

%This function takes a greyscale image and applies a pixel intensity

%threshold.  Pixels above (or equal to) this threshold are set to "255" and

%those below are set to "0".  There is also the option to increase the area

%surrounding the pixels above "255".  With this "bulking" option all the

%pixels within a square region around each pixel above the threshold is set

%to "255" regardless of their initial intensity.

%INPUTS:

%"input_im" is the input image, which is to be thresholded and turned into

%a mask

%"threshold" is the threshold to be applied to the input image.  It has to

%have a value between "0" and "255"

%"bulk" is a distance in pixels, which determines the area around all

%pixels with initial intensity above the threshold in which all pixels will

%be set to "255". Setting this to "0" effectively turns the feature off.

%OUTPUTS:

%"mask" is the modified version of "input_im", which displays all regions

%above the threshold with intensity "255" and those below with intensity

%"0"

function [mask] = makeMask(input_im, threshold, bulk)

%Initialising the array "mask".  This will be the same size as "input_im",

%so mask is initially just set to be a copy of "input_im"

mask = input_im;

%Applying the threshold to "input_im".  All pixels in "mask" corresponding

%to pixels in "input_im" below the threshold are set to "0"

mask(find(input_im(:,:) < threshold)) = 0;

%Applying the other part of the threshold.  All pixels above "threshold"

%are set to "255"

mask(find(input_im(:,:) >= threshold)) = 255;

%Applying the bulking feature.  This will always run, but if "bulk" is set

%to "0" the image will be unaffected.

%Generating a table called "mask_table", which lists the pixel locations of

%all pixels, which were initially above the threshold (i.e. have thus far

%been assigned an intensity of "255")

[mask_table(:,1), mask_table(:,2)] = find(mask == 255);

%Measuring the size of "mask_table".  This will tell how many rows are in

%"mask_table" and thus is used to determine how many times the bulking loop

%needs to run

size_mask_table = size(mask_table);

%Once for each row in "mask_table" this loop will run.  At each iteration

%the value "i" will increase by "1".  This is used to access the current

%row in "mask_table"

for i = 1:size_mask_table(1,1)

%It will try to access some pixels outside the image when bulking,

%which would cause the system to crash.  By using a "try" statement

%the system will skip any attempts at assigning a new intensity to

%a pixel where it fails.

try

%This section accesses a square region of "mask" around each pixel,

%which was initially above the threshold. The square region has the

%width (2*bulk)+1 (i.e. "bulk" either side of the current pixel and

%the pixel itself) and the same height.

mask((mask_table(i,1)-bulk):(mask_table(i,1)+bulk),...

(mask_table(i,2)-bulk):(mask_table(i,2)+bulk)) = 255;

end

end

%Along two edges of the image (the edges with pixel locations not equal to

%"1") "mask" will be "bulk" wider than "input_im".  This can cause problems

%when applying the mask (trying to multiple matrices of different

%dimensions is not possible), so those extra rows and columns are deleted.

%To know which rows and columns to remove the size of "mask" must be known.

%The number of rows and columns is set to "rows" and "cols".

[rows, cols] = size(mask);

%This takes the extra rows and columns and sets them to "[]", which has the

%effect of removing those rows or columns.

mask(rows-bulk+1:rows,:) = [];

mask(:,cols-bulk+1:cols) = [];

Cell membrane analysis

%This function calls the functions responsible for generating and applying

%the mask, such that only regions of the top image ("top_im") are

%accessible (all other regions being set to an intensity of "0").  The

%locations and intensities of all pixels classed as being in the cell

%membrane are stored in "cell_membrane_table" for analysis outside of the

%function.

%INPUTS:

%"base_im" is the image showing the cell membranes, which will be turned

%into a mask for "top_im"

%"top_im" is the image showing the proteins to be analysed

%"initial_threshold" is an intensity threshold, which determines which

%regions of "base_im" will be classed as being cell membranes or membrane

%"bulk" is the pixel width added to the cell membranes to increase the area

%in which proteins are classed as being co-localised with the cell membrane

%"filter_threshold" is an intensity threshold, which distinguishes between

%proteins and the background

%OUTPUTS:

%"cell_membrane_im" is "top_im" with the cell membrane mask applied

%"filt_cell_membrane_im" is "top_im" with the cell membrane mask applied

%and all pixels with intensities above "filter_threshold" set to 255 and

%those below set to "0"

%"cell_membrane_table" is a 4 column table containing information for all

%pixels identified as being in the cell membrane.  The columns are

%horizontal position, vertical position, intensity in "top_im" and

%post-filter intensity

function [cell_membrane_im filt_cell_membrane_im cell_membrane_table] = ...

cellMembraneAnalysis(base_im, top_im, initial_threshold, bulk, ...

filter_threshold)

%This function generates the mask outlining the cell membranes.  It also

%applies any cell membrane bulking as defined by the user (in "bulk")

[cell_membrane_mask] = makeMask(base_im, initial_threshold, bulk);

%Applying the mask to "top_im".  The mask is set as either "0" or "255", so

%for the purpose of applying the mask all values are divided by "255"

cell_membrane_im = top_im.*(cell_membrane_mask/255);

%Applying the filter to "cell_membrane_im", which sets all pixels above

%"filter_threshold" to "255" and all those below it to "0".  The filtered

%image is set to "filt_cell_membrane_im".

[filt_cell_membrane_im] = makeMask(cell_membrane_im, filter_threshold, 0);

%Setting the first two columns of "cell_membrane_table" to the row and

%column of each pixel in the cell membrane.  These are identified as all

%locations in "cell_membrane_mask" with an intensity equal to 255

[cell_membrane_table(:,1), cell_membrane_table(:,2)] = ...

find(cell_membrane_mask == 255);

%The number of pixels in the cell membrane is given by the size of

%"cell_membrane_table"

num_px = size(cell_membrane_table);

%For each pixel listed by its coordinates in "cell_membrane_table" the

%pre-filtered intensity (actual intensity in "top_im") and the

%post-filtered intensity ("0" or "255") are added to columns 3 and 4 of

%"cell_membrane_table" respectively.

for i = 1:num_px(1,1)

cell_membrane_table(i,3) = cell_membrane_im(...

cell_membrane_table(i,1),cell_membrane_table(i,2));

cell_membrane_table(i,4) = filt_cell_membrane_im(...

cell_membrane_table(i,1),cell_membrane_table(i,2));

end