Equation 1.

FNg 						ammonia flux (mg N/(m²*h))

Volume of air passed through the system 	stroke number * 0. 1 (l/stroke) (l)

conc 						reading of Draeger Tube minus background ppm

p0						sea level standard atmospheric pressure (1013.25 hPa)

pact	 					actual air pressure (hPa)

density of NH3 at temperature of 298.15 K: 696.11 (mg/l)

T25°C 	air temperature 25°C expressed in Kelvin = 298.15 K

Tact 	air temperature at measurement (K); calculate values from measurements in (°C) by adding 273.15.

SN 	dimensionless factor scaling molecular weight of NH3 to molecular weight of N: SN = 14 (g/mol)/17 (g/mol) = 0.824

SA 	scaling factor for expanding area covered by the 4 chambers to 1 square meter: SA (m-²) = 10000 (cm²/m²)/415 (cm²) = 24.096 (m-²)

ST 	scaling factor for expanding fluxes from duration of measurement to one hour: ST (h-1) = 3600 (s/h)/duration of measurement (s); duration of measurement determined with stop watch 8hand pump) or by stroke number of automated pump

Equation 2

ln (calibrated flux) = 0.444 ln (NH3-flux DTM) + 0.590 ln (v Wind 2m)

Equation 3

ln (calibrated flux) = 0.456 ln (NH3-flux DTM) + 0.745 ln (vwind 2 m) - 0.28 ln (vwind 0.2 m)

Explanation of variables in equations 2+3:

calibrated flux				calibrated flux (kg N/(h*m²)

NH3-flux DTM				flux values obtained from Eq. 1 expressed as (kg N/(ha*h)

v Wind 2 m 				wind speed at height of 2 m (m/s)

v Wind 0.2 m 				wind speed at height of 0.2 m (m/s)

Equation 4

Transfer coefficient kg (N/(ha*ppm)=

DTMfinal 	cumulated NH3 loss obtained by DTM for total experimental period (kg N/ha)

Samplercumulated 	sum of corrected concentrations of all passive sampler sampling intervals for total experimental period (ppm)