# Tables

# Table 1

| Plasmid | Amount |
| --- | --- |
| AAVS1-TALEN-L | 5 µg |
| AAVS1-TALEN-R | 5 µg |
| AAVS1-Puro-iCas9 | 40 µg |
| AAVS1-Neo-M2rtTA | 40 µg |

# Table 2

| Primer | Sequence |
| --- | --- |
| T7F | TAATACGACTCACTATAGGG |
| TracrR | AAAAGCACCGACTCGGTGCC |

# Table 3

| Component | Amount |
| --- | --- |
| ddH2O | 35.5 μl |
| 5X PCR reaction buffer | 10 μl |
| dNTP mix (25 mM) | 0.5 μl |
| T7F (10 µM) | 1.25 μl |
| TracrR (10 μM) | 1.25 μl |
| T7-gRNA IVT template (250 nM) | 1 µl |
| DNA polymerase | 0.5 µl |
| Total PCR reaction mix | 50 µl |

# Table 4

# PCR cycling conditions

| Cycle number | Denature | Anneal | Extend |
| --- | --- | --- | --- |
| 1 | 94 ºC , 2 min |  |  |
| 2-31 | 94 ºC, 20 sec | 60 ºC, 20 sec | 72 ºC, 1 min |
| 32 |  |  | 72 ºC, 2 min |

# Table 5

| Component | Amount |
| --- | --- |
| T7 ATP | 2 µl |
| T7 CTP | 2 µl |
| T7 GTP | 2 µl |
| T7 UTP | 2 µl |
| T7 10X buffer | 2 µl |
| T7 enzyme mix | 2 µl |
| PCR amplified template | 8 µl |
| Total in vitro gRNA transcription mix | 20 µl |
| Incubate at 37ºC for 6 hr to overnight |  |

# Table 6

| Component | Amount (μL) |
| --- | --- |
| Unpurified PCR Product | 8 |
| Buffer 2 10X | 2 |
| Distilled water (dH2O) | 10 |

# Table 7

# DNA denaturation and hybridization cycling conditions

| Temperature | Duration | Thermocycler conditions |
| --- | --- | --- |
| 95°C | 10 min |  |
| 85°C | 1 min | Ramp to 85°C at 2°C/s |
| 75°C | 1 min | Ramp to 75°C at 0.3°C/s |
| 65°C | 1 min | Ramp to 65°C at 0.3°C/s |
| 55°C | 1 min | Ramp to 55°C at 0.3°C/s |
| 45°C | 1 min | Ramp to 45°C at 0.3°C/s |
| 35°C | 1 min | Ramp to 35°C at 0.3°C/s |
| 25°C | 1 min | Ramp to 25°C at 0.3°C/s |
| 4°C | Hold |  |

# Table 8

| Stage | Day | Media | Supplement |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S1 | d0 | S1 | GDF8 100 ng/ml | CHIR-99021 3 µM |  |  |  |
|  | d1 | S1 | GDF8 100 ng/ml | CHIR-99021 0.3 µM |  |  |  |
|  | d2 | S1 | GDF8 100 ng/ml |  |  |  |  |

S1 media: MCDB 131 + 1x L-glutamine supplement + 0.5% BSA + 1.5 g/l NaHCO3 + 10 mM Glucose

# Table 9

| Stage | Day | Media | Supplement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S2 | d3-d4 | S1 | LAA 0.25 mM | FGF7 50 ng/ml | IWP-2 2.5 µM |  |  |  |  |  |  |  |  |  |  |  |
| S3 | d5-d6 | S3 | LAA 0.25 mM | FGF7 50 ng/ml | SANT-1 0.25 µM |  | RA 1 µM |  | LDN 100 nM |  | TPB 200 nM |  | ITS-X 1:200 |  |  |  |
| S4 | d7-d9 | S3 | LAA 0.25 mM | FGF7 2 ng/ml | SANT-1 0.25 µM |  | RA 0.1 µM |  | LDN 200 nM |  | TPB 100 nM |  | ITS-X 1:200 |  | IWP-2 2.5 µM |  |

S3 media: MCDB 131 + 1x L-glutamine supplement + 2% BSA + 2.5 g/l NaHCO3 + 10 mM Glucose

# Table 10

| Stage | Day | Media | Supplement |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S5 | d10-d12 | S5 | T3 1 µM | ALK5i II 10 µM | SANT-1 0.25 µM | RA 0.05 µM | LDN 100 nM | ITS-X 1:200 | ZnSO4 10 µM | Heparin 10 µg/ml |
| S6 | d13- d19 | S5 | T3 1 µM | ALK5i II 10 µM | GSiXX 100 nM |  | LDN 100 nM | ITS-X 1:200 | ZnSO4 10 µM | Heparin 10 µg/ml |
| S7 | d20- d33 | S5 | T3 1 µM | ALK5i II 10 µM | N-Cys 1mM | Trolox 10 µM | R428 2 µM | ITS-X 1:200 | ZnSO4 10 µM | Heparin 10 µg/ml |

S5 media: BLAR + 1x L-glutamine supplement + 2 % BSA + 1.5 g/l NaHCO3 + 20 mM Glucose