Supplemental Material 1. Computation of intracellular tension from calculated traction stresses

The intracellular tension is computed from the measured traction stresses by establishing a force balance inside the monolayer and enforcing Saint-Venant’s compatibility of strains. The traction stresses (Tx and Ty) are taken as loads acting on the cells, which are modeled as a thin elastic membrane. The resulting governing equations for the intracellular stresses (σxx, σxy, and σyy) are,

∂σxx∂x+∂σxy∂y=Txh

∂σxy∂x+∂σyy∂y=Tyh

∇2σxx+σyy=1+νh∂Tx∂x+ ∂Ty∂y

Where h=7μm is the thickness of the monolayer and ν=0.45 is the Poisson’s ratio. We defined the intracellular tension as the first invariant of the stress tensor (σxx + σyy).