SUPPLEMENTAL MATERIAL

Capnogram analysis macro code

Sub combine8()

'program last updated, 6-23-04

'to check if user has made a copy of the data file

'Column Layout in Sheet 1:

'A: Time (s)

'B: Pneumotach (raw)

'C: Analyzer (raw)

'D: Pneumotach (Volts)

'E: Flow Rate (mL/s)

'F: Volume (mL), Culmulative

'G: Analyzer (Volts)

'H: Empty at End

'I: Volume (mL), Individual Breath

resp = MsgBox("Have you made a backup copy of the original data file?", 4, "Check!!")

If resp = vbYes Then

'subname = InputBox("Please enter the subject name.  The subject name must be the same as the file name!", "Subject Name")

'Dim numbreaths As Integer

'numbreath = InputBox("Please Enter the Number of Breaths", "Number of Breaths")

'Const numbreaths = numbreath

Const numbreaths = 2

'This section renames the worksheet, and creates a new worksheet

'Previous to this revision this was done by hand before running the macro

'---------------------------------------

Sheets(1).Activate

subname = Sheets(1).Name

Sheets(1).Name = "Sheet1"

Sheets.Add

'---------------------------------------

Sheets("Sheet1").Activate

'declarations

Dim Start(1 To numbreaths + 2), stopp(1 To numbreaths + 2), area(1 To numbreaths + 2), vstart(1 To numbreaths + 2), vend(1 To numbreaths + 2), Vexp(1 To numbreaths + 2), mixedconc(1 To numbreaths + 2), normslop(1 To numbreaths + 2), NS(1 To numbreaths + 2)

Dim texp(1 To numbreaths + 2), tbreath(1 To numbreaths + 2), freq(1 To numbreaths + 2), FRC(1 To numbreaths + 2)

Dim deadsp(1 To numbreaths + 2), deadspace(1 To numbreaths + 2), r2(1 To numbreaths + 2), slop(1 To numbreaths + 2), inter(1 To numbreaths + 2), BegInspVol(1 To numbreaths + 2), EndInspVol(1 To numbreaths + 2), Vinsp(1 To numbreaths + 2), APS

Dim p, q, i, j, l, t As Integer

Dim rname, rname1 As String

Dim startprint(1 To numbreaths + 2), stoppprint(1 To numbreaths + 2) As Integer

Dim AvgFlowRateE(1 To numbreaths + 2), StDevE(1 To numbreaths + 2), Agreement(1 To numbreaths + 2), AvgFlowRateI(1 To numbreaths + 2), TidalVolumeI(1 To numbreaths + 2), TidalVolumeE(1 To numbreaths + 2), starttime(1 To numbreaths + 2), stoptime(1 To numbreaths + 2)

Dim AvgDiff(1 To numbreaths)

p = 1

q = 1

'parameters list

'number of samples

Const initSamp = 3000

'target flow rate for an expiration in mL/s

Const targetFlow = 250

'delay between pneumotach and capnometer

Const Delay = 400

'callibration gain for Pneumotach V to Flow rate

'a value of 200 corresponds with 12Lpm/1V

Const calconst = 1

'voltimes appears to be the multiplier on VD to get the washout V

Const voltimes = 2

'Points to remove at end of CO2 vs Volume curve

pointscut = 10

Const nocycles = 3

'Sampling rate in Hz

Const samprate = 100

'zeropnts is the number of points to skip after the first zero

'crossing while seperating the breaths.  This is to avoid errors

'due to noise around the zero crossing

Const zeropnts = 150

'Tolerance for Dead Space Iteration

Const CTol = 0.5

'Offset for Smoothing algorithm, increase for wider smoothing, set to 0 for none

Const Offset = 25

'Smoothing factor, increase for greater smoothing, default = 1

Const adjustmentPower = 65

'fracMax is the fraction of the maximum value at which to start dramatically increasing smoothing

Const fracMax = 0.77

'Set doSmooth to 0 for no smoothing

Const doSmooth = 0

'volumeCap is the maximum volume for a breath in mL.

'Used to prevent tidal volumes near closing volume.

Const volumeCap = 1250

tinterval = 1 / samprate

tint = tinterval * 1000

'This section deletes entries up to the first inspiration and sets the number of samples

currentStart = 0

isFinished = 0

While (isFinished = 0)

currentStart = currentStart + 1

If (Range("A" + CStr(currentStart)).Value < 0) Then

isFinished = 1

For counterFin = 1 To 50

If (Range("A" + CStr(currentStart + counterFin)).Value >= 0) Then

isFinished = 0

currentStart = currentStart + counterFin

counterFin = 51

End If

Next counterFin

End If

Wend

If (currentStart > 1) Then

Range("A1:A" + CStr(currentStart - 1)).Select

Selection.Delete Shift:=xlUp

Range("B1:B" + CStr(currentStart - 1)).Select

Selection.Delete Shift:=xlUp

End If

samp = initSamp - (currentStart - 1)

'to consider the effect of delay time

Points = Delay / tint

delstartc = "C" + CStr(2)

delstartg = "G" + CStr(2)

delendc = "C" + CStr(Points + 1)

delendg = "G" + CStr(Points + 1)

movebeginc = "C" + CStr(Points + 2)

movebeging = "G" + CStr(Points + 2)

moveendc = "C" + CStr(samp + 1)

moveendg = "G" + CStr(samp + 1)

moverangec = "C2:" + "C" + CStr(samp + 1 - Points)

moverangeg = "G2:" + "G" + CStr(samp + 1 - Points)

rbegin = 2

rend = samp + 1 - Points

asamp = "A" + CStr(samp + 1)

aasamp = "A4:" + asamp

dsamp = "D" + CStr(samp + 1)

ddsamp = "D3:" + dsamp

esamp = "E" + CStr(samp + 1)

eesamp = "E3:" + esamp

'insert recorded macro "dead space"

Range("A1").Select

Selection.EntireRow.Insert

Selection.EntireColumn.Insert

ActiveCell.FormulaR1C1 = "Time(sec)"

Range("B1").Select

ActiveCell.FormulaR1C1 = "Pneumotach"

Range("C1").Select

ActiveCell.FormulaR1C1 = "Analyser"

Range("C2").Select

Columns("B:B").ColumnWidth = 15

Columns("C:C").ColumnWidth = 15

Columns("A:A").ColumnWidth = 15

'to consider time interval as a variable

'fill time column

Range("A2").Value = tinterval

For m = 3 To (samp + 1)

currow = "A" + CStr(m)

prevrow = "A" + CStr(m - 1)

Range(currow).Value = tinterval + Range(prevrow).Value

Next m

Range("A3").Select

ActiveCell.FormulaR1C1 = "=+(R[-1]C+0.01)"

Range("A3").Select

Selection.Copy

'aasamp depends on sampling rate

Range(aasamp).Select

ActiveSheet.Paste

Range("A3").Select

ActiveSheet.Paste

Application.CutCopyMode = False

Range("D1").Select

ActiveCell.FormulaR1C1 = "Pneumotach (V)"

Range("E1").Select

ActiveCell.FormulaR1C1 = "Analyser(V)"

Range("E2").Select

Columns("D:D").ColumnWidth = 15

Columns("E:E").ColumnWidth = 15

Columns("D:D").ColumnWidth = 15

'Fill Pneumotach V and Analyzer V columns

Range("D2").Select

ActiveCell.FormulaR1C1 = "=+((1)*RC[-2])"

Range("E2").Select

ActiveCell.FormulaR1C1 = ""

Range("D2").Select

Selection.Copy

Range("E2").Select

ActiveSheet.Paste

Range("D2").Select

ActiveSheet.Paste

Application.CutCopyMode = False

Selection.NumberFormat = "0.0000"

Range("E2").Select

Selection.NumberFormat = "0.0000"

Range("D2").Select

Selection.Copy

'ddsamp depends on sampling rate

Range(ddsamp).Select

ActiveSheet.Paste

Range("D2").Select

ActiveSheet.Paste

Application.CutCopyMode = False

Range("E2").Select

Selection.Copy

'eesamp depends on sampling rate

Range(eesamp).Select

ActiveSheet.Paste

Range("E2").Select

ActiveSheet.Paste

Application.CutCopyMode = False

Range("E1").Select

Selection.EntireColumn.Insert

Selection.EntireColumn.Insert

ActiveCell.FormulaR1C1 = "Flow rate (ml/sec)"

Range("F1").Select

ActiveCell.FormulaR1C1 = "Volume (ml)"

Range("F2").Select

Columns("E:E").ColumnWidth = 20

'This next section smooths the Analyzer V column by average values +- offset from the

'given value and replacing the given value with the averaged value

'The value of Offset can be adjusted to produce wider smoothing

'adjustment variable causes average to be more heavily weighted (less smoothing) in

'phases I and II to allow for greater precision

If (Not (doSmooth = 0)) Then

maximumVolts = 0

For currentIndex = 2 To samp + 1

If (Range("G" + CStr(currentIndex)).Value > maximumVolts) Then

maximumVolts = Range("G" + CStr(currentIndex)).Value

End If

Next currentIndex

For currentIndex = 2 + Offset To samp + 1 - Offset

Sum = 0

totalSum = 0

For currentValue = currentIndex - Offset To currentIndex + Offset

adjustment = (1 / (Abs(currentValue - currentIndex) + 1)) ^ ((fracMax * maximumVolts / Abs(Range("G" + CStr(currentIndex)).Value + 0.01)) ^ adjustmentPower)

Sum = Sum + (Range("G" + CStr(currentValue)).Value) * adjustment

totalSum = totalSum + adjustment

Next currentValue

Range("K" + CStr(currentIndex)).Select

ActiveCell.FormulaR1C1 = Sum / totalSum

Next currentIndex

Range("K" + CStr(2 + Offset) + ":K" + CStr(samp + 1 - Offset)).Select

Selection.Cut

Range("G" + CStr(2 + Offset) + ":G" + CStr(samp + 1 - Offset)).Select

ActiveSheet.Paste

End If

'to consider callibration const as a variable

'Calculates flow rates in column E from voltages in column D

For m = 2 To (samp + 1)

currcol = "E" + CStr(m)

prevcol = "D" + CStr(m)

Range(currcol).Value = calconst * Range(prevcol).Value

Next m

'effect of delay time starts

Range(delstartc + ":" + delendc).Select

Selection.ClearContents

Range(delstartg + ":" + delendg).Select

Selection.ClearContents

Range(movebeginc + ":" + moveendc).Select

Selection.Cut Destination:=Range(moverangec)

Range(moverangec).Select

ActiveWindow.ScrollRow = 1

Range(movebeging + ":" + moveendg).Select

Selection.Cut Destination:=Range(moverangeg)

Range(moverangeg).Select

ActiveWindow.ScrollRow = 1

Range("A1").Select

'effect of delay time ends

'this part checks to see that the first two flowrate values are

'negative.  This makes sure that the data starts on an

'inspiration like it should

If (Range("E4").Value And Range("E5").Value) > 0 Then

ev = MsgBox("Data starts on an expiration", vbOKOnly, "DATA ERROR")

If ev = 1 Then End

End If

Range("F3").Select

ActiveCell.FormulaR1C1 = "=+((0.01/2)*(R[-1]C[-1]+RC[-1]))"

Range("F3").Select

Selection.Copy

Range("F4").Select

ActiveSheet.Paste

Range("F3").Select

ActiveSheet.Paste

Application.CutCopyMode = False

ActiveCell.FormulaR1C1 = "=+((0.01/2)*(R[-1]C[-1]+RC[-1])+R[-1]C)"

Range("F3").Select

Selection.Copy

Range("F4").Select

ActiveSheet.Paste

Range("F5:" + "F" + CStr(rend)).Select

ActiveSheet.Paste

Range("F3").Select

ActiveSheet.Paste

Application.CutCopyMode = False

Range("A1:G1").Select

Selection.Font.Bold = True

'end of recorded macro

'New section to split the breaths

'This section was completely rewritten since the last version

'The new section ignores noise around the zero crossing point when

'splitting the data into three breaths

'-----------------------------------------------------------------

t = 2

Do

'This section finds the starting point of a breath

endval = 0

Do

rname = "E" + CStr(t)

rname1 = "E" + CStr(t + 1)

If (Range(rname).Value <= 0 And Range(rname1).Value >= 0) Then

testval = 0

For counter = 1 To 100

cellnum = "E" + CStr(t + counter)

If Range(cellnum).Value < 0 Then testval = 1

counter = counter + 1

Next

If testval = 0 Then

Start(p) = t + 1

endval = 1

Debug.Print "p is "; p

Debug.Print "q is "; q

Debug.Print "start is"; t

End If

End If

t = t + 1

Loop Until (endval = 1)

'This section finds the ending point of a breath

t = t + zeropnts

endval2 = 0

Do

rname = "E" + CStr(t)

rname1 = "E" + CStr(t + 1)

If (Range(rname).Value >= 0 And Range(rname1).Value <= 0) Then

stopp(q) = (t)

endval2 = 1

Debug.Print "p is"; p

Debug.Print "q is"; q

Debug.Print "stop is"; t

End If

t = t + 1

Loop Until (endval2 = 1)

p = p + 1

q = q + 1

Loop Until (q = numbreaths + 1)

'This is the end of the new section

'---------------------------------------------------------

Debug.Print "value of p is"; p

Debug.Print "value of q is"; q

For j = 1 To (p - 1)

Debug.Print "the starting point is"; Start(j)

Next j

For j = 1 To (q - 1)

Debug.Print "the ending point is"; stopp(j)

Next j

'This section calculates the average inhaled and exhaled flow rates

'as well as the tidal volume for each breath

'--------------------------------------------------------------------

h = 1

For h = 1 To numbreaths

'This part finds the average (specifically, mean) exhaled flow rate

BeginPtE = Start(h)

EndptE = stopp(h)

BeginPtTimeE = Range(("A" + CStr(Start(h)))).Value

EndPtTimeE = Range(("A" + CStr(stopp(h)))).Value

NumOfPtsE = stopp(h) - Start(h)

TotalFlowE = 0

TotTimeE = EndPtTimeE - BeginPtTimeE

texp(h) = TotTimeE       'time of expiration

m = 0

For m = BeginPtE To EndptE

currentpte = "E" + CStr(m)

TotalFlowE = TotalFlowE + Range(currentpte).Value

Next m

AvgFlowE = TotalFlowE / NumOfPtsE

'This part finds the standard deviation on the average exhaled flow rate

sumStdev = 0

For m = BeginPtE To EndptE

currentpte = "E" + CStr(m)

sumStdev = sumStdev + (AvgFlowE - Range(currentpte).Value) ^ 2

Next m

StDevFlowe = (sumStdev / NumOfPtsE) ^ (1 / 2)

'This part finds the average inhaled flow rate

If h = 1 Then

BeginPtI = 2

BeginPtTimeI = 0.01

EndPtI = Start(h)

EndPtTimeI = Range(("A" + CStr(Start(h)))).Value

NumOfPtsI = EndPtI - BeginPtI

TotTimeI = EndPtTimeI - BeginPtTimeI

Else

BeginPtI = stopp(h - 1)

BeginPtTimeI = Range(("A" + CStr(stopp(h - 1)))).Value

EndPtI = Start(h)

EndPtTimeI = Range(("A" + CStr(Start(h)))).Value

NumOfPtsI = EndPtI - BeginPtI

TotTimeI = EndPtTimeI - BeginPtTimeI

End If

TotalFlowI = 0

n = 0

For n = BeginPtI To EndPtI

currentptI = "E" + CStr(n)

TotalFlowI = TotalFlowI + Range(currentptI).Value

Next n

AvgFlowI = TotalFlowI / NumOfPtsI

TidalVolI = AvgFlowI * TotTimeI * -1

TidalVolE = AvgFlowE * TotTimeE

AvgFlowRateE(h) = AvgFlowE

StDevE(h) = StDevFlowe

Agreement(h) = Abs(AvgFlowE - targetFlow) / StDevFlowe

AvgFlowRateI(h) = AvgFlowI

TidalVolumeI(h) = TidalVolI

TidalVolumeE(h) = TidalVolE

Next h

'----------------------------------------------------------------------------------

'to plot both signals on time scale

'note: rbegin depends on delay time & rend depends both on sampling rate as well as delay time

'Make main plot of volts vs time

arange = "A" + CStr(rbegin) + ":" + "A" + CStr(rend)

drange = "D" + CStr(rbegin) + ":" + "D" + CStr(rend)

grange = "G" + CStr(rbegin) + ":" + "G" + CStr(rend)

trange = arange + "," + drange + "," + grange

Range(trange).Select

Charts.Add

ActiveChart.ChartType = xlXYScatterSmoothNoMarkers

ActiveChart.SetSourceData Source:=Sheets("sheet1").Range( _

trange), PlotBy:=xlColumns

ActiveChart.Location Where:=xlLocationAsNewSheet

'Set Chart Title and Axis Labels

With ActiveChart

.HasTitle = True

.ChartTitle.Characters.Text = "Pneumotach & Analyser signals on time scale" + "   Subject Number: " + CStr(subname)

.Axes(xlCategory, xlPrimary).HasTitle = True

.Axes(xlCategory, xlPrimary).AxisTitle.Characters.Text = "Time"

.Axes(xlValue, xlPrimary).HasTitle = True

.Axes(xlValue, xlPrimary).AxisTitle.Characters.Text = "Volts"

End With

ActiveChart.Legend.Select

Selection.Delete

ActiveChart.PlotArea.Select

With Selection.Border

.ColorIndex = 16

.Weight = xlThin

.LineStyle = xlContinuous

End With

'Clear gray inside chart

Selection.Interior.ColorIndex = xlNone

ActiveChart.Axes(xlCategory).Select

With Selection.Border

.Weight = xlHairline

.LineStyle = xlAutomatic

End With

With Selection

.MajorTickMark = xlOutside

.MinorTickMark = xlOutside

.TickLabelPosition = xlNextToAxis

End With

Sheets("sheet1").Activate

'Skips next section

GoTo 15

'This section finds vco2 using the last breath

'--------------------------------------------------------------------------

10 Rem

'to clear everything in h column

Sheets("sheet1").Activate

HBeg = "H" + CStr(1)

HEnd = "H" + CStr(3499)

HRng = HBeg + ":" + HEnd

Range(HRng).Clear

hnam = "H" + CStr(FBRangeBeg)

hnam1 = "H" + CStr(FBRangeBeg + 1)

hstop = "H" + CStr(FBRangeEnd)

hrange = hnam1 + ":" + hstop

hrange1 = hnam + ":" + hstop

anam = "A" + CStr(FBRangeBeg)

anam1 = "A" + CStr(FBRangeEnd)

Debug.Print "hstart is "; hnam

Debug.Print "hstop is "; hstop

Debug.Print "hrange is "; hrange

Debug.Print "astart is "; anam

Debug.Print "astop is "; anam1

Range(hnam).Select

ActiveCell.FormulaR1C1 = _

"=+((((RC[-2]-R[-1]C[-2])/2)*(RC[-1]+R[-1]C[-1])+R[-1]C))"

Range(hnam).Select

Selection.Copy

Range(hrange).Select

ActiveSheet.Paste

Range(hnam).Select

ActiveSheet.Paste

Application.CutCopyMode = False

Range(hnam1).Select

tt = Range(anam1).Value - Range(anam).Value

Debug.Print "time period is "; tt

vco2 = (Range(hstop).Value) / tt

Debug.Print "vco2 is"; vco2

'to clear everything in h column

Range(HRng).Clear

If zzz = 2 Then GoTo 20

15 Rem

'MAIN LOOP STARTS

For i = 1 To (p - 1)

'to find begining and end value of expired volume and reset end if necessary

vst = "F" + CStr(Start(i))

ven = "F" + CStr(stopp(i))

vstart(i) = Range(vst).Value

vend(i) = Range(ven).Value

If (vend(i) - vstart(i) > volumeCap) Then

counterCap = Start(i)

While (Range("F" + CStr(counterCap)).Value - vstart(i) < volumeCap)

counterCap = counterCap + 1

Wend

stopp(i) = counterCap

vend(i) = Range("F" + CStr(counterCap)).Value

pointscut = 0

End If

'to find beg and end times

stapr = "A" + CStr(Start(i))

startprint(i) = Range(stapr).Value

stopr = "A" + CStr(stopp(i))

stoppprint(i) = Range(stopr).Value

'to find area under volts vs volume curve

hnam = "H" + CStr(Start(i))

hnam1 = "H" + CStr(Start(i) + 1)

hstop = "H" + CStr(stopp(i))

hrange = hnam1 + ":" + hstop

Debug.Print "hstart is "; hnam

Debug.Print "hstop is "; hstop

Debug.Print "hrange is "; hrange

'First New Column Addition

Range(hnam).Select

ActiveCell.FormulaR1C1 = _

"=+((((RC[-2]-R[-1]C[-2])/2)*(RC[-1]+R[-1]C[-1])+R[-1]C))"

Range(hnam).Select

Selection.Copy

Range(hrange).Select

ActiveSheet.Paste

Range(hnam).Select

ActiveSheet.Paste

Application.CutCopyMode = False

Range(hnam1).Select

area(i) = Range(hstop).Value

Debug.Print "area under curve is"; area(i)

'to print the volts vs volume curve (calculation of mixed expired conc. inserted)

'recorded macro starts

For x = Start(i) To stopp(i)

Range("I" + CStr(x)).Value = Range("F" + CStr(x)).Value - vstart(i)

Next x

crangestarti = "I" + CStr(Start(i))

crangestopi = "I" + CStr(stopp(i))

crangestartg = "G" + CStr(Start(i))

crangestopg = "G" + CStr(stopp(i))

'To find mixed expired conc. to be used later for calculating normalized slope

mixedconc(i) = area(i) / TidalVolumeE(i)

Debug.Print "end exp. vol = "; TidalVolumeE(i); "mixed conc = "; mixedconc(i)

rangeg = crangestartg + ":" + crangestopg

rangei = crangestarti + ":" + crangestopi

chartrange = rangeg + "," + rangei

crangestart = "F" + CStr(Start(i))

crangestop = "G" + CStr(stopp(i))

chartrange = crangestart + ":" + crangestop

'to find average value of analyser volts

gval1 = "G" + CStr(Start(i))

gval2 = "G" + CStr(stopp(i))

'Sheets("sheet1").Activate

Gavg = (Range(gval1).Value + Range(gval2).Value) / 2

Debug.Print "average value of analyser volts is "; Gavg

'START OF DEADSPACE CALCULATION

'to find volume at which analyser volts becomes half the average value

For j = Start(i) To stopp(i)

gname = "G" + CStr(j)

gname1 = "G" + CStr(j + 1)

fname = "F" + CStr(j + 1)

If (Range(gname).Value <= Gavg And Range(gname1).Value >= Gavg) Then

'Finds Preliminary VD

volume1 = Range(fname).Value - vstart(i)

'Finds Preliminary Washout V

volume2 = voltimes * volume1

'Finds Preliminary Culmulative Washout V

volume = vstart(i) + volume2

Debug.Print "volume is "; Range(fname).Value

Debug.Print "target volume is "; volume

Exit For

End If

Next j

'to get the required range for best fit line and get its equation

difference = 0

mytest = 0

For k = Start(i) To stopp(i)

Sheets("sheet1").Activate

fnam = "F" + CStr(k)

fnam1 = "F" + CStr(k + 1)

gnam1 = "G" + CStr(k + 1)

Inam1 = "I" + CStr(k + 1)

stopnewI = "I" + CStr(stopp(i) - pointscut)

stopnewf = "F" + CStr(stopp(i) - pointscut)

stopnewg = "G" + CStr(stopp(i) - pointscut)

range1 = fnam1 + ":" + stopnewf

range2 = gnam1 + ":" + stopnewg

rangecomb = fnam1 + ":" + stopnewg

'If at target washout V

If (Range(fnam).Value <= volume And Range(fnam1).Value >= volume) Then

Debug.Print "range is "; rangecomb

r2(i) = Application.WorksheetFunction.RSq(Range(range1), Range(range2))

slop(i) = Application.WorksheetFunction.Slope(Range(range2), Range(range1))

inter(i) = Application.WorksheetFunction.Intercept(Range(range2), Range(range1))

'Equation for deadspace based on B+D method

deadsp(i) = (-inter(i) / slop(i)) + (Sqr((inter(i) / slop(i)) ^ 2 - 2 * area(i) / slop(i) + vend(i) ^ 2 + 2 * inter(i) * vend(i) / slop(i)))

deadspace(i) = deadsp(i) - vstart(i)

check = Abs(deadspace(i) - difference)

If check <= CTol Then

difference = 0

mytest = 1

'Find average difference between curve and fit

sumDiff = 0

maxDiff = 0

maxCounter = 0

For counterDiff = k To stopp(i) - pointscut

'Section to calculate maximum difference between data and fit

'If (maxDiff < Abs(Range("G" + CStr(counterDiff)).Value - (slop(i) * Range("F" + CStr(counterDiff)).Value + inter(i)))) Then

'    maxDiff = Abs(Range("G" + CStr(counterDiff)).Value - (slop(i) * Range("F" + CStr(counterDiff)).Value + inter(i)))

'    maxCounter = counterDiff

'End If

sumDiff = sumDiff + Abs(Range("G" + CStr(counterDiff)).Value - (slop(i) * Range("F" + CStr(counterDiff)).Value + inter(i)))

Next counterDiff

AvgDiff(i) = sumDiff / (stopp(i) - pointscut - k + 1)

'This section draws the volts vs. volume curve and includes the best fit line on the

'plateau

Range(chartrange).Select

Charts.Add

ActiveChart.ChartType = xlXYScatterSmoothNoMarkers

ActiveChart.SetSourceData Source:=Sheets("sheet1").Range(chartrange), _

PlotBy:=xlColumns

ActiveChart.SeriesCollection(1).XValues = "=Sheet1!R" + CStr(Start(i)) + "C9:R" + CStr(stopp(i)) + "C9"

ActiveChart.SeriesCollection(1).Values = "=Sheet1!R" + CStr(Start(i)) + "C7:R" + CStr(stopp(i)) + "C7"

ActiveChart.Location Where:=xlLocationAsNewSheet

With ActiveChart

.HasTitle = True

.ChartTitle.Characters.Text = "Expiration " + CStr(i) + "    Subject Number : " + CStr(subname)

.Axes(xlCategory, xlPrimary).HasTitle = True

.Axes(xlCategory, xlPrimary).AxisTitle.Characters.Text = "volume"

.Axes(xlValue, xlPrimary).HasTitle = True

.Axes(xlValue, xlPrimary).AxisTitle.Characters.Text = "volts"

End With

ActiveChart.Legend.Select

Selection.Delete

ActiveChart.PlotArea.Select

With Selection.Border

.ColorIndex = 16

.Weight = xlThin

.LineStyle = xlContinuous

End With

Selection.Interior.ColorIndex = xlNone

ActiveChart.Axes(xlCategory).Select

Selection.TickLabels.NumberFormat = "0.00"

'This section adds a new data series to the graph.  This series is the range of data that

'is considered when fitting the best fit line to the plateau

ActiveChart.SeriesCollection(1).Select

ActiveChart.SeriesCollection.NewSeries

ActiveChart.SeriesCollection(2).XValues = "=sheet1!R" + CStr(k + 1) + "C9:R" + CStr(stopp(i) - pointscut) + "C9"

ActiveChart.SeriesCollection(2).Values = "=sheet1!R" + CStr(k + 1) + "C7:R" + CStr(stopp(i) - pointscut) + "C7"

ActiveChart.SeriesCollection(2).Select

With Selection.Border

.ColorIndex = 57

.Weight = xlThick

.LineStyle = xlDash

End With

With Selection

.MarkerBackgroundColorIndex = xlNone

.MarkerForegroundColorIndex = xlNone

.MarkerStyle = xlNone

.Smooth = False

.MarkerSize = 3

.Shadow = False

End With

ActiveChart.PlotArea.Select

ActiveChart.SeriesCollection(2).Select

ActiveChart.SeriesCollection(2).Trendlines.Add(Type:=xlLinear, Forward:=0, _

Backward:=0, DisplayEquation:=True, DisplayRSquared:=True).Select

ActiveChart.SeriesCollection(2).Trendlines(1).DataLabel.Select

Selection.Left = 443

Selection.Top = 23

If i = numbreaths Then

FBRangeBeg = CStr(k + 1)

FBRangeEnd = CStr(stopp(i) - pointscut)

zzz = 2

GoTo 10

End If

Debug.Print "1st VD for breath "; i; " = "; deadspace(i)

Exit For

Else

difference = deadspace(i)

Debug.Print "difference = "; difference

volume1 = deadspace(i)

volume2 = voltimes * volume1

volume = vstart(i) + volume2

Debug.Print "volume is "; deadspace(i)

Debug.Print "target volume is "; volume

k = Start(i)

End If

End If

Next k

20 Rem

Sheets("sheet1").Activate

Next i

For l = 1 To (p - 1)

Debug.Print "rsquare value is "; r2(l)

Debug.Print "slope is "; slop(l)

Debug.Print "intercept is "; inter(l)

Next l

'to print dead space for each breath and bronchial csarea

'totaldead = 0

For k = 1 To (p - 1)

'If (k < (p - 1)) Then

'totaldead = totaldead + deadspace(k)

Debug.Print "dead space for breath "; k; "is "; deadspace(k)

'Else

If (k = p - 1) Then

csarea = Sqr(vco2 / (slop(k) * 0.16))

Debug.Print "bronchial csarea is "; csarea

End If

Next k

x = 1

For x = 1 To numbreaths

Debug.Print "Average Exhaled Flow is "; AvgFlowRateE(x)

Debug.Print "Average Inhaled Flow is "; AvgFlowRateI(x)

Next x

'Why is deadspace(1) here?  It isn't added to totaldead in for loop above.

'AVGDEAD = (totaldead - deadspace(1)) / 2

'AVGDEAD = totaldead / numbreaths

'To calculate normalized slope

For i = 1 To numbreaths

normslop(i) = slop(i) / mixedconc(i)

NS(i) = normslop(i)

Debug.Print "NS"; i; " = "; NS(i)

Next i

'To calculate the AP from 4th breath(APS using Scherer's method)

'First of all, get Vco2 for whole breath (Vco2wb)

Sheets("Sheet1").Activate

For i = 1 To numbreaths

If (i = 1) Then

starttime(i) = 0.01

Else

starttime(i) = Range("A" + CStr(stopp(i - 1) + 1)).Value

End If

stoptime(i) = Range("A" + CStr(stopp(i))).Value

Next i

Vco2wb = area(numbreaths) / (stoptime(numbreaths) - starttime(numbreaths))

APS = Sqr(Vco2wb / (slop(numbreaths) * 0.16))

'To calculate RC using Farmery equation

For i = 1 To numbreaths

Vinsp(i) = TidalVolumeI(i)

Vexp(i) = TidalVolumeE(i)

tbreath(i) = stoptime(i) - starttime(i)    'time period of entire breath

freq(i) = 1 / tbreath(i)

Debug.Print "Frequency = "; freq(i); ",  Exp. time = "; texp(i)

FRC(i) = freq(i) * texp(i) / normslop(i) - Vexp(i) / 2 - (Vinsp(i) - Vexp(i))

Next i

'to print the results in a message box

nextline = Chr(13)

Title = "Results"

dsprint = ""

'to make copy of results in sheet2

Sheets("sheet2").Activate

R = 1

'This section sets up the table on sheet2

'START OF JUNK Section 1 - REMOVED

'The Start variable determines what row the numbered rows begin on (the heading comes before it)

Dim StartIndex As Integer

StartIndex = 6

'Make Headers

Columns("A:G").ColumnWidth = 10

'Main Heading

Range("A1").Select

Selection.EntireRow.Insert

Selection.EntireRow.Insert

ActiveCell.FormulaR1C1 = "RESULTS FOR SUBJECT NO " + CStr(subname)

Range("A1").Select

Selection.Font.Bold = True

'Column Titles

Range("A" + CStr(StartIndex - 2)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Breath"

Range("A" + CStr(StartIndex - 1)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Number"

Range("B" + CStr(StartIndex - 2)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Dead"

Range("B" + CStr(StartIndex - 1)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Space"

Range("C" + CStr(StartIndex - 2)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Slope"

Range("D" + CStr(StartIndex - 2)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "NS"

Range("E" + CStr(StartIndex - 2)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "FRC"

Range("F" + CStr(StartIndex - 2)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Avg Flow"

Range("F" + CStr(StartIndex - 1)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Exhaled"

Range("G" + CStr(StartIndex - 2)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "StDev On"

Range("G" + CStr(StartIndex - 1)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Ex. Flow"

Range("H" + CStr(StartIndex - 2)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Agreement"

Range("I" + CStr(StartIndex - 2)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Tidal"

Range("I" + CStr(StartIndex - 1)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Volume I"

Range("J" + CStr(StartIndex - 2)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Tidal"

Range("J" + CStr(StartIndex - 1)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Volume E"

Range("K" + CStr(StartIndex - 2)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "R Squared"

Range("L" + CStr(StartIndex - 2)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Avg Diff"

Range("l" + CStr(StartIndex - 1)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "From Fit"

'Fill Rows with breath numbers and average

For i = 1 To numbreaths

Range("A" + CStr(StartIndex + i)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = i

Next i

Range("A" + CStr(StartIndex + 1 + numbreaths)).Select

Selection.Font.Bold = True

With Selection

.HorizontalAlignment = xlCenter

.VerticalAlignment = xlBottom

.WrapText = False

.Orientation = 0

.AddIndent = False

.ShrinkToFit = False

.MergeCells = False

End With

ActiveCell.FormulaR1C1 = "Average"

'Start of Second Junk Section - REMOVED

'Display Results

sumDS = 0

sumSlope = 0

sumNS = 0

sumFRC = 0

sumFlow = 0

sumDev = 0

sumAgree = 0

sumTVolI = 0

sumTVolE = 0

sumR2 = 0

sumAvgDiff = 0

For cc = 1 To numbreaths

'Dead Space column

cellnum = "B" + CStr(StartIndex + cc)

Range(cellnum).Select

ds = deadspace(cc)

sumDS = sumDS + ds

cellcont = Application.WorksheetFunction.Fixed(ds, 4)

ActiveCell.FormulaR1C1 = cellcont

'Slope column

cellnum = "C" + CStr(StartIndex + cc)

Range(cellnum).Select

Slope = slop(cc) * 10000

sumSlope = sumSlope + Slope

cellcont = Application.WorksheetFunction.Fixed(Slope, 4) + "E-04"

ActiveCell.FormulaR1C1 = cellcont

'NS column

cellnum = "D" + CStr(StartIndex + cc)

Range(cellnum).Select

normSlope = NS(cc) * 10000

sumNS = sumNS + normSlope

cellcont = Application.WorksheetFunction.Fixed(normSlope, 4) + "E-04"

ActiveCell.FormulaR1C1 = cellcont

'FRC column

cellnum = "E" + CStr(StartIndex + cc)

Range(cellnum).Select

fresid = FRC(cc)

sumFRC = sumFRC + fresid

cellcont = Application.WorksheetFunction.Fixed(fresid, 4)

ActiveCell.FormulaR1C1 = cellcont

'Avg Flow Rate column

cellnum = "F" + CStr(StartIndex + cc)

Range(cellnum).Select

AvgFlowE = AvgFlowRateE(cc)

sumFlow = sumFlow + AvgFlowE

cellcont = Application.WorksheetFunction.Fixed(AvgFlowE, 4)

ActiveCell.FormulaR1C1 = cellcont

'StDev on Flow column

cellnum = "G" + CStr(StartIndex + cc)

Range(cellnum).Select

SdevE = StDevE(cc)

sumDev = sumDev + SdevE

cellcont = Application.WorksheetFunction.Fixed(SdevE, 4)

ActiveCell.FormulaR1C1 = cellcont

'Agreement Column

cellnum = "H" + CStr(StartIndex + cc)

Range(cellnum).Select

Agree = Agreement(cc)

sumAgree = sumAgree + Agree

cellcont = Application.WorksheetFunction.Fixed(Agree, 4)

ActiveCell.FormulaR1C1 = cellcont

'Tidal Volume I Column

cellnum = "I" + CStr(StartIndex + cc)

Range(cellnum).Select

TVolI = TidalVolumeI(cc)

sumTVolI = sumTVolI + TVolI

cellcont = Application.WorksheetFunction.Fixed(TVolI, 4)

ActiveCell.FormulaR1C1 = cellcont

'Tidal Volume E Column

cellnum = "J" + CStr(StartIndex + cc)

Range(cellnum).Select

TVolE = TidalVolumeE(cc)

sumTVolE = sumTVolE + TVolE

cellcont = Application.WorksheetFunction.Fixed(TVolE, 4)

ActiveCell.FormulaR1C1 = cellcont

'R2 Column

cellnum = "K" + CStr(StartIndex + cc)

Range(cellnum).Select

rSquared = r2(cc)

sumR2 = sumR2 + rSquared

cellcont = Application.WorksheetFunction.Fixed(rSquared, 4)

ActiveCell.FormulaR1C1 = cellcont

'AvgDiff Column

cellnum = "L" + CStr(StartIndex + cc)

Range(cellnum).Select

aDiff = AvgDiff(cc)

sumAvgDiff = sumAvgDiff + aDiff

cellcont = Application.WorksheetFunction.Fixed(aDiff, 4)

ActiveCell.FormulaR1C1 = cellcont

Next cc

'Print averages

cellnum = CStr(StartIndex + 1 + numbreaths)

Range("B" + cellnum).Select

ActiveCell.FormulaR1C1 = Application.WorksheetFunction.Fixed(sumDS / numbreaths, 4)

Range("C" + cellnum).Select

ActiveCell.FormulaR1C1 = Application.WorksheetFunction.Fixed(sumSlope / numbreaths, 4) + "E-04"

Range("D" + cellnum).Select

ActiveCell.FormulaR1C1 = Application.WorksheetFunction.Fixed(sumNS / numbreaths, 4) + "E-04"

Range("E" + cellnum).Select

ActiveCell.FormulaR1C1 = Application.WorksheetFunction.Fixed(sumFRC / numbreaths, 4)

Range("F" + cellnum).Select

ActiveCell.FormulaR1C1 = Application.WorksheetFunction.Fixed(sumFlow / numbreaths, 4)

Range("G" + cellnum).Select

ActiveCell.FormulaR1C1 = Application.WorksheetFunction.Fixed(sumDev / numbreaths, 4)

Range("H" + cellnum).Select

ActiveCell.FormulaR1C1 = Application.WorksheetFunction.Fixed(sumAgree / numbreaths, 4)

Range("I" + cellnum).Select

ActiveCell.FormulaR1C1 = Application.WorksheetFunction.Fixed(sumTVolI / numbreaths, 4)

Range("J" + cellnum).Select

ActiveCell.FormulaR1C1 = Application.WorksheetFunction.Fixed(sumTVolE / numbreaths, 4)

Range("K" + cellnum).Select

ActiveCell.FormulaR1C1 = Application.WorksheetFunction.Fixed(sumR2 / numbreaths, 4)

Range("L" + cellnum).Select

ActiveCell.FormulaR1C1 = Application.WorksheetFunction.Fixed(sumAvgDiff / numbreaths, 4)

Range("A" + CStr(StartIndex + 3 + numbreaths)).Select

dsprint1 = "Peripheral bronchial cross sectional area is " + Application.WorksheetFunction.Fixed(csarea, 4) + " Sq.cm"

dsprintfin = dsprintfin + dsprint1 + nextline

ActiveCell.FormulaR1C1 = dsprint1

'Start of 3rd Junk Section - REMOVED

currentrange = "A" + CStr(StartIndex + 5 + numbreaths)

Range(currentrange).Select

Selection.Font.Bold = True

ActiveCell.FormulaR1C1 = "Vco2 used in Cross sectional area calculation is " + Application.WorksheetFunction.Fixed(vco2, 4)

Sheets("sheet1").Activate

BegTm = Range(anam).Value

EndTm = Range(anam1).Value

BgTme = Application.WorksheetFunction.Fixed(BegTm, 4)

EdTme = Application.WorksheetFunction.Fixed(EndTm, 4)

Sheets("sheet2").Activate

Range("A" + CStr(StartIndex + 7 + numbreaths)).Select

Selection.Font.Bold = True

ActiveCell.FormulaR1C1 = "----------Integral of Last Breath-------------"

Range("A" + CStr(StartIndex + 8 + numbreaths)).Select

Selection.NumberFormat = "0.000"

Selection.Font.Bold = True

ActiveCell.FormulaR1C1 = "Begin Time: " + CStr(BgTme) + " seconds"

Range("A" + CStr(StartIndex + 9 + numbreaths)).Select

Selection.NumberFormat = "0.000"

Selection.Font.Bold = True

ActiveCell.FormulaR1C1 = "End Time:   " + CStr(EdTme) + " seconds"

'response = MsgBox(dsprintfin, 0, Title)

Sheets("sheet1").Activate

Range("A1").Select

Selection.EntireRow.Insert

Range("A1").Select

ActiveCell.FormulaR1C1 = "SUBJECT NAME :" + CStr(subname)

Range("A1").Select

Selection.Font.Bold = True

Sheets("Sheet2").Activate

'Range("A24").Select

'ActiveCell.FormulaR1C1 = "Subject"

'Selection.Font.Bold = True

'Range("B24").Select

'ActiveCell.FormulaR1C1 = "VD"

'Selection.Font.Bold = True

'Range("C24").Select

'ActiveCell.FormulaR1C1 = "NS (2)"

'Selection.Font.Bold = True

'Range("D24").Select

'ActiveCell.FormulaR1C1 = "NS (3)"

'Selection.Font.Bold = True

'Range("E24").Select

'ActiveCell.FormulaR1C1 = "NS (4)"

'Selection.Font.Bold = True

'Range("F24").Select

'ActiveCell.FormulaR1C1 = "FRC (2)"

'Selection.Font.Bold = True

'Range("G24").Select

'ActiveCell.FormulaR1C1 = "FRC (3)"

'Selection.Font.Bold = True

'Range("H24").Select

'ActiveCell.FormulaR1C1 = "FRC (4)"

'Selection.Font.Bold = True

'Range("I24").Select

'ActiveCell.FormulaR1C1 = "AP"

'Selection.Font.Bold = True

'Range("J24").Select

'ActiveCell.FormulaR1C1 = "APS"

'Selection.Font.Bold = True

'Range("K24").Select

'ActiveCell.FormulaR1C1 = "VCO2wb"

'Selection.Font.Bold = True

'Range("L24").Select

'ActiveCell.FormulaR1C1 = "VCO2alv"

'Selection.Font.Bold = True

'Range("A24:L24").Select

'With Selection

'    .HorizontalAlignment = xlCenter

'    .VerticalAlignment = xlBottom

'    .WrapText = False

'    .Orientation = 0

'    .AddIndent = False

'    .IndentLevel = 0

'    .ShrinkToFit = False

'    .ReadingOrder = xlContext

'    .MergeCells = False

'End With

'Range("A25").Select

'thiscell = CStr(subname)

'ActiveCell.FormulaR1C1 = thiscell

'Range("B25").Select

'thiscell = Application.WorksheetFunction.Fixed(AVGDEAD, 4)

'ActiveCell.FormulaR1C1 = thiscell

'Range("C25").Select

'thiscell = Application.WorksheetFunction.Fixed(NS(2), 4) + "E-04"

'ActiveCell.FormulaR1C1 = thiscell

'Range("D25").Select

'thiscell = Application.WorksheetFunction.Fixed(NS(3), 4) + "E-04"

'ActiveCell.FormulaR1C1 = thiscell

'Range("E25").Select

'thiscell = Application.WorksheetFunction.Fixed(NS(4), 4) + "E-04"

'ActiveCell.FormulaR1C1 = thiscell

'Range("F25").Select

'thiscell = Application.WorksheetFunction.Fixed(FRC(2), 4)

'ActiveCell.FormulaR1C1 = thiscell

'Range("G25").Select

'thiscell = Application.WorksheetFunction.Fixed(FRC(3), 4)

'ActiveCell.FormulaR1C1 = thiscell

'Range("H25").Select

'thiscell = Application.WorksheetFunction.Fixed(FRC(4), 4)

'ActiveCell.FormulaR1C1 = thiscell

'Range("I25").Select

'thiscell = Application.WorksheetFunction.Fixed(csarea, 4)

'ActiveCell.FormulaR1C1 = thiscell

'Range("J25").Select

'thiscell = Application.WorksheetFunction.Fixed(APS, 4)

'ActiveCell.FormulaR1C1 = thiscell

'Range("K25").Select

'thiscell = Application.WorksheetFunction.Fixed(Vco2wb, 4)

'ActiveCell.FormulaR1C1 = thiscell

'Range("L25").Select

'thiscell = Application.WorksheetFunction.Fixed(vco2, 4)

'ActiveCell.FormulaR1C1 = thiscell

'Range("A27").Select

'ActiveCell.FormulaR1C1 = "Vco2wb = CO2 elimination rate over entire breath"

'Selection.Font.Bold = False

'Range("A28").Select

'ActiveCell.FormulaR1C1 = "VcO2alv = CO2 elimination rate under alveolar plateau"

Else

res = MsgBox("please make a backup copy and run the macro again", 0, "Action to be taken!!")

End If

End Sub