# TITLE:

Screening of Axonal Degeneration in Carpal Tunnel Syndrome Using Ultrasonography and Nerve Conduction Studies

# AUTHORS AND AFFILIATIONS:

# Xue Deng1, Lai-Heung Phoebe Chau2, Suk-Yee Chiu2, Kwok-Pui Leung3, Yong Hu1, Wing-Yuk Ip1

1Department of Orthopedics & Traumatology, The University of Hong Kong, Hong Kong Special Administrative Region, China

2Clincal Electro-diagnostic Unit, Tung Wah Hospital, Hong Kong Special Administrative Region, China

3Department of Medicine, The University of Hong Kong, Hong Kong Special Administrative Region, China

Corresponding Author:

Wing-Yuk Ip 			(wyip@hku.hk)

Tel: (852) 2255 4581

Email Addresses of Co-authors:

Xue Deng 			(danny023@connect.hku.hk)

Lai-Heung Phoebe Chau 	(3bchau@gmail.com)

Suk-Yee Chiu 			(csy673@ha.org.hk)

Kwok-Pui Leung 		(kpleungb@hku.hk)

Yong Hu 			(yhud@hku.hk)

# KEYWORDS:

Carpal tunnel syndrome; ultrasound; nerve conduction studies; demyelination; axonal degeneration; diagnostic accuracy

# SUMMARY:

Here we present a protocol to screen potential axonal degeneration associated in carpal tunnel syndrome using nerve conduction studies and ultrasound. The criteria for differentiation are established. Compared to conventional approaches, this method is non-invasive, convenient and efficient with an overall satisfactory accuracy, sensitivity and specificity.

# ABSTRACT:

Axonal degeneration, indicative of surgical decompression, may co-exist in carpal tunnel syndrome (CTS) as the disease progresses. However, the current diagnostic and severity gradation system cannot clearly indicate its co-existence, resulting in confusion of appropriate treatment prescription. There are also constraints in conventional methods for differentiation as well. This study aims at introducing an innovative, efficient and quick screening protocol to differentiate axonal degeneration associated within CTS using ultrasound and nerve conduction studies (NCS). It starts by using NCS to perform orthodromic stimulation at wrist to obtain the sensory conduction of the median and ulnar nerve respectively. Meanwhile, the motor conduction of the median nerve is collected via stimulating at palm, wrist and elbow, followed by stimulation of the ulnar nerve at wrist, below and above elbow. Then, ultrasound assessment is performed using a linear array transducer, with cross-sectional area (CSA) and perimeter (P) at wrist and one third distal forearm calipered. Ratios (R-CSA, R-P) and changes from wrist to one third distal forearm (ΔCSA and ΔP) are calculated according to standard format. Potential axonal degeneration co-existing in CTS will be screened according to the criteria of NCS and cut-off values of ultrasound measurements established in a previous study. In terms of its non-invasiveness, low cost, convenience and efficiency, ultrasound can be easily applied complimentarily in clinical practice to prescreen patients with potential co-existing axonal degeneration. Nevertheless, the ultrasonographic imaging cannot directly reflect axonal degeneration. It still relies on conventional but invasive methods such as electromyography (EMG) and biopsy for confirmation if needed.

# INTRODUCTION:

Carpal tunnel syndrome (CTS) is pathologically a disorder with segmental demyelination whereas secondary axonal degeneration, which is indicative of surgical decompression, may co-exist as the disease progresses1. However, the current diagnostic and severity gradation scale (from mild to very severe grade) for CTS cannot clearly indicate co-existence of axonal degeneration, resulting in confusion of appropriate treatment prescription. Conventional methods for confirming axonal degeneration such as needle electromyography (EMG) and nerve biopsy can be sensitive and accurate, but they are both restricted in the clinical practice due to invasiveness.2

To overcome these shortcomings, ultrasound has been introduced for assisting diagnosing3-5 and grading the severity of CTS 6-8. Our previous studies also successfully identified its cut-off values for discriminating axonal degeneration associated in CTS, with overall satisfactory, sensitivity and specificity9. This study aims at introducing this efficient and non-invasive protocol to practice in the clinical context. The rationale of this protocol is to combine the neurophysiological and structural information provided by NCS and ultrasound respectively to indicate the pathological progress1,10. It is assumed to be more accurate to describe the pathological progress than the current severity gradation system, helping clinicians to better figure out plan of care.   Compared to other conventional neuroimaging technique such as diffusion tensor imaging (DTI), this evidence-based approach can be more easily applied in clinical settings with cheaper cost 11.

# PROTOCOL:

The protocol follows the guideline and all its procedures have been approved by the University of Hong Kong/Hospital authorities Hong Kong West Institutional Review Board (HKU/HA HKW IRB, Ref. Number: UW17-129).

This protocol is applicable to patients who demonstrated clinical symptoms such as numbness, tingling or pain over the median nerve-innervated area of the hand, with positive outcome of Tinel’s and Phalen’s test12. Those subjects with comorbidities such as diabetes mellitus, cancer, rheumatoid arthritis and other peripheral neuropathies are not eligible for this protocol.

1. Measurement of the Nerve Conduction Studies

1. Ask the patient to wash his/her hands with warm water. Dry the hands before placing the electrodes. Keep the hand skin temperature above

32 °C.

2. Measure the conduction in the median sensory nerve (Figure 1).

1. Place the recording (E1) ring electrode at the proximal interphalangeal joint while the reference electrode (E2) is attached over the distal interphalangeal joint.

2. Place the recording electrodes at the wrist between the tendons of the flexor carpal radialis and palmaris longus (12 cm proximal to the E1 electrode) and, ideally, proximal to the distal wrist crease. Ensure that the ground electrode is between the stimulation and

recording sites.

3. Apply supramaximal stimulus 10x to the median nerve via the attached electrodes over the index finger.

4. Collect the averaged readings of the distal sensory latency, the sensory nerve action potential (SNAP) amplitude, and the conduction velocities displayed in the NCS system.

3. Measure the conduction in the ulnar sensory nerve (Figure 1).

1. Place the E1 ring electrode halfway on the proximal phalanx of the fifth digit while the E2 ring electrode should be located 4 cm distal to the E1 ring electrode.

2. Place the recording electrodes near the tendon of the flexor carpi ulnaris (12 cm proximal to the E1 ring electrode). Ensure that the ground electrode is between the stimulation and recording sites.

3. Apply supramaximal stimulus 10x to the ulnar nerve via attached electrodes over the fifth finger.

4. Collect the averaged readings of the distal sensory latency, the SNAP amplitude, and the conduction velocities of the ulnar nerve displayed in the NCS system.

4. Measure the conduction in the median motor nerve (Figure 2).

1. Place the recording electrode E1 in the most prominent eminence of the thenar area at the motor point of the abductor pollicis brevis (APB).

2. Place the reference electrode E2 on the proximal phalanx of the thumb.

3. Stimulate at the mid-palm (3 to 4 cm distal to the distal wrist crease), 6.5 cm proximal to the E1 at the wrist (between the tendons of the flexor carpi radialis and palmaris longus) and the elbow (medial aspect of the antecubital space, just lateral to the brachial artery), using an electronic stimulator.

4. Collect the supramaximal readings of the distal motor latency, the compound motor nerve action potential (CMAP) amplitude, and the conduction velocities displayed in the NCS system.

5. Measure the conduction in the ulnar motor nerve (Figure 3).

1. Place the E1 over the belly of the abductor digiti minimi while the E2 is attached to the distal phalanx of the fifth digit. The ground electrode is between the stimulation and the recording sites.

2. Stimulate at the wrist (7 cm proximal to the E1 electrode, just lateral or medial to the flexor carpi radialis tendon), below and above the elbow (5 cm distal and proximal to the ulnar groove) (Figure 3).

3. Collect the supramaximal readings of the distal motor latency, the CMAP amplitude, and the conduction velocities of the ulnar nerve displayed in the NCS system.

- Measurement using ultrasound

1. Ensure that the patient is seated on a plinth facing the examiner, with their hands resting in a horizontal supination position and the fingers semi-extended (Figure 4).

2. Place some ultrasound gel over the probe of the transducer, the wrist site, and the distal one-third forearm (Figure 4).

3. Perform a transverse scan using a 14 - 13 MHz linear array transducer at the inlet of the carpal tunnel. Freeze the real-time imaging and continuously caliper the hyperechoic epineurium of the median nerve at the inlet of the carpal tunnel (Figure 5).

4. Scan proximally along the innervated area of the median nerve to the site of the one-third distal forearm. Freeze the real-timing imaging and caliper the hyperechoic epineurium of the median nerve at the one-third distal forearm (Figure 6).

3. Screening of potential co-existing axonal degeneration in carpal tunnel syndrome

1. Screen the possible co-existing axonal degeneration primarily based on NCS criteria listed in table 1. Those who fulfilled the criteria for associated axonal degeneration according to NCS criteria in Table 1 will be primarily screened as potential co-existence of axonal degeneration.

2. Consider ultrasound readings as complimentary information for screening axonal degeneration. The client will be considered associated with co-existence of axonal degeneration should any of the measured readings of ultrasound parameter is/are above the cut-off value(s).

3. Take the ultrasound readings into account when the NCS readings failed to fulfil the NCS criteria in table 1. (e.g. NCS signal is absent; conduction block or temporal dispersion occurs etc.) Client can also be considered potential associated axonal degeneration should any of the measured readings of ultrasound parameter is/are above the cut-off value(s). Further golden standard test, such as EMG or biopsy, can be applied for confirmation if needed.

# REPRESENTATIVE RESULTS:

As was described in Table 1, associated axonal degeneration has been primarily screened out should the subject fulfill the criteria of NCS as follow: (1) sensory conduction velocity of the median nerve is less than 42m/s; and/or (2) distal sensory latency is more than 4.6 ms or distal motor latency is more than 3.2 ms; (3) the sensory nerve action potential (SNAP) amplitude at the wrist is less than 10μV with compound motor action potential amplitude drop is no more than 20%. Those with temporal dispersion (CMAP amplitude drop >20%) and/or conduction block (CMAP amplitude drop >50%) will be excluded. Regarding  criteria of NCS parameters, which includes conduction velocity, distal latency, SNAP amplitude, exclusion of conduction block13 and temporal dispersion14, they were all set based on previous well-established references and our laboratory practical standard.

Then, the measured readings of ultrasound parameters will also be taken into account. The cross-sectional area(CSA) is the coronal size of the nerve measured by tracing the hyperechoic epineurium, while the perimeter(P) is the circumference of the traced hyperechoic epineurium. R-CSA/R-P is calculated by the CSA/P measured at the carpal inlet divided by that measured in the one third distal forearm.  ΔCSA/ΔP is the changes of CSA/P from wrist to distal one third forearm via wrist minus one third distal forearm measurements. The cut-off values and their sensitivity, specificity and accuracy of the ultrasound parameters were determined by plotting the Receiver Operative Characteristics (ROC) curves, which were identified in our previous studies 9. The positive predictive value and negative predictive value were also displayed in table 1. Those with any value(s) of ultrasound parameter(s) above the correspondent cut-off value(s) will be considered as potential co-existence of axonal degeneration. Alternatively, the ultrasound readings can also be considered as indicators of potential axonal degeneration should subject do not fulfill the NCS criteria in Table 1.

A group of 80 subjects, who fulfilled the NCS criteria in table 1 were enrolled via convenience sampling to examine the effectiveness of the protocol. They were assigned into group A (Demyelination only, n1=45) and group B (Associated axonal degeneration, n2=35) based on the NCS criteria in table 1. By between-group comparison, significant differences were found in NCS and ultrasound measurements (Table 2). The results indicate this protocol can be effective in screening axonal degeneration associated in carpal tunnel syndrome.

On the other hand, another group of 46 subjects (Group C), who did not fulfill the NCS criteria in table 1, were enrolled with ultrasound measurements generated descriptively. Based on the ultrasound performance, it indicated the enrolled subjects were all potentially associated with co-existing axonal degeneration.

# FIGURE AND TABLE LEGENDS:

# Figure 1: Measurement of the conduction in the sensory median and ulnar nerve

# Figure 2: Measurement of the conduction in the motor median nerve

# Figure 3: Measurement of the conduction in the motor ulnar nerve

# Figure 4: Measurement by ultrasound

# Figure 5: Measurement of the inlet of carpal tunnel and its ultrasound imaging

# Figure 6: Measurement of the distal one-third forearm and its ultrasound imaging

Table 1: Criteria for determining axonal degeneration associated in carpal tunnel syndrome by nerve conduction studies and ultrasound. (Remarks: R-CSA or R-P= CSA or P at wrist/CSA or P at distal one third forearm; ΔCSA or ΔP= CSA or P at wrist – CSA or P at distal one third forearm)

Table 2: Demographics, performance of nerve conduction studies and ultrasound measurement of the enrolled subjects

# DISCUSSION

It is challenging to set up a universal standard for NCS measurement due to the influence of demographic and physical factors such as age, gender, ethnicities and body weight etc15.  Regarding SNAP amplitude, it should be noted that the difference between orthodromic (The Direction of currency runs proximally towards the body) and antidromic (The direction of currency runs distally away from the body) technique as the SNAP amplitude measured by orthodromic method is smaller than antidromic method16. In our laboratory, orthodromic stimulation has been generally applied clinically such that we set 10 μV as the cut-off value of determining axonal damage. A higher SNAP amplitude is required should the antidromic technique be applied.

On the other hand, as for measurement of ultrasound, we took the inlet of carpal tunnel and distal one third forearm as the bony marks for measurement. These measurement sites were consistent with a variety of previous studies, bearing comparatively higher accuracy for diagnosing the disease3,4,8,17-19, which may also consequently increase the accuracy for discrimination. In addition, one of the most innovative aspects using this method is that the cut-off values of ultrasound parameters can take a complimentary role when the electric signal of NCS is absent or did not fulfil the NCS criteria for differentiation as they displayed overall satisfactory sensitivity and specificity in our previous study9. Compared to previous cut-off values identified for severity gradation8,20,21, these cut-off values are believed to be more indicative clinically for treatment prescription as they can reflect pathological progress beyond conventional severity grade.

Nevertheless, there are also constraints using this method. First, the specificity is inferior to the sensitivity in this protocol, as can be seen from the ultrasound findings among subjects in group C, all of which were considered with potential associated axonal degeneration based on the ultrasound cut-off readings. This can be explained as we have not yet established a concrete assessment package to confirm axonal degeneration based on various ultrasound parameters. Besides, the nature of ultrasound imaging cannot directly display axonal degeneration. It still relies on golden standard test to confirm axonal degeneration. Also, there is yet no universal agreement upon NCS criteria for discriminating axonal degeneration9. Further studies are required to examine its reliability by comparison with the golden standard test. Despite these limitations, we still propose this method as an efficient screening approach as it can quickly inform clinicians at early stage in a non-invasive manner, thus avoiding deterioration.

Bearing overall satisfactory accuracy, sensitivity and specificity, the combinative use of ultrasound and NCS cannot only compensate the technical pitfalls of NCS (absent signals etc.) by providing anatomical and structural information, but also it can disclose or identify the pathological progress. This method can be easily applied in routine clinical practice for the initial screening of potential co-existence of axonal degeneration associated in CTS. A more refined plan of care can be attainable using this method.

# ACKNOWLEDGMENTS

Mr. Xue Deng, Dr. Yong Hu, and Dr. Ip Wing-Yuk were dedicated to the concept of the study design, subject recruitment, and draft and revision of the manuscript. Ms. Lai-Heung Phoebe Chau and Ms. Suk-Yee Chiu were dedicated to subject liaison, data collection, and the manipulation of NCS. Dr. Kwok-Pui Leung was dedicated to the electrophysiological diagnosis, clinical guidance and manipulation of the ultrasound.

# DISCLOSURES:

# There are no conflicts of interest declared.

# REFERENCES:

1	Caetano, M. R. Axonal degeneration in association with carpal tunnel syndrome. Arquivos de neuro-psiquiatria. 61 (1), 48-50, (2003).

2	Werner, R. A. & Andary, M. Electrodiagnostic evaluation of carpal tunnel syndrome. Muscle Nerve. 44 (4), 597-607, (2011).

3	Duncan, I., Sullivan, P. & Lomas, F. Sonography in the diagnosis of carpal tunnel syndrome. AJR. American journal of roentgenology. 173 (3), 681-684, (1999).

4	Miyamoto, H. et al. Carpal Tunnel Syndrome- Diagnosis by Means of Median Nerve Elasticity—Improved Diagnostic Accuracy of US with Sonoelastography. Radiology. 270 (2), 481-486, (2014).

5	Azami, A. et al. The diagnostic value of ultrasound compared with nerve conduction velocity in carpal tunnel syndrom. Int J Rheum Dis. 17 612-620, (2014).

6	Altinok, T. et al. Ultrasonographic assessment of mild and moderate idiopathic carpal tunnel syndrome. Clin Radiol. 59 916-925, (2004).

7	Zhang, L. et al. Does the ratio of the carpal tunnel inlet and outlet cross-sectional areas in the median nerve reflect carpal tunnel syndrome severity? Neural Regen Res. 10 (7), 1172-1176, (2015).

8	Klauser, A. S. et al. Sonographic cross-sectional area measurement in carpal tunnel syndrome patients: can delta and ratio calculations predict severity compared to nerve conduction studies? Eur Radiol. 25 (8), 2419-2427, (2015).

9	Deng, X. et al. Exploratory use of ultrasound to determine whether demyelination following carpal tunnel syndrome co-exists with axonal degeneration. Neural Regeneration Research. 13 (2), 317-323, (2018).

10	Moon, H. I., Kwon, H. K., Lee, A., Lee, S. K. & Pyun, S. B. Sonography of Carpal Tunnel Syndrome According to Pathophysiologic Type: Conduction Block Versus Axonal Degeneration. J Ultrasound Med. 36 (5), 993-998, (2017).

11	Razek, A. A. K. A., Shabana, A. A. E., El Saied, T. O. & Alrefey, N. Diffusion tensor imaging of mild-moderate carpal tunnel syndrome- correlation with nerve conduction study and clinical tests. Clinical rheumatology. 36 (10), 2319-2324, (2017).

12	Miedany, E. Y., Ashour, S., Youssef, S., Mehanna, A. & Meky, F. A. Clinical diagnosis of carpal tunnel syndrome: old tests-new concepts. Joint Bone Spine. 75 (4), 451-457, (2008).

13	Weber, F. Conduction block and abnormal temporal dispersion--diagnostic criteria. Electromyogr Clin Neurophysiol. 37 (5), 305-309, (1997).

14	Kiernan, C. M., Mogyoros, I., & Burke D. Conduction block in carpal tunnel syndrome. Brain. 122 (5), 933-941, (1999).

15	Fong, S. Y., Goh, K. J., Shahrizaila, N., Wong, K. T. & Tan, C. T. Effects of demographic and physical factors on nerve conduction study values of healthy subjects in a multi-ethnic Asian population. Muscle Nerve. 54 (2), 244-248, (2016).

16	Valls-Sole, J., Leote, J. & Pereira, P. Antidromic vs orthodromic sensory median nerve conduction studies. Clinical Neurophysiology Practice. 1 18-25, (2016).

17	Wong, S. M., Griffith, J. F., Hui, A. C., Tang, A. & Wong, K. S. Discriminatory sonographic criteria for the diagnosis of carpal tunnel syndrome. Arthritis Rheum. 46 (7), 1914-1921, (2002).

18	Mondelli, M., Filippou, G., Gallo, A. & Frediani, B. Diagnostic utility of ultrasonography versus nerve conduction studies in mild carpal tunnel syndrome. Arthritis Rheum. 59 (3), 357-366, (2008).

19	Wong, S. M. et al. Carpal tunnel syndrome: diagnostic usefulness of sonography. Radiology. 232 (1), 93-99, (2004).

20	Baiee, R. H., AL-Mukhtar, N. J., Al-Rubiae, S. J., Hammoodi, Z. H. & Abass, F. N. Neurophysiological Findings in Patients with Carpal Tunnel Syndrome by Nerve Conduction Study in Comparing with Ultrasound study. Journal of Natural Sciences Reserach. 5 (16), 111-128, (2015).

21	Filius, A. et al. Multidimensional ultrasound imaging of the wrist: Changes of shape and displacement of the median nerve and tendons in carpal tunnel syndrome. J Orthop Res. 33 (9), 1332-1340, (2015).