Table 1: CMR sequences for thrombus evaluation

| Conventional CMR Sequences | Indications | Comments |
| --- | --- | --- |
| Cine SSFP | Bright blood structure and function evaluation | Mass delineation and mobility evaluation. Axial plane is often helpful to correlate with previous radiological imaging. |
| T2w - Triple IR | Saturated blood, fat, and myocardium | Helpful to distinguish thrombus from avidly T2w hyperintense masses such as myxomas. |  |
| 1st pass arterial perfusion | Mass characterization, ischemia evaluation | Thrombus will have no perfusion while vascular masses will perfuse |  |
| Delayed Viability | Myocardial ischemia/disease characterization | Thrombus may have subtle delayed rim enhancement but no internal enhancement |
| Emerging CMR sequences |  |  |  |
| T1 Mapping (pre contrast native T1 and post contrast extracellular volume) | Quantification of T1 relaxation times within myocardium and masses. | T1 values are dependent on field strength and scanner/prescription. |  |
| T2 Mapping | Quantification of T2 relaxation times. Myocardial and mass characterization. | T2 signal is sensitive to mild stressors such as viral illness. Large inter-patient T2w signal variability limiting widespread utility |  |

CMR – cardiac magnetic resonance; SSFP – steady state free precession; T2w – T2 weighted; IR – inversion recovery;