# Submission ID #: 59250

# Scriptwriter Name: Leila Shokri

# Project Page Link: http://www.jove.com/files_upload.php?src=18071298

# Title: Compartmentalization of Human Stem Cell-Derived Neurons within Pre-Assembled Plastic Microfluidic Chips

# Authors and Affiliations:

Smita R Paranjape1,2, Tharkika Nagendran1,2, Valerie Poole3, Joseph Harris3, Anne Marion Taylor1,2,3

1UNC Neuroscience Center

2UNC/NC State Joint Department of Biomedical Engineering, UNC, Chapel Hill, NC, USA

3Xona Microfluidics, LLC, Temecula, CA, USA

# Corresponding Author:

# Anne Marion Taylor			anne.marion.taylor@gmail.com

# Email addresses for Co-authors:

# Smita R Paranjape 			smita_paranjape@hotmail.com

# Tharkika Nagendran 			tharkika@gmail.com

# Valerie Poole 				vpoole@xona.us

# Joseph Harris 				jharris@xona.us

Author Questionnaire:

1. Microscopy: Does your protocol involve video microscopy, such as filming a complex dissection or microinjection technique? N

2. Does your protocol include software usage? N

3. Which steps from the protocol section below are the most important for viewers to see?

2.3-2.4 – Poly L Ornithine coating

2.6 – Laminin coating

3.2 – NSC seeding

3.4 – Media change to differentiation media

4. What is the single most difficult aspect of this procedure and what do you do to ensure success?

2.6 , 3.2 , 3.4 – Ensuring that the coating of P-L-O and Laminin is uniform. The cell density while seeding is critical. The first media change to differentiation media is also a critical step.

5. Will the filming need to take place in multiple locations? N

# Section - Introduction

Videographer: Interviewee Headshots are required. Take a headshot for each interviewee.

- REQUIRED Interview Statements: (Said by you on camera)

  - Anne Taylor: Compartmentalized microfluidic chips allow neurons to be studied in their highly polarized form. These chips are instrumental for both basic and translational research. [1].

    - INTERVIEW

  - Anne Taylor: This protocol describes how to use cyclic olefin copolymer chips to compartmentalize neurons differentiated from human stem cells. These COC chips produce healthier cultures of differentiated neurons over PDMS multi-compartment devices [1].

    - INTERVIEW

OPTIONAL Interview Statements: (Said by you on camera)

  - Smita Paranjape: In this protocol we demonstrate the culture of neuron differentiated from the NIH-approved H9 stem cell line. Similar procedures can be used to differentiate neurons from iPSCs [1].

    - INTERVIEW

# Section - Protocol

- Preparation of the Multi-compartment Chips

  - The multi-compartment chips must be placed into a secondary containment with extra humidification [1].

    - MED: Talent places the chips into a humidifier tray and adds sterile water to the reservoirs of the tray.

  - The channels of the multi-compartment chips must be filled with a pre-coat solution and then flushed with PBS [1-TXT] [2].

    - CU: Talent adds solution to the microfluidic chip. TEXT: See JoVE (Nagendran et al.)

    - ECU: Show a chip filled with PBS.

- Coating of Multi-compartment Chips for hSC-neurons

  - To begin, dissolve poly-L-ornithine (pronounced: “poly·​l·​or·ni·​thine”) in cell culture-grade distilled water to make 600 microliters of solution per chip [1].

    - MED: Talent does few actions to make the solution. TEXT: 20 µg/mL working solution Video editor: Please show text overlay when VO says, “per chip”.

  - Aspirate the remaining PBS from the wells, making sure that the pipet tip is away from the channel opening [1-TXT].

    - CU/ECU: Talent aspirates PBS from the wells. Videographer: Please show the pipet tip if it is possible. TEXT: Do not aspirate liquid from the channels/compartments Video editor: Please show text overlay when VO says, “from the wells”.

  - Load 150 microliters of poly-L-ornithine working solution in the upper right well [1]. Wait for 90 seconds and add 150 microliters of the solution to the lower right well [2-TXT].

    - CU: Talent loads the working solution in the upper right well.

    - CU: Talent loads the solution to the lower right well. TEXT: Until liquid begins to fill the lower right well Video editor: Please show text overlay when VO says, “Wait for 90 seconds”.

  - Wait 5 minutes and repeat the loading process with the solution for the left wells [1-TXT]. Then, place the humidifier tray with chip at 4 °C overnight. If using a Petri dish, wrap it with parafilm and place it at 4 °C overnight [2-TXT].

    - LM: Figure 1C. Video editor: Please emphasize the left wells.  TEXT: To let the liquid pass through the microgrooves Video editor: Please show text overlay when VO says, “Wait for 5 minutes”.

    - MED: Talent transfers the wrapped holder humidifier tray of the chip to a fridge. TEXT: Alternatively, place the chip and tray at 37 °C for 1 h

  - Next, aspirate the solution from the wells, ensuring that the pipet tip is placed away from the channel opening [1], and repeat loading the right and left wells with 150 microliters each of sterile water, twice [2] [3].

    - CU/ECU: Talent aspirates the solution from the wells. Videographer: Please show the pipet tip if it is possible.

    - CU: Talent loads water in the upper right well.

    - CU: Talent loads water in the upper left well.

  - Prepare the laminin working solution [1].

    - MED: Talent does few actions to make the solution. TEXT: 10 µg/mL working solution

  - Load the right and left wells with 150 microliters each of the laminin working solution [1] [2]. Wait 5 minutes and then incubate the chip within the tray for 2 hours at 37 °C [3].

    - CU: Talent loads the laminin working solution in the upper right well.

    - CU: Talent loads the laminin working solution in the upper left well.

    - MED: Talent transfers the chip within the tray to an incubator.

  - Rinse the chip with PBS without Ca2+ and Mg2+ [1]. Load the right [2] and the left wells with 150 microliters each of PBS [3].

    - MED: Talent rinses the chip with PBS without Ca2+ and Mg2+.

    - CU: Talent loads PBS in the upper right well.

    - CU: Talent loads PBS in the upper left well.

  - Wait 5 minutes and rinse the chip with NSC (pronounced: “n·s·​c”) media [1-TXT]. Load the right [2-TXT] and the left wells  with 150 microliters each of the NSC media [3].

    - MED: Talent rinses the chip with NSC media. NSC: Human neural stem cell

    - CU: Talent loads the NSC media in the upper right well.

    - CU: Talent loads the NSC media in the upper left well.

  - Wait 5 minutes and then incubate the chip in the tray overnight in a 37 °C incubator with 5% CO2 to pre-condition the chip [1].

    - MED: Talent transfers the chip to an incubator.

- Seeding NSCs into the Multi-compartment Chip

  - To seed human neural stem cells into the multi-compartment chip, first count the cell concentration using a hemocytometer [1-TXT]. Then, aspirate the media from wells [2-TXT].

    - MED/CU: Talent does few actions to count the cell concentration using a hemocytometer. TEXT: Adjust using NSC media to get a concentration of 7 x 106 cells/ml

    - CU: Talent aspirates the media from the wells. TEXT: Avoid aspirating media from the main channels

  - Pipet 5 microliters of the cell solution to the upper right well, followed by 5 microliters to the lower right well [1-TXT]. Use a microscope to check that cells have entered the channel [2]. Wait 5 minutes for cells to adhere to the bottom of the chip [3].

    - CU: Talent pipets 5 µL of the cell solution to the upper right well, and 5 µL of the cell solution to the lower right well. TEXT: 70,000 cells pipetted towards the main channel openings

    - MED: Talent places the sample on the stage, adjusts the magnification level, and looks through the eyepiece.

    - SCOPE: Show the cells adhered to the bottom of the chip. (Author Comment: See uploaded image: 4.2.3_stem cell seeding.tif.)

  - Pipet approximately 150 microliters of NSC media to the right and left wells [1]. Incubate at 5% CO2, 37 °C within a suitable humidified container [2].

    - CU: Talent loads the NSC media in the upper and lower right wells.

    - CU: Talent loads the NSC media in the upper and lower left wells.

    - MED: Talent transfers the chip to an incubator.

  - After 48 hours, aspirate NSC media from the wells [1] and replace it with neural differentiation media by adding 150 microliters to each top well [2] and each bottom well [3].

    - MED: Talent aspirates NSC media from the wells.

    - CU: Talent loads the neural differentiation media in the upper right well.

    - CU: Talent loads the neural differentiation media in the bottom left well.

  - Culture neurons within a 5% CO2, 37 °C incubator within a humidifier tray [1].

    - MED: Talent transfers the chip to an incubator.

- Viral Fluorescent Labeling of hSC-neurons within the Chip

  - Remove the remaining neural differentiation media from the axonal compartment and place it into a centrifuge tube [1-TXT]. Store at 37 °C [2].

    - CU: Talent removes the differential media from the axonal compartment and places it into a tube. TEXT: Make sure that the channels remain filled with liquid

    - MED: Talent transfers the tube to an incubator.

  - Mix 50 microliters of the virus solution with 150 microliters of the warmed media [1]. Pipet 100 microliters of the mixture to both wells of the axon compartment [2-TXT]. Incubate for 2 hours within a 37 °C incubator [3].

    - MED: Talent mixes the virus solution with the warmed media.

    - CU: Talent pipets 100 microliters of the mixture to one of the compartments. TEXT: Volume difference between axonal and somatic compartments keeps virus in axonal compartment

    - MED: Talent transfers the chip to an incubator.

  - Withdraw and dispose of virus-containing media [1]. Gently pipet 75 microliters of the fresh media to one axon compartment well and let the media flow through the channel into the other axon well [2] [3]. Repeat this process once.

    - CU: Talent withdraws the virus-containing media.

    - CU: Talent pipets the media to one axon compartment well.

    - ECU: Show the media flows through the channel into the other axon well.

  - Finally, fill the axon compartment with the saved media [1-TXT] and transfer the chip to the incubator [2].

    - CU: Talent fills the axon compartment with the saved media. TEXT: Pipet additional 50 µL fresh media to count towards loss of media during the process

    - MED: Talent transfers the chip to the incubator.

# Section – Results

- Results: Using Compartmentalized Microfluidic Chips to Isolate Neurons Differentiated from Human Stem Cells

  - After one week in the chip with the differentiation media, human neural stem cells differentiated into neurons, attached and distributed evenly within the somatic compartment [1]. In comparison, neurons in PDMS (pronounced: “p·d·​m·s”) devices aggregated as early as 5 days post addition of differentiation media, leading to compromised cell health [2].

    - LM: Figure 3B_left.

    - LM: Figure 3B_right.

  - Visualization of labeled neurons and dendritic spines with mCherry fluorescent protein, showed that NSC-derived neurons differentiated within the chips, form mature synapses [1].

    - LM: Figure 4.

  - Neurons were labeled for the excitatory synaptic marker, vGlut1(pronounced: “v·glut·​one”) [1]. Immunostaining results showed that virally labeled neurons could co-localize with vGlut1 [2], and neuron specific marker, β-tubulin III (pronounced: “beta· tubulin·three”) [3].

    - LM: Figure 5A, C. Video editor: Please show 5C when VO says: “Neurons” and 5A when VO says: “the excitatory synaptic marker, vGlut1”.

    - LM: Figure D, E. Video editor: Please show 5D and then the zoomed-in panel 5E. Please emphasize the “yellow dots”.

    - LM: Figure 5H. Video editor: Please emphasize the “white dots”.

  - Virally transduced mCherry neurons extend projections into an axon localized microenvironment established within the preassembled chip [1].

    - Figure 6B. Video editor: Please emphasize the right panel when VO says: “Virally transduced mCherry neurons”, and the left panel when VO says: “extend projections into an axon localized microenvironment”.

  - A comparison between the images of mCherry labeled neurons before and immediately after axotomy, showed completely severed axons [1].

    - LM: Figure 7. Video editor: Please emphasize the 7A, when VO says: “mCherry labeled neurons before” and the left panel of the 7B when VO says: “immediately after axotomy, showed completely severed axons”.

# Section - Conclusion

- Conclusion Interview Statements: (Said by you on camera)

  - Smita Paranjape: It is critical to make sure that the channels remain filled with fluid except when performing the axotomy procedure [1].

    - INTERVIEW

  - Anne Taylor: COC compartmentalized chips are easy to use and can open the door to numerous investigations related to neuronal injury, synaptogenesis, synaptic plasticity, and pathophysiology of disease [1].

    - INTERVIEW