Supplemental coding file

% Instrumented Pull Test Analysis Workflow

% MATLAB Pseudocode

% This function returns the experimental parameters (postural reaction

% time, peak trunk deceleration, step reaction time, total step

% displacement, peak pull force, and rate of force development) as a struct

% object (Results). Inputs required are *.csv file paths to the motion

% tracking and load cell data.

function Results = GetPullTestResults(MotionTrackerPath, LoadcellPath)

% Read data from files

MotionData = array2table(csvread(MotionTrackerPath));

LoadcellData = array2table(csvread(LoadcellPath));

% Align data using trigger signal

MotionIdx = find(MotionData.Trigger, 1);

LoadcellIdx = find(LoadcellData.Trigger, 1);

MotionData(:, MotionIdx) = [];

LoadcellData(:, LoadcellIdx) = [];

% Resample to 1kHz

fs = 1000;

MotionData = resample(MotionData, MotionData.t, fs);

LoadcellData = resample(LoadcellData, LoadcellData.t, fs);

% High-pass filter with cut-off at 0.05 Hz

fc = 0.05;

wc = fc/(fs/2);

[B, A] = butter(20, wc, 'high');

MotionData = filtfilt(B, A, MotionData);

LoadcellData = filtfilt(B, A, LoadcellData);

% Differentiate displacement data

MotionData.TrunkVelocity.Y = diff(MotionData.Trunk.Y);

MotionData.TrunkAcceleration.Y = diff(MotionData.TrunkVelocity.Y);

% Slice data to find single trial epochs

[~, TriggerIdx] = findpeaks(MotionData.Trigger);

% Loop through epochs and generate results

Results.N = length(TriggerIdx);

for i = 1:Results.N-1

s = TriggerIdx(i); % start index

e = TriggerIdx(i+1); % end index

% Check for anticipation

reject = FindAnticipation(MotionData.Trunk.Y(s:e),...

LoadcellData.Force(s:e));

if ~reject

% Populate results struct

Results.TrunkReactionTime(i) = ...

GetTrunkReactionTime(MotionData.Trunk.Y(s:e),...

MotionData.TrunkAcceleration.Y(s:e), fs);

Results.PeakTrunkDeceleration(i) = ...

max(MotionData.TrunkAcceleration.Y(s:e));

Results.LeftStepReactionTime(i) = ...

GetStepReactionTime(MotionData.Trunk.Y(s:e),...

MotionData.LeftFoot.Y(s:e), fs);

Results.RightStepReactionTime(i) = ...

GetStepReactionTime(MotionData.Trunk.Y(s:e),...

MotionData.RightFoot.Y(s:e), fs);

Results.LeftTotalStepDisplacement(i) = ...

GetTotalStepDisplacement(MotionData.LeftFoot.Y(s:e));

Results.RightTotalStepDisplacement(i) = ...

GetTotalStepDisplacement(MotionData.RightFoot.Y(s:e));

Results.PeakPullForce(i) = ...

max(LoadcellData.Force(s:e));

Results.ForceRate(i) = ...

max(diff(LoadcellData.Force(s:e)));

end

end

end

function CanReject = FindAnticipation(TrunkDisplacement, PullForce)

CanReject = 0;

% Find onset of trunk displacement

BaselineMean = mean(TrunkDisplacement);

BaselineSTD = std(TrunkDisplacement);

thresh = BaselineMean + 3*BaselineSTD;

onset = find(TrunkDisplacement>thresh, 1);

% Find onset of pull

BaselineMean = mean(PullForce);

BaselineSTD = std(PullForce);

thresh = BaselineMean + 3*BaselineSTD;

pull = find(PullForce>thresh, 1);

% If trunk displacement occurs before pull, then reject trial

if onset < pull

CanReject = 1;

end

end

function RT = GetTrunkReactionTime(TrunkDisplacement, TrunkAcceleration, fs)

% Find onset of trunk displacement

BaselineMean = mean(TrunkDisplacement);

BaselineSTD = std(TrunkDisplacement);

thresh = BaselineMean + 3*BaselineSTD;

onset = find(TrunkDisplacement>thresh, 1);

% Use zero crossing detector to find start of deceleration

decel = zcd(TrunkAcceleration);

% Calculate reaction time

RT = (decel-onset)/fs;

end

function RT = GetStepReactionTime(TrunkDisplacement, StepDisplacement, fs)

% Find onset of trunk displacement

BaselineMean = mean(TrunkDisplacement);

BaselineSTD = std(TrunkDisplacement);

thresh = BaselineMean + 3*BaselineSTD;

onset = find(TrunkDisplacement>thresh, 1);

% Find onset of step displacement

BaselineMean = mean(StepDisplacement);

BaselineSTD = std(StepDisplacement);

thresh = BaselineMean + 3*BaselineSTD;

step = find(StepDisplacement>thresh, 1);

% Calculate reaction time

RT = (step-onset)/fs;

end

function Step = GetTotalStepDisplacement(StepDisplacement)

Step = max(StepDisplacement);

if Step < 50 % reject if step is less than 50 mm

Step = NaN;

end

end