| Mask | Details |
| --- | --- |
| Binucleated | Levelset(M07, Ch07, Middle, 3) |
|  | Dilate(Levelset(M07, Ch07, Middle, 3) 2) |
|  | Watershed (Dilate(Levelset(M07, Ch07, Middle, 3) 2)) |
|  | Range(Watershed (Dilate(Levelset(M07, Ch07, Middle, 3) 2)),115-5000, 0.4-1) |
|  | Component(1, Area, BN cell mask, Descending) |
|  | Component(2, Area, BN cell mask, Descending) |
| Polynucleated | Levelset(M07, Ch07, Middle, 3) |
|  | Dilate(Levelset(M07, Ch07, Middle, 3) 2) |
|  | Watershed (Dilate(Levelset(M07, Ch07, Middle, 3) 2)) |
|  | Range(Watershed (Dilate(Levelset(M07, Ch07, Middle, 3) 2)),135-5000, 0.4-1) |
|  | Component(1, Area, Poly cell mask, Descending) |
|  | Component(2, Area, Poly cell mask, Descending) |
|  | Component(3, Area, Poly cell mask, Descending) |
|  | Component(4, Area, Poly cell mask, Descending) |
| Micronucleus | Spot(M07, Ch07, Bright, 2, 6, 2) |
|  | Range(LevelSet(M07, Ch07, Middle, 3), 80-5000, 0-1) |
|  | Dilate(Range(LevelSet(M07, Ch07, Middle, 3), 80-5000, 0-1), 2) |
|  | Spot(M07, Ch07, Bright, 2, 6, 2) And Not Dilate(Range(LevelSet(M07, Ch07, Middle, 3), 80-5000, 0-1), 2) |
|  | Spot Mask 1: Range(Spot(M07, Ch07, Bright, 2, 6, 2) And Not Dilate(Range(LevelSet(M07, Ch07, Middle, 3), 80-5000, 0-1), 2)), 10-80, 0.4-1) |
|  | Spot(M07, Ch07, Bright, 2, 6, 1) |
|  | Morphology(M07, Ch07) |
|  | Dilate(Morphology(M07, Ch07), 3) |
|  | Spot(M07, Ch07, Bright, 2, 6, 1) And Not Dilate(Morphology(M07, Ch07), 3) |
|  | Spot Mask 2: Range(Spot(M07, Ch07, Bright, 2, 6, 1) And Not Dilate(Morphology(M07, Ch07), 3)) 8-80, 0.4-1) |
|  | MN mask: Spot Mask 1 OR Spot Mask 2 |
| Micronucleus component masks | Component(1, Area, MN mask, Descending) |
|  | Component(2, Area, MN mask, Descending) |
|  | Component(3, Area, MN mask, Descending) |
|  | MN components mask: Component(1, Area, MN mask, Descending) Or Component(2, Area, MN mask, Descending) Or Component(3, Area, MN mask, Descending) |
| Additional micronucleus masks | Dilate(BNC mask, 4) |
|  | Skeleton(Dilate(BNC mask, 4), Ch07, Thin) |
|  | Dilate(Skeleton(Dilate(BNC mask, 4), Ch07, Thin), 1) |
|  | Dilate(Skeleton(Dilate(BNC mask, 4), Ch07, Thin), 1) or MN components mask |
|  | AdaptiveErode(M01, Ch01, 85) |
|  | Dilate(AdaptiveErode(M01, Ch01, 85), 4) |
|  | Dilate(AdaptiveErode(M01, Ch01, 85), 4) or MN components mask |