# Submission ID #: 59341

# Scriptwriter Name: Leila Shokri

# Project Page Link: http://www.jove.com/files_upload.php?src=18099053

# Title: DNA Electrophoresis Using Thiazole Orange Instead of Ethidium Bromide or Alternative Dyes

# Authors and Affiliations:

Casey S. O’Neil1, Jacie L. Beach1, Todd D. Gruber1

1Department of Molecular Biology and Chemistry, Christopher Newport University, Newport News, VA

# Corresponding Author:

# Todd D. Gruber		todd.gruber@cnu.edu

# Email addresses for Co-authors:

# Casey S. O’Neil 		csoneil@terpmail.umd.edu

# Jacie L. Beach 		jacie.beach.15@cnu.edu

Author Questionnaire:

1. Microscopy: Does your protocol involve video microscopy, such as filming a complex dissection or microinjection technique? (Y/N)  N

2. Does your protocol include software usage? (Y/N Y

If yes, we will need you to record using screen recording software to capture the steps. If you use a Mac, QuickTime X also has the ability to record the steps.

3. Which steps from the protocol section below are the most important for viewers to see?

2.2, 4.1, 4.2, 4.3

4. What is the single most difficult aspect of this procedure and what do you do to ensure success?

4.2/4.3, 5.1 The most difficult thing is to make sure you have proper wavelengths on your system. Fluorescence requires appropriate excitation filters and emission filters, and non-spectroscopists may not be accustomed to the necessity of an emission filter. On different instruments it may be difficult to figure out how to select the correct wavelength.

5. Will the filming need to take place in multiple locations? (Y/N) N

Same building, different nearby rooms

# Section - Introduction

Videographer: Interviewee Headshots are required. Take a headshot for each interviewee.

- REQUIRED Interview Statements: (Said by you on camera)

  - Todd D. Gruber: This protocol is an easy and inexpensive alternative to using ethidium bromide for DNA detection during electrophoresis [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Todd D. Gruber: By removing ethidium bromide, either UV light or blue light can be used for detection. Blue light is not damaging to DNA, which improves the chance of success for downstream experiments [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

OPTIONAL Interview Statements: (Said by you on camera)

  - Jacie L. Beach: It’s really easy to try this out. You just have to replace the ethidium bromide in your gels with thiazole orange [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

# Section - Protocol

- Preparing the Gel

  - To begin, prepare 1percent mini gels by mixing approximately 0.7 grams of agarose in 70 milliliters of tris-acetate-EDTA or tris-borate-EDTA buffers [1-TXT].

    - MED: Talent mixes the agarose and one of the buffers. TEXT: Percentage can be varied for particular size separations Video editor: Please show text overlay when VO says:”1% mini gels”. TEXT: TAE: 40 mM Tris, 20 mM acetate, 1 mM EDTA, pH approximately 8.6 Video editor: Please show text overlay when VO says: ”tris-acetate-EDTA”. TEXT: TBE: tris-borate-EDTA, 90 mM Tris, 90 mM borate, 2 mM EDTA, pH approximately 8.3 Video editor: Please show text overlay when VO says: ”tris-borate-EDTA”.

  - Next, dissolve thiazole (pronounced: “thi·a·zole”) orange in DMSO (pronounced: “d·m·s·o”) to make a 10,000x stock solution [1-TXT]. While not particularly light sensitive, store thiazole orange in the dark when not in use [2]. For long term storage, make aliquots of the thiazole orange-DMSO mixture and freeze them [3] [4-TXT].

    - CU: Talent dissolves thiazole orange in DMSO. TEXT: 13 mg/mL Video editor: Please show text overlay when VO says:”10,000x stock solution”.

    - MED: Talent stores it in a dark drawer.

    - MED: Talent makes couple of aliquots.

    - MED: Talent transfers aliquots to a freezer. TEXT: Melt and resuspend before use

  - Then, add thiazole orange to a final concentration of 1.3 microgram per milliliter [1-TXT], and microwave the mixture of agarose, buffer, and thiazole orange to dissolve agarose for approximately 60 seconds [2-TXT].

    - CU: Talent adds thiazole orange to the agarose mixture. TEXT: The gel can also be stained with TO after electrophoresis

    - MED: Talent transfers the mixture to a microwave oven. TEXT: Thiazole orange can also be added after microwaving Video editor: Please show text overlay when VO says:” and thiazole orange”. TEXT: Quickly swirl to aid dissolution if needed

  - Pour the agarose mixture into a gel casting apparatus containing an appropriate comb [1-TXT], and allow the agarose solution to solidify into a gel [2].

    - CU: Talent puts a comb into a gel casting and pours the mixture into the gel casting. TEXT: Allow the agarose solution to cool briefly Video editor: Please show text overlay when VO says:” pours the mixture into the gel casting”.

    - ECU: Show the solidified gel in the cast.

- Loading and Running the Gel

  - To load the gel, first place the gel in the electrophoresis apparatus if not already present [1]. Then, add TAE or TBE running buffer to cover the surface of the gel [2].

    - MED: Talent places the gel in the electrophoresis apparatus.

    - CU: Talent adds the buffer to cover the surface of the gel.

  - Next, using a loading dye, load 10 microliters of a DNA sample [1]. Include a DNA sizing ladder for reference [2].

    - ECU: Talent loads one sample.

    - ECU: Talent loads the ladder.

  - Attach the cover and electrodes, and run the mini gel at 100 volts until the loading dye traveled approximately 4 to 7 centimeters for a mini gel [1] [2].

    - MED: Talent attaches the cover and electrodes, and runs the voltage.

    - ECU: Show the loading dye traveled 4-7 cm.

  - If thiazole orange was not added prior to gel casting, stain the gel by immersing it in the buffer containing thiazole orange with gentle agitation for about 20 minutes or until the bands are fully detected [1-TXT].

    - CU: Talent immerses the gel in the buffer-thiazole mixture and places them on a shaker. Show a timer set to count down from 20 minutes in the shot. TEXT:  Or if dye separates from smaller DNA fragments, resulting in weak staining Video editor: Please show text overlay when VO says:” If thiazole orange was not added prior to gel casting”.

- Visualization of Thiazole Orange Agarose Gel (UV Transilluminator)

  - To visualize the gel using a UV transilluminator, remove the gel from the electrophoresis apparatus [1] and place it on the UV transilluminator [2-TXT].

    - MED: Talent removes the gel from the electrophoresis apparatus.

    - MED: Talent places the gel on a UV transilluminator. TEXT: Wear appropriate eye, face and hand protection

  - To visualize the gel using a blue-light transilluminator or a flashlight, place the gel on the blue-light transilluminator [1-TXT], or direct the blue LED flashlight at the gel from above or below [2-TXT].

    - MED: Talent places the gel on the blue-light transilluminator. TEXT: 470 nm maximum emission wavelength

    - MED: Talent directs the blue LED flashlight at the gel. TEXT: 470 nm maximum emission wavelength

  - Use an amber emission filter to filter out the blue light, enabling visualization of fluorescence from DNA-thiazole orange complexes and to protect eyes [1-TXT].

    - MED: Talent uses the filter to visualize the bands. TEXT: 560 nm longpass, either goggles or square

  - For further digestion or ligation, cut out desired DNA bands from the gel, and proceed to extracting DNA using a readily available kit or protocol [1-TXT].

    - ECU: Talent cuts the band. TEXT: Expose the gel to UV light for shortest time possible; Alternatively use a blue-light excitation source if possible

- Image Capture

  - Select appropriate excitation and emission settings in the gel-imaging apparatus [1].

    - MED: Talent adjusts the appropriate excitation and emission settings on the software.

  - In the absence of an imaging system with appropriate filters, place an amber filter between the camera and the gel/blue-light excitation source.

    - MED: Talent places an amber filter between the camera and the gel/blue-light excitation source.

# Section – Results

- Results: DNA Staining with Thiazole Orange in Electrophoresis Experiments

  - Detection limits are similar between ethidium bromide, thiazole orange, and a common, blue-light–detectable commercial DNA dye, with the detection limit for all three dyes being 1 to 2 nanograms per lane in a mini-gel [1].

    - LM: Fig1_multigels.

  - Thiazole orange-stained agarose gel of a restriction digest is detectable when it is excited with a UV or a blue-light transilluminator or with a blue-light flashlight [1].

    - LM: Fig2_gelsv2. Video Editor: please show panel A when VO says: “with a UV”, and panel B when VO says: “a blue-light transilluminator”, and panel C when VO says: “a blue-light flashlight”.

# Section - Conclusion

- Conclusion Interview Statements: (Said by you on camera)

  - Todd Gruber: While thiazole orange appears to be less hazardous than ethidium bromide, standard personal protective equipment, such as gloves and goggles, should still be worn [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.