# Submission ID #: 59615

# Scriptwriter Name: Brigid Stadinski

# Project Page Link: http://www.jove.com/files_upload.php?src=18182623

Title: Characterization of Proteins by Size-Exclusion Chromatography Coupled to Multi-Angle Light Scattering (SEC-MALS)

# Authors and Affiliations:

Daniel Some1, Hadar Amartely2, Ayala Tsadok3 and Mario Lebendiker2

1Wyatt Technology Corporation, Santa Barbara, CA, USA

#### 2Wolfson Centre for Applied Structural Biology, The Alexander Silberman Institute of Life Science, The Hebrew University of Jerusalem, Jerusalem, Israel.

3Danyel Biotech Ltd., Rehovot, Israel

# Corresponding Author:

Daniel Some

dsome@wyatt.com

# Email addresses for Co-authors:

Hadar Amartely (hadar.amartely@mail.huji.ac.il)

Ayala Tsadok (ayala@danyel.co.il)

Mario Lebendiker (mario.l@mail.huji.ac.il)

# PLEASE READ THE INSTRUCTIONS IN THE GRAY BOXES CAREFULLY AND USE TRACK CHANGES WHILE MAKING ANY EDITS TO THE DOCUMENT.

# This document has several sections on separate pages, so take care to view each page.

Author Questionnaire:

Authors, please fill out the unanswered questions below.

1. Microscopy: Does your protocol involve video microscopy, such as filming a complex dissection or microinjection technique? (Y/N)  N

Can you record movies/images using your own microscope camera? (Y/N) N/A

If no, JoVE will need to record the microscope images using our scope kit (through a camera port or one of the oculars). Please list the make and model of your microscope.

2. Does your protocol include software usage? (Y/N) Y

If yes, we will need you to record using screen recording software to capture the steps. If you use a Mac, QuickTime X also has the ability to record the steps.

3. Which steps from the protocol section below are the most important for viewers to see? Please list 4-6 individual steps using the step numbers listed in this document. This information is important to prepare your Videographer for your shoot. (You do not need to include steps that will be screen captured. Please do not list entire sections.)

Authors, please answer this question with the steps listed here in the Protocol section below for use by the videographer.

4. What is the single most difficult aspect of this procedure and what do you do to ensure success? Please list 1-2 individual steps using the step numbers listed in this document. (Please do not list entire sections.)

Authors, please answer this question with the steps listed here in the Protocol section below for use by the videographer.

5. Will the filming need to take place in multiple locations? (Y/N) N

If yes, how far apart are the locations?

# Section - Introduction

Videographer: Interviewee Headshots are required. Take a headshot for each interviewee.

Authors, these headshots will be used for the JoVE Dedicated Author Webpage. Here is one example if you wish to take a look.

- REQUIRED Interview Statements: (Said by you on camera)  - All interview statements may be edited for length and clarity.

- The total introduction length (i.e., Required and Optional Interview Statements) cannot exceed 150 words.

- Restrict the length of each statement to no more than 30 words.

- Please answer the questions below in full sentences to highlight the significance of your protocol. You will be expected to memorize and deliver these sentences as spoken interview statements during filming.

- Indicate the full name of each author who will give each statement. If only one author is giving the REQUIRED statements, the same author may speak both statements.

# Why is your protocol significant? OR What key questions can this method help answer?

  - Dan Some:  SEC-MALS is a quantitative method for determining the molecular weight, size and conformation of macromolecules in solution. It can identify oligomers and complexes, and characterize difficult samples like glycoproteins and membrane proteins.

# What is the main advantage of this technique?

  - Hadar Amartely:  SEC-MALS is an absolute method. It relies on fundamental physics and does not depend on calibration against molecular standards, the molecule’s conformation or how it interacts with the separation medium.

OPTIONAL Interview Statements: (Said by you on camera)  - All interview statements may be edited for length and clarity.

- If you would like to have additional speakers, the following OPTIONAL questions may be answered to provide additional introductory information about your protocol.

- These statements must be spoken by different authors than those who gave the Required Interview Statements, and are limited to one statement per additional author.

- The length of each statement is restricted to no more than 30 words and contributes to the total introduction length, which cannot exceed 150 words.

- Indicate the full name of each author who will give each OPTIONAL statement.

# Can this method be applied to any other systems?

  - Mario Lebendiker: SEC-MALS can be applied to many systems that are separated by SEC, from peptides and small polymers to proteins, nucleic acids, polysaccharides, synthetic polymers, virus-like particles and small viruses.

# Do you have any advice to offer to somebody who is trying this technique for the first time?

  - Ayala Tsadok: It is important to verify that your system and buffers meet the vendor’s recommendations for low particulate content. Consult with the vendor’s technical support representative to be sure.

# Section - Protocol

- Read through the entire protocol carefully to understand what you will need on the filming day and prepare accordingly.

- The two-digit numbers (e.g. 2.1., 2.2.) represent the “steps” of you protocol and will be read by a professional voiceover talent.

- The three-digit numbers (e.g. 2.1.1., 2.2.2.) represent the “shots” that our videographer will capture at your lab.

- To ensure that your protocol can be filmed in a single work day, the protocol is restricted to 30 steps and/or 60 shots.

- It is critical for a smooth and organized shoot that all materials and work spaces are prepared and labeled (if applicable) in advance.

- Any specimens/samples that require long or overnight incubation steps should be prepared in advance. (i.e. day 0 sample preparation will be filmed on the day of the shoot; day 1 samples should be prepared the day before the shoot so their processing can be filmed on the day of the shoot/after their overnight culture/treatment/etc.)

# Authors, to avoid having to re-record the audio for your video, please indicate how the narrators should pronounce the following in the narration of the video - (ie as the letters or as something else).

# SEC-MALS S-E-C mahls

# dRI D-R-I

# dn/dc D-N-D-C

# I/O box I-O box

# Rg R-G

# EASI easy

# Ctrl + click control-click

# Mw M-W or weight-average molar mass

- Preparation of Buffers, System, and Sample, and Loading of Sample

# Authors, generally we avoid sections of <3, ~3 line points because it results in a choppy video. For this reason, I have combined some sections. Please let me know if you would like them combined in another manner.

  - Begin this procedure with preparation of the size exclusion chromatography with multi-angle light scattering system as described in the text protocol [1].

    - WIDE: Talent works to prepare the FPLC.

  - Using HPLC-grade reagents, prepare 1 Liter of phosphate-buffered saline with 50 to 100 milliMolar sodium chloride [1]. Filter the buffer to 0.1 micron using a bottle-top polyether sulfone filter or similar [2].

    - MED: Talent dissolves sodium chloride in water. Use labeled containers whenever possible to help with viewer clarity.

    - CU: Bottle-top filter as talent pours the solution into it to filter.

  - Filter the first 50 to 100 milliliters of buffer to a waste bottle and discard, in order to eliminate particulates from the dry filters [1]. Then, filter the remainder to a clean, sterile bottle that has been washed thoroughly with filtered, de-ionized water and capped to prevent dust from entering [2].

    - CU: Solution filters into a waste bottle.

    - CU: Sterile bottle as talent attaches the filter there and begins filtering the solution there.

  - Flush overnight at a flow rate of 0.5 milliliters per minute to equilibrate the column in the buffer and remove particulates [1].

    - WIDE: Talent sets up the FPLC for the overnight run.

  - Use the FPLC’s Continuous flow mode and ensure that the flow does not stop until all SEC-MALS runs are complete. Place the dRI flow cell in Purge mode during the overnight flush [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the FPLC software as talent uses the FPLC’s Continuous flow mode. Then talent places the dRI flow cell in Purge mode during the overnight flush. Authors, please upload this screen capture to your project page. TEXT (video editors, please show for first sentence): FPLC = fast protein liquid chromatography

  - Turn the purge off before beginning sample runs. When beginning the flush, gradually ramp the flow rate to prevent "column shedding" effect caused by a sudden change of pressure in the column [1]. Turn the purge off before beginning sample runs.

    - SCREEN: To be provided by the authors - Screen capture movie of the FPLC software as talent gradually ramp the flow rate to flush avoiding a sudden change of pressure in the column. Then talent turns the purge off before beginning sample runs. Authors, please upload this screen capture to your project page.

  - Check system cleanliness by lightly tapping the tubing downstream of the column to release accumulated particles [1]. Observe the signal in the 90 degree detector on the front-panel display of the MALS instrument. Verify that the peak-to-peak noise is no more than 50 to 100 microVolts [2]. Also verify that the RI signal is stable to less than 1x10-7 RIU [3].

    - CU: Tubing as talent lightly taps it downstream of the column.

    - CU: MALS instrument display showing peak-to-peak noise is no more than 50 to 100 microVolts.

    - CU: RI instrument display showing RI signal stable to within 1x10-7 RIU.

  - Perform a ‘blank’ injection to verify that the injector is clean of particles. A ‘blank’ is simply the running buffer, prepared in a fresh, sterile vial [1].

    - CU: Blank injection as talent loads it onto the machine.

  - If the particle peak is no more than 1 milliliter in volume and no more than 5 milliVolts above baseline, then the system is ready for samples [1]. Otherwise, perform additional blank injections until clean, or perform maintenance to clean the injector [2].

    - MED-over the shoulder: Talent observes the particle peak on the display.

    - WIDE: Talent loads another blank.

  - Now, prepare at least 200 microliters of BSA at 1 to 2 milligrams per milliliter in the size-exclusion chromatography, or SEC, buffer [1-TXT]. Filter the protein to 0.02 microns using a syringe-tip filter. Discard the first few drops of filtrate in order to eliminate particles from the dry filters [2].

    - MED: Talent prepares 200 microliters of BSA at 1 to 2 milligrams per milliliter in the SEC buffer. Use labeled containers whenever possible for viewer clarity. TEXT: BSA = bovine serum albumin

    - CU: Syringe as talent filters the protein, discarding the first few drops.

  - Alternatively, centrifuge the sample at 10,000 x g for 15 min to enable precipitation of non-soluble aggregates and other large particles. – Authors – do you generally use centrifugation of filtration for this step? We can reference the alternative to the text protocol. It is common, and I would like to state this in the video, but there is nothing additional in the text protocol to reference.

  - Inject 100 microliters of the BSA solution into the loop [1].

    - CU: Loop as talent injects 100 microliters of the BSA solution there.

- Preparation of the MALS software and the FPLC software

  - Open New, Experiment from Method in the MALS software menu and selects the Online method from the Light Scattering system methods folder. If a DLS detector is present, select the Online method from the Light Scattering, With QELS folder [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the MALS software as talent opens New | Experiment from Method in the MALS software menu and select the Online method from the Light Scattering system methods folder. If a DLS detector is present, select the Online method from the Light Scattering | With QELS folder. Authors, please upload this screen capture to your project page.

  - In the Configuration section, set parameters of the sample and mobile phase. In the Generic Pump view, set the flow rate to that used in the FPLC. In the Generic Pump view, Solvent branch, Name field, select PBS [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the MALS software as talent navigates to the Configuration section, sets parameters of the sample and mobile phase. In the Generic Pump view, talent sets the flow rate to that used in the FPLC. In the Generic Pump view, Solvent branch, Name field, select PBS. Authors, please upload this screen capture to your project page.

  - In the Injector view, Sample branch, enter the Name as BSA. Set dn/dc to 0.185, A2 to zero, and UV extinction coefficient to 0.667 milliliters per milligram-centimeter [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the MALS software as talent navigates to the Injector view, Sample branch, and enters the Name as BSA. Talent sets dn/dc to 0.185, A2 to zero, and UV extinction coefficient to 0.667 milliliters per milligram-centimeter. Authors, please upload this screen capture to your project page.

  - In the Procedures section, Basic Collection view, select the checkbox Trigger on Autoinject and set the duration of the run to 70 minutes so that data are collected for the entire elution until the total permeation volume of the SEC column is reached [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the MALS software as talent navigates to the Procedures section, Basic Collection view, selects the checkbox Trigger on Autoinject and sets the duration of the run to 70 minutes so that data are collected for the entire elution until the total permeation volume of the SEC column is reached. Authors, please upload this screen capture to your project page.

  - Start the experiment in the MALS software by clicking on the Run button. It will start reading the data after receiving the pulse signal from the FPLC instrument via the MALS detector [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the MALS software as talent clicks Run and data from the MALS detector appears on screen. Authors, please upload this screen capture to your project page.

  - Zero the dRI signal by clicking the Autozero button on the instrument’s front panel [1].

    - CU: Front panel as talent clicks the Autozero button.

  - Working in the FPLC software, navigate to Manual, Execute manual instructions, Set mark and insert the name of protein and the run. Switch the injection valve from Manual load to Inject under Flow path, Injection valve [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the FPLC software as talent navigates to Manual, Execute manual instructions, Set mark and inserts the name of protein and the run. Talent switches the injection valve from Manual load to Inject under Flow path, Injection valve. Authors, please upload this screen capture to your project page.

  - Include a pulse signal by inserting a 0.5 second pulse under I/O box, Pulse digital out. This will trigger data collection in the MALS software [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the FPLC software as talent includes a pulse signal by inserting a 0.5 second pulse under I/O box, Pulse digital out. Authors, please upload this screen capture to your project page.

  - Inject the sample into the loop [1]. Then, click Execute in the FPLC software to start the experiment run [2].

    - CU: Loop/syringe as talent injects the sample there.

    - MED-over the shoulder: Talent clicks  Execute in the FPLC software to start the experiment run.

- Analysis of SEC-MALS BSA Data

  - Perform analysis, step by step, under the Procedures section in MALS software. Verify that peaks appear, at approximately the same elution volume in UV, MALS and RI, by checking the Basic Collection view. In the Baseline view, define baseline for all signals [1].

    - SCREEN: To be provided by the authors - Screen capture movie in the MALS software as talent checks the Basic Collection view. Then talent navigates to the Baseline view and defines baseline for all signals. Authors, please upload this screen capture to your project page.

  - In the Peaks view, define the peaks to be analyzed by clicking and dragging the mouse. Select the central 50% of each peak. First select the monomer peak and then the dimer peak. Verify correct values of dn/dc equal to 0.185 and UV 280 nanometer extinction coefficient equal to 0.667 for BSA under each peak [1].

    - SCREEN: To be provided by the authors - Screen capture movie in the MALS software as talent navigates to the Peaks view and defines the peaks to be analyzed by clicking and dragging the mouse. Then talent selects the central 50% of the monomer peak verifies correct values of dn/dc equal to 0.185 and UV 280 nanometer extinction coefficient equal to 0.667 for BSA under the peak. Authors, please upload this screen capture to your project page.

  - In the Procedures, Alignment view, select the central region of the peaks by clicking and dragging the mouse, click Align Signals and then OK [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the MALS software as talent navigates to Procedures, Alignment view and selects the central region of the peaks by clicking and dragging the mouse, click Align Signals and then OK. Authors, please upload this screen capture to your project page.

  - In the Procedures, Band Broadening view, choose the central 50% of the monomer peak. Make sure the refractive index detector is specified as the Reference Instrument. Then click Perform Fit and Apply to match the UV and light scattering signals to the refractive index signal [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the MALS software as talent navigates to the Procedures, Band Broadening view, and chooses the central 50% of the monomer peak. Talent ensures the refractive index detector is specified as the Reference Instrument. Then talent clicks Perform Fit and Apply to match the UV and light scattering signals to the refractive index signal. Authors, please upload this screen capture to your project page.

  - Zoom in to the peaks to verify that they overlap very closely within the central 50 to 70 percent, and then click OK.  In the Procedures, Normalization view, select Peak 1, enter 3.0 nanometers as the Rg value, click Normalize, and then click OK [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the MALS software as talent zooms into the peaks to verify that they overlay very closely within the central 50 to 70 percent, and then click OK. Talent navigates to the Procedures, Normalization view, selects Peak 1, enters 3.0 nm as the Rg value, clicks Normalize and then OK. Authors, please upload this screen capture to your project page.

  - View the graph of the results in the EASI Graph view. Select Molar Mass from the Display drop-down at the top of the window. Use Ctrl + click and drag to zoom in on the peak region [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the MALS software as talent views the graph of the results in the EASI Graph view. Talent selects Molar Mass from the Display drop-down at the top of the window. Use Ctrl + click and drags to zoom in on the peak region. Authors, please upload this screen capture to your project page.

  - View the final tabulated weight-average molar mass results for the monomer and dimer peaks in the Results, Report-summary view under Peak Results, Molar mass moments, Mw. Purity is reported under Peak Results, Mass fraction [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the MALS software as talent navigates to the Peak Results, Molar mass moments, Mw and to Results, Report-summary to view the final tabulated weight-average molar mass results. Then talent navigates to Peak Results, Mass fraction to see the purity. Authors, please upload this screen capture to your project page.

  - From the File menu, select Save as Method and save the analyzed BSA data as a standard method for future measurements of all types of proteins. The normalization and band-broadening parameters determined for BSA will be carried over in the analysis [1].

    - SCREEN: To be provided by the authors - Screen capture movie of the MALS software as talent navigates to the File menu, selects Save as Method and saves the analyzed BSA data as a standard method for future measurements of all types of proteins. Authors, please upload this screen capture to your project page.

OPTIONAL – Critical Step Statement:

- An OPTIONAL brief statement may be submitted for further elaboration of the best way to perform the required technique for the single most critical step of this procedure.

- If there is no single critical step, then there is no need to fill out this statement.

- This will be an interview style shot interjected after the relevant step within the Protocol section of the video.

- This statement is limited to 30 words or less.

- Please indicate the full name of the Author who will give this statement and the step of the protocol to which the statement pertains using the step numbers from the Protocol section (above).

# Fill in the details below based on the instructions above for the “Critical Step Statement”

# Author name, Step            :           (Write your answer here in the form of a spoken statement. Don’t forget to replace “Author Name” with the name of the person who will be speaking the statement on camera)

# Section – Results

# The Results section is restricted to 200 words of narrative. Please read through the results as presented to make sure that it accurately represents your findings. If you would like to highlight other data, please revise this section accordingly, keeping in mind the word count restriction. Please note that we cannot include narrative without an accompanying visual.

- Results: SEC-MALS analysis Authors, feel free to change this title if you have something else in mind (please just keep it to <2 lines of text)

  - Shown here are SEC-MALS analyses of BSA using a 200 Angstrom pore size-exclusion column. Chromatogram traces are normalized to the monomer peak and offset for clarity [1].

    - LAB MEDIA: Figure 1 final.pdf - Authors: If you have flattened this figure, please submit it in its original file format as unflattened, layered images so that different components can be emphasized in the video. - Video editors, please remove the “A” and “B” labels.  The original is Excel (embedded in Powerpoint) which can be fully manipulated to highlight any component. Will upload.

  - Common artifacts that may be ignored are pointed out [1], including a particle peak near the beginning of the light scattering signal… [2] as well as salt and dissolved air peaks near the total permeation volume in the refractive index signal [3].

    - LAB MEDIA: Figure 1 final.pdf - Video editors, please zoom into the left panel.

    - LAB MEDIA: Figure 1 final.pdf - Video editors, staying zoomed in, please emphasize the text “particle peak” and the correlating arrow.

    - LAB MEDIA: Figure 1 final.pdf - Video editors, staying zoomed in, please emphasize the text “salt peak” and “dissolved air peak” and the correlating arrows.

  - The chromatogram exhibits excellent monomer-dimer-trimer separation and the light scattering signal exhibits high signal-to-noise. The monomer and dimer MW values exhibit high homogeneity [1].

    - LAB MEDIA: Figure 1 final.pdf - Video editors, staying zoomed in, please pan over to the right panel.

  - Here are examples of low-quality SEC-MALS analyses [1].

    - LAB MEDIA: Figure 2 final.pdf - Authors: If you have flattened this figure, please submit it in its original file format as unflattened, layered images so that different components can be emphasized in the video. - Video editors, please remove the “A” and “B” labels. ditto

  - In this example, inadequate separation on a 75 Angstrom pore size exclusion column results in a particle peak between 8 to 9 minutes that is not well separated from the proteins [1].

    - LAB MEDIA: Figure 2 final.pdf - Video editors, please zoom into the left panel and then emphasize the text “particle or aggregate peak” and the correlating arrow.

  - Here, there is an inadequate signal-to-noise ratio, and extensive particles adjacent to the proteins are apparent in the light scattering signal [1].

    - LAB MEDIA: Figure 2 final.pdf - Video editors, staying zoomed in, please pan over to the right panel and then emphasize the text “Noisy LS signal” and the correlating arrow.

# Section - Conclusion

- Conclusion Interview Statements: (Said by you on camera) - All interview statements may be edited for length and clarity.

- Below are questions for statements that can be used to further emphasize the significance of your protocol. At least one statement is required.

- Each statement is limited to 30 words.

- Answer the questions in full sentences, as you will be expected to memorize and deliver the sentences as spoken interview statements during filming.

- Indicate the full name of the author who will give each Conclusion Interview statement.

# What is most important thing to remember when attempting this procedure? Please indicate the steps (e.g., 2.4., 2.5.) in the Protocol section this advice correlates to.

  - Hadar Amartely: The keys to good SEC-MALS results are, selecting an appropriate column, ensuring that the system is equilibrated with low light-scattering noise, and pre-filtering or centrifuging the samples to remove particles..  (Step: 2.7-2.10)

# Following this procedure, what other methods can be performed? What questions would these additional methods answer?

  - Dan Some: Following SEC-MALS, protein samples can be further analyzed for binding affinity by ELISA, SPR, ITC, BLI, MST or CG-MALS. Its structure may be determined by x-ray crystallography, cryo-EM or NMR.

# After its development, did this technique pave the way for researchers to explore new questions within a specific scientific field? If so, how?

  - Author Name: ____ (Write your answer here in the form of a spoken statement. Don’t forget to replace “Author Name” with the name of the person who will be speaking the statement on camera)

# Are any of the reagents or instruments hazardous? If so, please use this interview statement to remind viewers of what precautions they should take.

  - Author Name: ___(Write your answer here in the form of a spoken statement. Don’t forget to replace “Author Name” with the name of the person who will be speaking the statement on camera)

Thank you for following the instructions and addressing our questions. We will incorporate your answers/suggestions and send you the finalized script before your shoot. You will also receive detailed shoot preparation instructions in the email accompanying the finalized script.