# Submission ID #: 59618

# Scriptwriter Name: Bridget Colvin

Project Page Link: http://www.jove.com/files_upload.php?src=18183538

Title: Quantifying the Relative Thickness of Conductive Ferromagnetic Materials Using Detector Coil-Based Pulsed Eddy Current Sensors

## Authors and Affiliations: Nalika Ulapane1 and Karthick Thiyagarajan2 and David Hunt2 and Jaime Valls Miro2

1Melbourne School of Engineering, University of Melbourne

2Center for Autonomous Systems, University of Technology Sydney

# Corresponding Author:

Nalika Ulapane

nalika.ulapane@unimelb.edu.au, nalika.ulapane@gmail.com

Email addresses for Co-authors:

Karthick.thiyagarajan@uts.edu.au

david.hunt@uts.edu.au

Jaime.VallsMiro@uts.edu.au

Author Questionnaire:

1. Microscopy: Does your protocol involve video microscopy? N

2. Does your protocol demonstrate software usage? Y

3. Which steps from the protocol section below are the most important for viewers to see?

n/a

4. What is the single most difficult aspect of this procedure and what do you do to ensure success?

n/a

5. Will the filming need to take place in multiple locations (greater than walking distance)? N

# Section - Introduction

Videographer: Interviewee Headshots are required. Take a headshot for each interviewee.

- REQUIRED Interview Statements (Said by you on camera): All interview statements may be edited for length and clarity.

  - Nalika Ulapane: Pulsed eddy current sensing is an electromagnetic sensing technique used to assess certain physical properties and defects of electrically conductive, and sometimes ferromagnetic, materials [1].

    - LAB MEDIA: Provided by Authors: Named talent says above statement in an interview-style shot, looking slightly off-camera

  - Karthick Thiyagarajan: We will introduce the use a of software interface for visualizing pulsed eddy current signals captured from detector coil-based sensor architecture and how to extract information from these signals [1].

    - LAB MEDIA: Named talent says above statement in an interview-style shot, looking slightly off-camera

OPTIONAL Interview Statements: (Said by you on camera) - All interview statements may be edited for length and clarity.

  - David Hunt: This protocol enables the relative thickness quantification of ferromagnetic metallic wall-like structures using pulsed eddy current sensing [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

  - Karthick Thiyagarajan: This technique enables the measurement of the extent of corrosion, graphitization, or wall loss of ferromagnetic wall-like structures for assessment of their structural health and integrity [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

# Section - Protocol

- PEC Signal Processor Installation

  - For pulsed eddy current signal processor installation, locate and execute the PEC_Signal_Processor.exe file [1-TXT] and, when the interface appears, click Next [2].

    - WIDE: Talent at computer, executing file TEXT: See text for principles and details on PEC signal-based relative thickness quantification

    - SCREEN: Installation: 00:44-00:49

  - When the next interface opens, specify the file location for installation and check the Add a shortcut to the desktop checkbox to add the software icon to the desktop [1].

    - SCREEN: Installation: 00:50-00:54

  - Click Next and specify the installation location for the required Runtime Environment [1].

    - SCREEN: Installation: 00:55-01:02

  - When the installation is complete, click Finish. The desktop icon will appear [1].

    - SCREEN: Installation: 01:03-01:11 Video Editor: can emphasize desktop icon in bottom left corner as appropriate

- Signal Collection and Application Execution

  - Before beginning the analysis, use a detector coil-based pulsed eddy current sensing unit with an operating electronics box [1] and a detector coil-based sensor [2] to collect pulsed eddy current signals from the metal structure of interest [3-TXT].

    - WIDE: Talent picking up unit and/or indicate operating electronics box

    - Shot of unit Video Editor: please emphasize sensor

    - Talent collecting PEC signals from pipe TEXT: See text for principles of PEC signal processing details

  - At the end of the scan, export the collected signals from the sensing unit to a compatible editable word processor [1] and confirm that the signals are arranged as a table [2].

    - Talent connecting unit to CPU

    - Talent adding data to table, with monitor visible in frame

  - Copy the table to the desktop [1] and double click the desktop icon to run the application. The interface will open [2].

    - SCREEN: Running: 00:26-00:36

    - SCREEN: Running: 00:40-00:50

  - To load the signals, click the Load Signals tab and select the file containing the signals to import the signals to the software interface [1].

    - SCREEN: Running: 00:51-01:02

  - When the Number of Signals contained within the table containing raw signals appears, click Plot Signals and observe the signals plotted in logarithmic scale [1].

    - SCREEN: Running: 01:04-01:16

  - David Hunt:  The lower and upper margins should encompass a straight-line region within the late stage of signals. If unsure, generating relative thickness values using several margins and comparing results is recommended [1]. Author NOTE: Move this statement to appear immediately after Step 3.7, and before Step 3.8.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

  - Click the Zoom tab and adjust the plot window for the linear region until it is clearly visible [1].

    - SCREEN: Running: 01:16-01:30 Video Editor: can speed up

  - After entering reasonable lower and upper margins for the region [1], click Plot Margins and wait for the margins to be plotted in green [3].

    - SCREEN: Running: 01:33-01:45

    - SCREEN: Running: 01:47-01:54

  - Click Extract Features. The straight-line segments will be plotted in red [1].

    - SCREEN: Running: 02:13-02:16

  - Click Calculate Relative Thickness to observe the plotting of a histogram of calculated relative thickness values [1].

    - SCREEN: Running: 02:21-02:30

  - Click Save Relative Thickness to save the calculated relative thickness values, enter a filename, and click OK [1].

    - SCREEN: Running: 02:35-02:47

  - Then click OK again to confirm the filename. The relative thickness values will be saved as a table on the desktop [1].

    - SCREEN: Running: 02:47-02:51

# Section – Results

- Results: Representative PEC Signals

  - Here the typical shape of a time domain signal expressed in the logarithmic form captured from a detector coil-based pulsed eddy current sensor is shown [1].

    - LAB MEDIA: Figure 1: JoVE Video Editor please add/emphasize red arrow and PEC Signal Amplified Saturation text

  - An indicative linear region of the later stage of the logarithmic signal can be observed, for which t is much greater than 0 and from which the decay rate feature beta is extracted [1].

    - LAB MEDIA: Figure 2: JoVE Video Editor please add red arrow and accompanying formula and/or add/emphasize red dotted lines

  - Here a set of signals captured on different thicknesses of grey cast iron is shown [1].

    - LAB MEDIA: Figure 3

  - This signal corresponds to the maximum thickness, with a maximum beta value in the set [1].

    - LAB MEDIA: Figure 3: JoVE Video Editor please add arrow and Maximum beta text OR emphasize data line indicated by arrow and text

  - Such values can be selected as reference beta values to facilitate quantification of the relative thickness using the equation as indicated [1-TXT].

    - LAB MEDIA: Figure 3: JoVE Video Editor please emphasize four data lines to left of maximum beta data line

    - LAB MEDIA: Figure 3: TEXT: di%= βiβref×100%

  - In this Table, the actual relative thickness values [1] and the relative thickness values calculated from beta values obtained from pulsed eddy current signals for a representative experiment are shown [2].

    - LAB MEDIA: Table 1: JoVE Video Editor please emphasize Actual Thickness of Test Pieces column

    - LAB MEDIA: Table 1: JoVE Video Editor please emphasize Relative Thickness % Quantified from Beta

  - Here the correlation between pulsed eddy current signal-based relative thickness estimates and the actual relative thickness values can be observed1].

    - LAB MEDIA: Figure 4

  - The effectiveness of the method is demonstrated by the high correlation depicted by the linear relationship [1].

    - LAB MEDIA: Figure 4: JoVE Video Editor please draw correlation line between data points and/or emphasize data points and/or Correlation = 99.83% text

# Section - Conclusion

- Conclusion Interview Statements: (Said by you on camera) - All interview statements may be edited for length and clarity.

  - David Hunt: Take care to avoid the curved region of the pulsed eddy current signals and to select the linear region at the late stage to allow extraction of the beta feature [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera (Step: 3.8., Step 3.9.)

  - David Hunt: Pulsed and other eddy current techniques are commonly used for anomaly detection and physical property measurements and for the assessment of non-metallic coatings covering metallic structures [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera