Macro for annexinV fluorescence analysis

outputFolder = getDirectory("image")

Imagetitle = getTitle;

run("Split Channels");

selectWindow(Imagetitle+" (blue)");

close();

selectWindow(Imagetitle+" (red)");

close();

selectWindow(Imagetitle+" (green)");

//run("Threshold...");

setAutoThreshold("Default dark");

setThreshold(0, 120);

run("Measure");

selectWindow("Results");

saveAs("Measurements", "" + outputFolder + Imagetitle

+"Background.xls");

selectWindow(Imagetitle+" (green)");

//run("Threshold...");

setAutoThreshold("Default");

setThreshold(120, 255);

run("Analyze Particles...", "size=30-10000 show=[Overlay Masks] display");

selectWindow ("Results");

saveAs("Measurements", "" + outputFolder + Imagetitle +".xls");

selectWindow(Imagetitle+" (green)");

close();