- (Software installation) 00:00.00-00:02.56

  - (Install data acquisition and processing software.) 00:03.49-00:09.32

    - (Install the data acquisition system Cheetah 6.4, which includes libraries to interact with the Cheetah Acquisition System.) 00:10.25-00:28.19

    - (Install SpikeSort3D for spike sorting.) 00:38.60-00:53.98

    - (Install the NetComDevelopmentPackage.) 00:55.92-01:15.03

  - (Install Matlab. Make sure that Matlab is enabled in the Windows firewall.) 01:17.67-01:24.03

    - (Log in to a Matlab account. Choose the licence. Choose the version. Choose the operating system.) 01:26.05-01:58.05

  - (Download the following library for online event triggering from: http://github.com/leomol/cheetah-interface and extract files to the computer’s ‘Documents/Matlab’ folder. A copy of the code is provided in the accompanying supplemental materials.) 02:38.05-02:54.98

- (Initial data acquisition) 03:09.58-03:11.60

  - (Start data acquisition using Cheetah software.) 03:11.68-03:15.95

  - (Record a few minutes of spiking data to populate template waveforms.) 03:17.19-03:22.16

  - (Stop the data acquisition and perform spike sorting.) 03:22.86-03:26.05

    - (Open SpikeSort3D, click File | Menu | Load Spike File, and select a spike file from the folder with recorded data.) 03:27.99-03:38.01

    - (Click Cluster Menu and then AutoCluster using KlustaKwik, leaving the default settings and click Run.) 03:38.55-03:45.46

- (Closed-loop experiment) 03:46.40-03:49.19

  - (Resume data acquisition in Cheetah.) 03:49.35-03:52.38

  - (Open Matlab.) 03:52.45-03:55.09

    - (In the Matlab command window, execute ClosedLoop()) 03:55.25-03:59.37

    - (Load the spike information defined on the initial recording by clicking on Load, browsing to the recording folder, and selecting one of the spiking data files (.ntt, .nse)) 03:59.52-04:04.57

    - (Select one or multiple neurons that will trigger stimulation by clicking the checkbox under the plotted waveforms.) 04:05.19-04:10.47

    - (Define the minimum number of neurons that will trigger stimulation by typing an integer in the “min matches” text box; and define the time window in which spikes matching different waveforms are considered co-active by typing a number in the “window” text box.) 04:10.86-04:14.43

    - (Click Send to start. This will begin online triggering of events (tones as default) based on spiking activity of selected neurons.) 04:14.59-04:19.87