- 60031_screenshot_1

  - 6.1.2 (Open camera software and select the 40x objective) 00:00 – 00:05

  - (Image being captured) 00:05 – 00:20

  - (Export captured image as 8-bit TIFF file) 00:20 – 00:35

- 60031_screenshot_2

  - 6.2.2 (Open captured image in ImageJ) 00:00 – 00:12

  - (Click Analyze | Set Scale to calibrate the image according to microscope settings in pxl/μm) 00:12 – 00:35

  - (Click Plugins | NeuronJ to open NeuronJ plugin) 00:35 – 00:45

  - (Import captured image) 00:45 – 00:52

  - (Click Add tracings to begin tracing neurite) 00:52 – 00:57

  - (Trace the longest neurite of each neuron from the cell body to the tip of the growth cone) 00:57 – 01:21

  - (To measure traced neurite, click Measure tracings, check Display tracing measurements and Calibrate measurements in the dialog box, then click Run) 01:21 – 01:29

  - (Record the measured length of neurite) 01:29 – 01:36