# Submission ID #: 60092

# Scriptwriter Name: Anastasia Gomez

# Project Page Link: https://www.jove.com/account/file-uploader?src=18328108

# Title: Light-Induced Molecular Adsorption of Proteins using the PRIMO System for Micro-Patterning to Study Cell Responses to Extracellular Matrix Proteins

# Authors and Affiliations:

Cristina Melero*1, Aljona Kolmogorova*1, Paul Atherton1, Brian Derby2, Adam Reid3,4, Karin Jansen5, Christoph Ballestrem1

1Faculty of Biology, Medicine and Health. Division of Cell Matrix, Biology and Regenerative Medicine, Wellcome Trust Centre for Cell-Matrix Research, The University of Manchester, Manchester, UK

2School of Materials, The University of Manchester, Manchester, UK

3Blond McIndoe Laboratories, School of Biological Sciences, Faculty of Biology, Medicine and Health, Division of Cell Matrix Biology and Regenerative Medicine, The University of Manchester, Manchester Academic Health Science Centre, Manchester, UK

4Department of Plastic Surgery & Burns, Wythenshawe Hospital, Manchester University NHS Foundation Trust, Manchester Academic Health Science Centre, Manchester, UK.

5UMC Utrecht, Department of Pathology, Utrecht, The Netherlands.

*These authors contributed equally

# Corresponding Author:

# Christoph Ballestrem: christoph.ballestrem@manchester.ac.uk

# Karin Jansen: kjansen6@umcutrecht.nl

# Email addresses for Co-authors:

# Cristina Melero: cristina.melero@manchester.ac.uk

# Aljona Kolmogorova: aljona.kolmogorova@postgrad.manchester.ac.uk

# Paul Atherton: paul.atherton@manchester.ac.uk

# Brian Derby: brian.derby@manchester.ac.uk

# Adam Reid: adam.reid@manchester.ac.uk

Author Questionnaire:

1. Microscopy: Does your protocol involve video microscopy, such as filming a complex dissection or microinjection technique?

NO

Can you record movies/images using your own microscope camera?

YES

If no, JoVE will need to record the microscope images using our scope kit (through a camera port or one of the oculars). Please list the make and model of your microscope.

We can record movies but in any case the microscope we use for this protocol is a Nikon Eclipse Ti inverted.

2. Does your protocol include software usage?

YES

If yes, we will need you to record using screen recording software to capture the steps. If you use a Mac, QuickTime X also has the ability to record the steps.

3. Which steps from the protocol section below are the most important for viewers to see? Please list 4-6 individual steps using the step numbers listed in this document. This information is important to prepare your Videographer for your shoot. (You do not need to include steps that will be screen captured. Please do not list entire sections.) 4.1, 4.6, 5.1, 5.2, 6.3, 7.1

4. What is the single most difficult aspect of this procedure and what do you do to ensure success? Please list 1-2 individual steps using the step numbers listed in this document. (Please do not list entire sections.) 4.6

5. Will the filming need to take place in multiple locations? YES

If yes, how far apart are the locations?

Not greater than walking distance but yes going 2 floors upstairs and downstairs multiple times.

There is a lift that can be used to transport the filming equipment between these two different locations.

# Section - Introduction

Videographer: Interviewee Headshots are required. Take a headshot for each interviewee.

Authors, these headshots will be used for the JoVE Dedicated Author Webpage. Here is one example if you wish to take a look.

- REQUIRED Interview Statements: (Said by you on camera)  - All interview statements may be edited for length and clarity.

  - Cristina Melero: Micro-patterning is a very powerful tool because it allows studying cellular responses to defined biochemical and biophysical signals that they encounter in living organisms.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Aljona Kolmogorova: In contrast to other techniques, the described technique is more flexible for pattern generation. For example, it allows generation of gradients and micro-patterns of multiple proteins with very good reproducibility.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

OPTIONAL Interview Statements: (Said by you on camera)  - All interview statements may be edited for length and clarity.

  - Christoph Ballestrem: Our group uses LIMAP not only to study neuronal pathfinding but also how other cells respond to signals that have influence on cell proliferation, differentiation and migration.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Christoph Ballestrem: Micro-patterning is used to answer fundamental questions in cell biology. The gained knowledge can be used to modify existing devices in regenerative medicine, for example nerve repair after PNI (peripheral nerve injury).

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Paul Atherton/ Aljona Kolmogorova: When trying this technique for the first time, just follow the detailed protocol!

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Paul Atherton: Visual demonstration of this method is critical because it greatly facilitates understanding of key technical steps in the protocol.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

# Section - Protocol

- Design of Pattern Templates and Plasma Cleaning

# Author NOTE: Demonstrated by Cristina

  - Begin by designing templates for patterning on digital drawing software [1]. Select an image size of 1824 pixels in length and 1140 pixels in width [2], design the template [3], and save the template as an 8-bit Tiff file [4].

    - WIDE: Establishing shot of talent in front of a computer designing patterning templates.

    - SCREEN: 00:00 to 00:12 Appropriate image size being selected.

    - SCREEN: 00:12 to 00:54 Pattern template being designed.

    - SCREEN: 00:54 to 01:07 Template being saved.

  - Prior to patterning, clean and activate the surface with a plasma cleaner [1] using a process pressure of 1000 to 1300 millitorr and a power of 29.6 Watts for 1 to 5 minutes [2].

    - Talent using the plasma cleaner.

    - Added shot: CU of plasma inside plasma chamber.

  - Use a glass-bottom dish with a 20-millimeter inner well size and a glass thickness of 0.16 to 0.19 millimeters. Select either a 6-well or a single-well dish, depending on the number of conditions being tested [1].

    - Appropriate dishes for the experiment, 6-well and single-well.

- Passivation with PLL-PEG

# Author NOTE: Demonstrated by Aljona

  - Cut PDMS stencils under sterile conditions [1] to ensure that they fit in the inner glass bottom well [1B], remove the inner micro-well fillings with sterile forceps [2], and stick the stencils on the glass well [3].

    - Talent cutting stencils.

# 3.1.1B. Added shot: CU of Talent cutting stencil

    - Talent removing micro-well fillings.

    - Talent sticking stencils on well.

  - Prepare a 0.1 milligram per milliliters PLL-PEG solution (pronounce ‘P-L-L-peg’) in PBS and add 20 microliters to each microwell [1]. Then, incubate the dish at room temperature for 1 hour [2]. After incubation, remove 15 microliters of the PLL-PEG from each well [3 or 3B] and wash it 5 times with 20 microliters of PBS without letting it dry [4].

    - Talent adding PLL-PEG solution to a few wells.

    - Dish incubating at room temperature.

    - Talent removing PLL-PEG from a well.

# 3.2.3B. New shot: close up of PLL-PEG removal from well

    - Talent washing a well with PBS.

  - Prepare one micro-well to create a reference pattern by removing 18 microliters of PBS from a single well and adding 5 microliters of photo-initiator, leaving PBS in the remaining micro-wells [1]. Then, use a fluorescent highlighter to mark an empty inner glass well [2].

    - Talent removing PBS and adding photo-initiator, with the PLPP label visible in the shot.

    - Talent marking an empty inner glass well.

- System Calibration (Cristina)

  - Position the well above the objective [2] and adjust the objective focus [3] until the PRIMO logo and the tag line ‘Take care of your cells’ are in focus [4], then record the Z-position of the focal plane [5]. Videographer: This step is important!

    - NOTE: this one was moved to 3.3.2.

    - Talent positioning the well above the objective.

# 4.1.2B. Added shot: CU hand on joystick.

# 4.1.2C. Added shot: CU plate.

    - Talent adjusting the focus on the microscope.

# 4.1.3B. Added shot: CU of hand on focus knob.

    - SCREEN: The logo and tag line coming into focus.

    - Talent writing down the Z-position.

  - To create a reference pattern, visualize the edge of the micro-well with the photo-initiator using the transmitted light through the camera and choose the ROI symbol from the right-hand menu [1].

    - SCREEN: Edge of micro-well visualized and ROI symbol selected.

  - Select PRIMO to upload the desired pattern template, which will be projected on the ROI as a design unit [1]. Navigate to a peripheral region of the micro-well, select ‘Lock’ [2], and adjust the focus to the Z-position of calibration [3]. Then, select ‘Play’ to start photo-patterning the reference pattern [4].

    - SCREEN: 00:00 to 00:18 Desired pattern being uploaded and projected on the ROI.

    - SCREEN: 00:18 to 00:38 Peripheral region being located, ‘Lock’ selected.

    - Talent adjusting focus on the microscope. Videographer NOTE: Reuse shots 4.1.3. and 4.1.3B.

    - SCREEN: 00:38 to 00:43 ‘Play’ selected to start photo-patterning the reference pattern.

  - Wash the reference pattern micro-well 3 times with 20 microliters of PBS to remove the photo-initiator [1], then add 20 microliters of fluorescently-labeled extracellular matrix, or ECM, protein solution [2]. Incubate the dish at room temperature for 10 to 20 minutes, making sure to protect the reference well from light [3].

    - Talent washing the micro-well with PBS.

    - Talent adding the ECM protein to well.

    - Talent wrapping dish in aluminum foil and putting it down to incubate. Videographer: Obtain multiple reusable takes of this shot because it will be reused.

  - After the incubation, remove 18 microliters of protein solution [1] and wash the well 3 times with 20 microliters of PBS [2].

    - Talent removing the protein solution.

    - Talent washing the well with PBS.

  - Use an epifluorescence microscope to visualize the reference pattern [1]. Adjust the focus on the pattern edges through the camera path and record the Z-position according to the best focus [2]. This position will be the optimal laser focus used for subsequent patterning [3]. Videographer: This step is important and difficult!

    - Talent at the epifluorescence microscope. Videographer: Obtain multiple reusable takes of this shot because it will be reused. Videographer NOTE: Reuse shots 4.1.3. and 4.1.3B.

    - SCREEN: Reference pattern coming into focus. Author NOTE: Video capture 4.6.2 (recently uploaded)

    - Talent adjusting focus on the microscope and recording the Z-position.

- Software Set-up and Photopatterning (Aljona)

  - Under sterile conditions, remove 18 microliters of PBS from all the micro-wells [1], add 5 microliters of the photo-initiator to each well, and homogenize by pipetting up and down [2]. If patterning multiple proteins in the same micro-well, upload all sets of desired pattern templates simultaneously [3]. Then, select the number of rows, columns, and dose [4]. Videographer: This step is important!

    - Talent removing PBS from a few wells.

    - Talent adding photo-initiator to a few wells and homogenizing by pipetting up and down

    - SCREEN: Multiple pattern templates being uploaded.

    - SCREEN: Number of rows, columns, and dose being selected. Author NOTE: 5.1.3 and 5.1.4 are joint together in the same screen capture because both steps are done as a whole.

  - Save the template configuration as a file in the software [1]. Navigate to the area where the reference pattern was produced [2] and adjust the focus to the optimal Z-position [2B]. Then, select ‘Play’ to initiate photo-pattering [3]. The design units turn from red to blue as the photo-patterning is completed [4]. Videographer: This step is important!

    - SCREEN: Template configuration being saved.  Author NOTE: 2nd round action being de-selected and template configuration being saved

    - Talent locating reference pattern area and adjusting focus to the optimal Z-position.

# 5.2.2B. Added shot: CU of talent adjusting focus to optimal Z position

    - Talent selecting ‘Play’ on the screen and the laser working.  Videographer NOTE: not possible to film the laser firing.

    - SCREEN: Design units turning from red to blue. Author NOTE: 2nd round action being hidden, transmitted light being switched off and Play being selected and design units turning from red to blue. Edit screen capture and cut after 01:01

- Protein Incubation and Blocking Unspecific Binding Sites (Cristina)

  - Remove the photo-initiator from the micro-wells by washing them 3 times with 20 microliters of PBS [1]. After the last wash, remove 18 microliters of PBS from each micro-well and add 20 microliters of ECM protein solution [2]. Incubate the dish at room temperature for 20 to 30 minutes, making sure to protect it from light [3].

    - Talent washing wells with PBS. Videographer: Obtain multiple reusable takes of this shot because it will be reused.

    - Talent removing PBS from a few wells and adding the ECM protein.

# 6.1.2B. Added shot: Talent adding the ECM protein

    - Use 4.4.3.

  - Next, remove 15 microliters of the ECM protein solution [1] and wash the microwells 3 times using 20 microliters of PBS [2].

    - Talent removing ECM solution from a few wells.

    - Use 6.1.1.

  - To prevent cross-binding, add 20 microliters of PLL-PEG or BSA to the micro-wells [1] and incubate the dish at room temperature for 1 hour in the dark [2]. Then, remove 15 microliters of the blocking agent [3] and wash all micro-wells 3 times with 20 microliters of PBS [4]. Videographer: This step is important!

    - Talent adding blocking agent to a few wells.

    - Use 4.4.3.

    - Use 6.1.1. Videographer NOTE: 6.3.2 – 6.3.3 shots captured as a different action from 4.4.3 and 6.1.1.

    - Added shot: Talent washing all wells 3 times.

  - Remove 18 microliters of PBS and add 5 microliters of the photo-initiator to each micro-well [1], ensuring that the photo-initiator is homogeneous on the whole surface of the micro-wells [2].

    - Talent removing PBS from a few wells and adding the photo-initiator.

    - Homogeneous photo-initiator on the micro-well surface.

- Second Round of Photopatterning and Protein Incubation (Aljona)

  - Navigate to the first micro-well and load the template configuration saved previously, then select the actions to be patterned for the second round of photopatterning, making sure to deselect actions from the first round [1]. Videographer: This step is important!

    - SCREEN: First micro-well located, template configuration being loaded, and actions being selected and deselected. Author NOTE: First micro-well located, template configuration being loaded, 1st round action being deselected and hidden, transmitted light switched off and Play selected. Screen capture Edit to cut after 01:32

  - After photopatterning, remove PLPP by washing with PBS [1], then incubate for 20 minutes [2] with the second ECM protein solution [3] and wash the micro-wells 3 times with 20 microliters of PBS [4]. Use an epifluorescence microscope to visualize and image the printed patterns [5].

    - Wash PLPP with PBS

    - Add second ECM protein solution

    - Incubate with second ECM protein solution for 20 min Videographer NOTE: Reuse shot 6.1.3.

    - Wash the micro-wells 3 times with PBS

    - SCREEN: Resulting cross-pattern. Author NOTE: video capture with new numbering 7.2.5. recently uploaded.

  - When ready to plate cells, add 1 milliliter of medium with cells to the inner glass well, making sure to cover all micro-wells [1], and place the culture dish in a 37 C and 5% carbon dioxide incubator [2].

    - Talent adding cells to a well and distributing them on all the surface.

    - Talent placing dish in incubator.

# Section – Results

- Results: Micropatterning for the Study of Cell Responses to Extracellular Matrix Proteins

  - In order to ensure that the generated patterns are a precise representation of the template [1], fluorescence intensity measurements are obtained along the stripes and from a corresponding background region above each stripe [2].

    - Figure 1 A and B. Video Editor: Emphasize B when VO says ‘generated patterns and A when VO says ‘template’.

    - Figure 1 C – F. Video Editor: Emphasize D and F.

  - Reproducible, defined micropatterns are achieved ranging from 20 to 2 micrometers in width [1], but an edge effect can be observed when printing features 20 micrometers or wider [2].

    - Figure 1 B and D. Video Editor: Emphasize the spikes in D.

    - Figure 1 B and D. Video Editor: Emphasize the top line in B and the first spike in D with the two peaks.

  - Defined protein concentrations are attained by either incubating with varying concentrations of protein or varying the laser dose that is used to cleave the anti-adhesive film [1].

    - Figure 7. Video Editor: Emphasize A and B when VO says ‘varying concentrations of protein’ and C and D when VO says ‘varying the laser dose that is used to cleave the anti-adhesive film’.

  - Laser intensity is proportional to the greyscale level of the template, generating gradients of UV illumination [1]. The measurement of the fluorescence intensity profile along the gradient stripe is linear in the pattern template and in the generated gradient pattern [2].

    - Figure 8.

    - Figure 8. Video Editor: Emphasize A and B when VO says ‘pattern template’ and C and D when VO says ‘generated gradient pattern’.

  - This protocol can be used to create micropatterns with multiple proteins in the same microwell, such as cross-patterns with horizontal stripes of fibronectin and vertical stripes of laminin [1], but a blocking step is necessary to prevent cross-binding of proteins [2].

    - Figure 9. Video Editor: Emphasize B, C, F, and G.

    - Figure 9. Video Editor: Zoom in on C and emphasize the second peak with both blue and purple lines when VO says ‘cross-binding’.

  - The generated cross-patterns can be subsequently used for cellular assays with CAD (pronounce ‘cad’) cells or rat dorsal-root ganglion neurons [1].

    - Figure 10. Video Editor: Emphasize A and B when VO says ‘CAD’ cells and C when VO says ‘rat dorsal-root ganglion neurons’.

# Section - Conclusion

- Conclusion Interview Statements: (Said by you on camera) - All interview statements may be edited for length and clarity.

  - Cristina Melero: When attempting this protocol, the most important things to remember are [1] proper system calibration [2], not letting the microwells dry out during the multiple washing steps [3] and using the right parameters for protein incubation and blocking [4].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

    - Use 4.6.1.

    - Use 3.2.4.

    - Use 6.3.1.

  - Aljona Kolmogorova: Analysis of cellular behaviour often requires microscopy (fluorescence, AFM). Further analysis can comprise other methods such as biochemistry and gene expression profiling.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.