# Submission ID #: 60125

# Scriptwriter Name: Bridget Colvin

Project Page Link: http://www.jove.com/files_upload.php?src=18338173

# Title: Biaxial Basal Tone and Passive Testing of the Murine Reproductive System Using a Pressure Myograph

Authors and Affiliations:  Shelby E. White, Cassandra K. Conway, Gabrielle L. Clark, Dylan J. Lawrence, Carolyn L. Bayer, and Kristin S. Miller

Department of Biomedical Engineering, Tulane University

# Corresponding Author:

# Kristin S. Miller

# Kmille11@tulane.edu

Email addresses for Co-authors:

Swhite18@tulane.edu

cconway2@tulane.edu

gclark2@tulane.edu

dlawren@tulane.edu

Carolynb@tulane.edu

Author Questionnaire:

1. Microscopy: Does your protocol involve video microscopy? Y, Leica MZ6

2. Does your protocol demonstrate software usage? Y

3. Which steps from the protocol section below are the most important for viewers to see?

n/a

4. What is the single most difficult aspect of this procedure and what do you do to ensure success?

n/a

5. Will the filming need to take place in multiple locations (greater than walking distance)? N

# Section - Introduction

Videographer: Interviewee Headshots are required. Take a headshot for each interviewee.

- REQUIRED Interview Statements (Said by you on camera): All interview statements may be edited for length and clarity.

  - Gabrielle Clark: This protocol investigates the biaxial mechanical properties of the reproductive organs, elucidating the contribution from the smooth muscle cells and the passive matrix [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

  - Cassandra Conway: This technique maintains organ geometry and native smooth muscle cell-matrix interactions, permitting subjection of the organ to a physiologic range of pressures and axial extensions [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

OPTIONAL Interview Statements: (Said by you on camera) - All interview statements may be edited for length and clarity.

  - Shelby White: Investigating the organ biaxial function under physiologically relevant loads may aid in a better understanding of the etiology of reproductive pathologies [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

  - Dylan Lawrence: Biaxially testing the cervix and vagina provides insight into the fields of women’s health and biomechanics. Similar methods are utilized to study blood vessels, the esophagus, and large intestines [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

  - Dylan Lawrence: When working with a new organ, it may take time to finalize the technical details, such as how to best suture the tissue or how to determine the unloaded length [1].

    - INTERVIEW: Above Talent speaking the statement above in an interview-style shot, looking slightly off-camera

Ethics title card: (for human subjects or animal work, does not count toward word length total)

  - Procedures involving animal subjects have been approved by the Institutional Animal Care and Use Committee (IACUC) at Tulane University.

# Section - Protocol

- Reproductive System Dissection and Cannulation

# Video Editor: The scope footage is upside down (done to help staff see the scope)

  - To collect the reproductive system from the experimental animal, place the mouse on an absorbent pad under a dissecting microscope [1-TXT] and use angled tweezers and scissors to lift the skin around the abdomen [2].

    - WIDE: Talent placing mouse onto pad Videographer: More Talent than mouse in shot TEXT: See text for euthanasia details

    - SCOPE: Skin being lifted

  - Make an initial incision at the base of the abdomen above the pubic bone shallow enough to not puncture the abdominal muscle wall [1] and continue the incision superiorly toward the rib cage and deep through the abdominal muscles [2].

    - SCOPE: Initial incision being made

    - SCOPE: Incision being continued

  - After removing the superficial fat [1] and carefully cutting the pubic symphysis [2], use straight tweezers to grasp the bladder to create tension [3] and use blunt dissection to separate the surrounding tissue from the vagina [4].

    - SCOPE: Shot of abdominal cavity without superficial fat

    - SCOPE: Shot of cut pubic bone

    - SCOPE: Bladder being grasped

    - SCOPE: Tissue being dissected

  - Then cut the base of the reproductive system to remove the tissues from the body cavity [1].

    - SCOPE: Base being cut

  - After determining the appropriate size cannula for the harvested system [1-TXT], mount the cervical side on the force transducer portion of the cannulation device [2] and mount the opposite end of the organ on the micrometer portion of the device [3].

    - Talent selecting cannula TEXT: See text for suggested cannula size details

    - SCOPE: Cervical side being mounted

    - SCOPE: Vaginal or uterine side being mounted

  - Then tighten both ends with sutures [1].

    - SCOPE: Suture being placed

- Basal Tone Mechanical Testing

  - To find the unloaded length of the mounted tissue, first stretch the organ so that the wall is not in tension [1].

    - WIDE: Talent stretching organ

  - For the vagina, observe the grooves on the vaginal wall [1]. For the cervix, cut immediately below the ink dots located above and below the central cervix mark [2].

    - SCOPE: Shot of grooves on vaginal wall

    - SCOPE: Shot of ink dots above and below central cervix mark

  - Use calipers to measure the length of the system from suture to suture [1] and increase the pressure of the myograph system from 0 to 10 millimeters of mercury in 1-millimeter of mercury increments [2-TXT].

    - Tissue being measured

    - Talent increasing pressure TEXT: See text for myograph setup details

  - The pressure in which the organ is no longer collapsed can be determined as the largest jump in the outer diameter at a given pressure [1].

    - SCREEN: 60125_Screenshot1: JoVE Video Editor please emphasize data line at 44 mg

  - After recording the pressure and the outer diameter, note these data as the first point wherein the organ is not collapsed and zero the force [1].

    - Talent noting data and/or zeroing force

  - To find the experimental in vivo stretch, adjust the organ to the estimated in vivo length while at the unloaded pressure [1] and click Start to assess the pressure vs. force values [2] for the pressure values ranging from the unloaded pressure [3] to the maximum pressure [4].

    - Organ being adjusted

    - Talent clicking Start, with monitor visible in frame

    - SCREEN: 60125_Screenshot2: JoVE Video Editor please emphasize data points at 0 mm Hg

    - SCREEN: 60125_Screenshot2: JoVE Video Editor please emphasize data points at 18 mmHg

  - Then click Stop and save the file [1].

    - Talent clicking Stop

  - For pressure-diameter pre-conditioning, set the pressure to 0 millimeters of mercury, the length to the experimental in vivo length, and the gradient to 1.5 mmHg/s [1].

    - SCREEN: 60125_Video1: pressure being set, length being set, gradient being set

  - Run a sequence that takes the pressure from 0 millimeters of mercury to the maximum pressure plus the unloaded pressure, holds for 30 seconds, and returns to 0 millimeters of mercury with an additional 30-second hold period [1].

    - SCREEN: 60125_Screenshot3: JoVE Video Editor please emphasize Mean Pressure graph

  - After repeating this sequence for a total of 5 cycles [1], click Stop and save the file [2].

    - Shot of pressure being applied to tissue

    - SCREEN: 60125_Video2: Stop being clicked

  - For force-length pre-conditioning, enter 1/3 of the maximum pressure plus the unloaded pressure for both the inlet and outlet pressures [1] and adjust the organ to minus 2% of the in vivo length [2].

    - SCREEN: 60125_Video3: 1/3 maximum pressure being entered plus unloaded pressures

    - Organ being adjusted

  - Click Start and adjust the length to plus 2% in vivo length and back down to minus 2% at a 10-micrometer/second rate [1].

    - SCREEN: 60125_Screenshot4

  - Repeat the axial extension for a total of 5 cycles [1] before clicking Stop and saving the file [2].

    - Shot of axial extension being applied

    - SCREEN: 60125_Video4: Stop being clicked

  - For pressure diameter testing of the in vivo length, click Start and adjust the organ to the in vivo length [1], set the pressure to 0 millimeters of mercury, and set the gradient to 1.5 millimeters of mercury/second [2].

    - Organ being adjusted

    - SCREEN: 60125_Video5: Pressure being set and gradient being set

  - Increase the pressure from 0 millimeters of mercury to the maximum pressure before bringing the pressure back down to 0 millimeters of mercury with a 20-second hold period [1].

    - SCREEN: 60125_Screenshot5

  - Then repeat the test for 5 cycles before stopping the system and saving the data [1].

    - Shot of pressure being applied to organ

  - For force-length testing of 1/3 of the maximum pressure plus the unloaded pressure, set the pressure to 1/3 of the maximum pressure plus the unloaded pressure [1] and adjust the organ to minus 2% of the in vivo length [2].

    - SCREEN: 60125_Video6: Pressures being set

    - Organ being adjusted

  - After clicking Start, stretch the organ to plus 2% of the in vivo length and back to minus 2% of the in vivo length at a rate of 10 micrometers/second [1].

    - SCREEN: 60125_Screenshot6

  - Then repeat the test for a total of 3 cycles before stopping the system and saving the data [1].

    - Shot of organ being stretched

  - After the last test, remove the KRB (K-R-B) testing medium [1-TXT] and wash the organ with calcium-free KRB medium [2].

    - Medium being removed TEXT: KRB: Krebs Ringer Buffer

    - Organ being washed, with calcium-free KRB container visible in frame

  - After the wash, incubate the organ with fresh calcium free KRB solution supplemented with 2-millimolar EGTA (E-G-T-A) for 30 minutes [1] before replacing the treatment solution with fresh calcium-free KRB [2-TXT].

    - Medium being added, with KRB and EGTA containers visible in frame

    - Talent adding medium to organ, with KRB container visible in frame TEXT: EGTA: egtazic acid

- Passive Mechanical Testing

  - At the completion of the basal tone testing, for pressure diameter pre-conditioning, click Start [1] and set the pressure to 0 millimeters of mercury, the length as the estimated in vivo length, and the gradient to 1.5 millimeters of mercury/second [2].

    - WIDE: Talent clicking start, with monitor visible in frame

    - SCREEN: 60125_Video7: pressure, length, and gradient being set

  - Begin running a sequence that takes the pressure from 0 of millimeters of mercury to the maximum pressure and back to 0 millimeters of mercury [1].

    - SCREEN: 60125_Screenshot7

  - Repeat this process through 5 cycles with a 30-second hold time [1].

    - Organ being pressure diameter pre-conditioned

  - For force-length preconditioning, adjust the organ to the in vivo length [1] and manually enter the unloaded pressure for both pressures [2].

    - Organ being adjusted

    - Talent entering pressure, with monitor visible in frame

  - Click Start and set the gradient to 1.5 millimeters of mercury and the pressure to 1/3 of the maximum [1].

    - SCREEN: 60125_Screenshot8

  - Stretch the organ up to plus 2% and back down to minus 2% stretch at 10 micrometers/second and repeat the cycle for a total of 5 times [2].

    - Organ being force-length preconditioned

  - For pressure-diameter testing, with the organ at minus 2% of the experimentally determined in vivo length and the pressure at 0 millimeters of mercury [1], click Start and increase the pressure from 0 millimeters of mercury to the maximum pressure and back to 0 millimeters of mercury [2].

    - Shot of organ at -2% experimentally-determined in vivo length

    - SCREEN: 60125_Video8: Start being clicked

  - Hold the 2-to-0 millimeters of mercury-step for 20 seconds [1] and repeat the cycle 5 times [2].

    - SCREEN: 60125_Screenshot9

    - Shot of pressure-diameter testing

  - For force-length testing, set the pressure to a nominal pressure [1] and adjust the organ to minus 2% of the in vivo length [2].

    - SCREEN: 60125_Video9: Pressure being set

    - Organ being adjusted

  - Stretch the organ up to plus 2% of the in vivo length and back to minus 2% of the in vivo length at a rate of 10 micrometers/second [1].

    - SCREEN: 60125_Screenshot10

  - After a total of 3 cycles, save the data [1] and repeat the force-length testing for 1/3 of the maximum pressure, 2/3 of the maximum pressure, and at the maximum pressure [2].

    - Force length of organ being tested

    - Talent at computer, setting up next test preparing to clean-up.

# Section – Results

- Results: Representative Biaxial Basal Tone and Passive Murine Reproductive System Testing

  - For a successful analysis of the mechanical properties of the female reproductive organs, it is imperative to explant the uterine horns to the vagina without any defects [1].

    - LAB MEDIA: Figure 1A

  - Depending on the organ type, the cannula size will vary [1].

    - LAB MEDIA: Figure 2

  - The cannulation must be performed so that the organ cannot move during the experiment but without damaging the wall of the organ during the procedure [1].

    - LAB MEDIA: Figure 3

  - The pressure myograph system can be used to monitor various aspects of the organ as it undergoes mechanical testing [1] and the ultrasound system can be used to measure the thickness of the organs in the unloaded state with and without basal tone [2].

    - LAB MEDIA: Figure 5B

    - LAB MEDIA: Figures 6A, 6B, 6C, and 6D

  - After mechanical testing, the tangent moduli may be calculated for the circumferential and axial directions [1].

    - LAB MEDIA: Table 2

  - Both basal tone testing and passive testing yield key mechanical properties of the reproductive tract [1] with and without the contractile contribution of smooth muscle cells [2].

    - LAB MEDIA: Figure 7: JoVE Video Editor please emphasize left column of graphs

    - LAB MEDIA: Figure 7: JoVE Video Editor please emphasize right column of graphs

  - Scaling between the organs requires a few adjustments to the protocols, as the cervix and vagina experience different loads in vivo [1].

    - LAB MEDIA: Table 1

# Section - Conclusion

- Conclusion Interview Statements: (Said by you on camera) - All interview statements may be edited for length and clarity.

  - Cassandra Conway: (Step: 2.2.-2.9.) It is key to perform the dissection quickly - to maintain the viability of smooth muscle cells - but carefully - to not puncture the desired organ [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

  - Shelby White: Opening angle experiments, which measure the residual strain of the organ, may be performed after the mechanical testing procedures as well as various histological and biochemical assays [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

  - Gabrielle Clark: After its development and investigation of the smooth muscle cell basal contribution, this technique also permits evaluating the reproductive system smooth muscle maximum contractile response under physiologically relevant loads [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera