# Submission ID #: 60171

# Scriptwriter Name: Bridget Colvin

Project Page Link: http://www.jove.com/files_upload.php?src=18352203

Title: A Porcine Corneal Endothelial Organ Culture Model Using Split Corneal Buttons

Authors and Affiliations: Daniel A. Wenzel1, Berenike C. Kunzmann2, Nils A. Steinhorst1, Martin S. Spitzer1, and Maximilian Schultheiss1

1Department of Ophthalmology, University Medical Center Hamburg-Eppendorf (UKE)

2Center of Ophthalmology, University Eye Hospital Tübingen

# Corresponding Author:

# Daniel A. Wenzel

# danielwenzel@mailbox.org

Email addresses for Co-authors:

berenike.kunzmann@med.uni-tuebingen.de

n.steinhorst@uke.de

m.spitzer@uke.de

maximilianschultheiss@gmail.com

Author Questionnaire:

1. Microscopy: Does your protocol involve video microscopy? Y, Zeiss Stemi 2000

2. Does your protocol demonstrate software usage? Y

3. Which steps from the protocol section below are the most important for viewers to see?

2.1., 3.3.-3.5., 3.7., 4.1.

4. What is the single most difficult aspect of this procedure and what do you do to ensure success?

3.4.it is important to us a standardized trephine and to remove the outer layers evenly using a scalpel in order to obtain split corneal buttons with the correct depth.

5. Will the filming need to take place in multiple locations (greater than walking distance)? N

# Section - Introduction

Videographer: Interviewee Headshots are required. Take a headshot for each interviewee.

- REQUIRED Interview Statements (Said by you on camera): All interview statements may be edited for length and clarity.

  - Daniel A. Wenzel: This protocol presents the first ex vivo research model allowing organo-typical, long-term cultivation of corneal endothelial cells achieved with the preparation of split corneal buttons [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

  - Maximilian Schultheiss: By partially removing the outer corneal layers, endothelial cell loss induced by stromal swelling is significantly reduced, allowing the cultivation of corneal endothelial cells for at least 15 days [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

OPTIONAL Interview Statements: (Said by you on camera) - All interview statements may be edited for length and clarity.

  - Maximilian Schultheiss: Therefore, this model is valuable for investigating the effects of specific substances and techniques of interest, such as irrigation solutions or ophthalmic viscoelastic devices, on the corneal endothelium [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Ethics title card: (for human subjects or animal work, does not count toward word length total)

  - Procedures involving animal subjects follow the ethical guidelines of the University Medical Center Hamburg-Eppendorf (UKE)

# Section - Protocol

- Porcine Eye Preparation

  - After receiving pig eyes that were removed shortly postmortem [1-TXT], use eyes scissors and colibri forceps to remove all of the orbital adnexes [2].

    - WIDE: Talent entering lab with specimen container TEXT: Process w/in few h of acquisition

    - Adnex(es) being removed Videographer: Important step TEXT: Discard eyes w/ obvious external damage

  - Place 5-6 eyes into 5%-iodine-PBS solution for a total of 5 minutes to properly disinfect the eye surfaces [1] with careful stirring once a minute to ensure that the surfaces are fully submerged to achieve a complete disinfection [2].

    - Talent placing eyes into solution, with solution container visible in frame

    - Eyes being stirred

  - After 5 minutes, transfer the disinfected eyes to a cup of pure PBS [1] and carefully stir to thoroughly wash the iodine-PBS-solution from the eye surfaces [2].

    - Talent adding eyes to PBS, with PBS container visible in frame

    - Eyes being stirred

- Dissection and Incubation

  - Before beginning the dissection, use a blunt catheter to fill a 5-milliliter syringe with 2 milliliters of freshly thawed fetal calf serum [1] and strain the serum through a 0.22-micrometer filter into 80 milliliters of dextran-free culture medium [2-TXT].

    - WIDE: Talent filling syringe

    - Talent filtering FCS into medium, with medium container visible in frame TEXT: See text for all medium/solution preparation details

  - Gently agitate the mixture to achieve a homogenous distribution of the serum [1] and add 3 milliliters of the medium to each well of a 12-well cell culture plate [2].

    - Solution being agitated

    - Talent adding medium to well(s)

  - When the plate is ready, transfer an eye bulb into an eye bulb holder under an ophthalmic surgical microscope with the cornea facing up [1] and use a syringe filled with 0.9% sodium chloride to apply slight suction to the eye over the eye bulb holder to fix the eye in place [2].

    - Talent placing bulb into holder under microscope

    - SCOPE: Suction being applied to eye Videographer: Important step

  - Use a trephine containing a standardized inlay to cut superficially into the central cornea no deeper than 300 micrometers [1] and use colibri forceps and a single-use scalpel with a triangular blade to cut and remove the partially trephined fragment of the cornea horizontally through the stroma [2-TXT].

    - SCOPE: Cornea being trephined Videographer: Important/difficult step

3.4.1.A Cornea being trephined – different angle from camera.

    - SCOPE: Cornea piece being removed Videographer: Important/difficult step TEXT: Discard fragment Videographer NOTE: Please use 3rd take.

  - Superficially place a 10-0 suture into the stroma to allow differentiation of the endothelial from the stromal side during the experiments without penetrating the corneal endothelium [1-TXT] and use a trephine without an inlay to advance the trephine cut to the full depth until the anterior eye chamber is reached [2].

    - SCOPE: Suture being placed Videographer: Important step TEXT: Discard eye bulb if endothelium penetrated

    - SCOPE: Eye being fully trephined Videographer: Important step

3.5.2.A eye being fully trephined – different angle from camera.

  - A distinct drop in resistance and fluid leaking from the anterior eye chamber can be perceived after complete penetration of the cornea [1].

    - SCOPE: Shot of fluid leaking decrease

  - Place the obtained split corneal button containing part of the stroma, the Descemet’s membrane, and the corneal endothelium, into one well of the 12-well plate with the tagged side facing down [1-TXT].

    - Button being placed into well Videographer: Important step TEXT: Caution: If endothelial side faces down, endothelium may get damaged

  - Then label each well on the culture plate with an individual number for every split corneal button for identification during follow ups [1].

    - Plate being labeled

  - When all of the corneal buttons have been collected, place the 12-well plate in a cell culture incubator at 37 degrees Celsius, 5% carbon dioxide, and 95% humidity [1], replacing the supernatant on day 8 if a 7-day incubation period is exceeded [2].

    - Talent placing plate into incubator Videographer NOTE: No Slate

    - Talent adding medium to well, with medium container visible in frame

- Microscopy and Endothelium Examination

  - To perform an unstained examination, add 3 milliliters of hypotonic balanced salt solution to each well of a 12-well plate [1] and carefully place a single split corneal button into each well to induce swelling of the corneal endothelial cells and to improve the visibility of the cells for counting [2].

    - WIDE: Talent adding hBSS to well(s), with hBSS container visible in frame

    - Button being placed into well

  - After 1-2 minutes, use a microscope camera to acquire at least three images of three different areas of the endothelium to obtain a representative impression of the actual condition of the corneal endothelium [1] and add a scale bar with the true to scale length of 100 micrometers for later analysis [2].

    - Talent at microscope, imaging cell(s), with monitor visible in frame as possible

    - LAB MEDIA: Figure 5A

  - To prevent osmotic damage, transfer the split corneal button from the solution after a maximum of 5 minutes back into the culture medium [1].

    - Talent placing button into medium plate, with hBSS plate visible in frame

  - To perform a stained examination, place the split corneal buttons in individual Petri dishes endothelial side up [1] and use a pipette to slowly drop 0.25% trypan blue solution onto each corneal endothelium for 90 seconds per specimen [2].

    - Talent placing button(s) into dish(es)

    - Trypan being dripped onto cornea Videographer: Important step

  - Next, carefully rinse the buttons three times in 0.9% sodium chloride in a small glass beaker [1] before using a pipette to slowly drop 0.2% alizarin red S solution onto the endothelia for 90 seconds [2].

    - Button(s) being placed into beaker, with NaCl container visible in frame

    - Alizarin red being added to corneal button

  - Then image the samples as just demonstrated [1].

    - Talent at microscope, imaging button

- Corneal Endothelium Image Processing

  - To prepare the images for cell density and morphology assessment, open the images in an appropriate graphics editing software program [1] and project a square with a true to scale side length of 100 micrometers on the image [2].

    - WIDE: Talent opening file(s), with monitor visible in frame

    - SCREEN: screenshot_1

  - To crop the scale bar, use the Rectangular Marquee Tool to border the scale bar and select Edit and Crop [1].

    - SCREEN: screenshot_2

  - Click File and New and select Clipboard [1]. Note the corresponding pixels for the other pictures and click OK [2].

    - SCREEN: screenshot_4

    - SCREEN: screenshot_5

  - Click File and New again and insert the width and length of the counting square in pixels the popup window according to the length of the scale bar [1].

    - SCREEN: screenshot_6

  - Select White as the background color [1] and click OK to Select All and Copy the selected image [2].

    - SCREEN: screenshot_7

    - SCREEN: screenshot_8

  - Select the image of the corneal endothelium and Paste the square on the corneal endothelium [1].

    - SCREEN: screenshot_9

  - Select the square layer and set the Opacity to 30% [1].

    - SCREEN: screenshot_10

  - Then click OK and save the image with the projected square with a true to scale side length of 100 micrometers [1-TXT].

    - SCREEN: screenshot_11 TEXT: Repeat for each image

- Corneal Endothelial Cell Density and Morphological Parameter Analysis

  - To assess the endothelial cell density, open the images with the projected squares in ImageJ [1] and open the CellCounter PlugIn [2].

    - WIDE: Talent opening image(s) in ImageJ, with monitor visible in frame

    - SCREEN: screenshot_13

  - To count the cells within the square, click Initialize [1] to count the endothelial cells on two sides of the square and record the result [2-TXT].

    - SCREEN: screenshot_14

    - SCREEN: screenshot_15 TEXT: Analyze ≥6 squares/cornea to determine average corneal endothelial cell density per square

  - To assess the morphological parameters, count the joint meeting of at least 4 cells/cell border [1] … the characteristic rosette-shaped appearance [2] … and the alizarin red areas within the projected squares [3].

    - LAB MEDIA: Figure 4A image: JoVE Video Editor please add/emphasize 1, 2, 3, 4 texts

    - LAB MEDIA: Figure 4B image: JoVE Video Editor please add/emphasize dotted circle

    - LAB MEDIA: Figure 4C image: JoVE Video Editor please add/emphasize dashed circles

# Section – Results

- Results: Representative Corneal Button Culture Analyses

  - Over a period of 15 days, split corneal buttons demonstrate a steady decline in the endothelial cell density, with an average weekly percentage endothelial cell loss of 4.90% [1].

    - LAB MEDIA: Figure 3: JoVE Video Editor sequentially emphasize day 1, day 8, and day 15 data boxes

  - Staining of the endothelial cell layer on day 15 allows morphological analysis of the corneal cells [1], with reformation figures making up approximately 7% of the merging cell borders [2], a median of approximately 1 rosette formation/millimeter-squared per sample [3], and about 13 punctual cell losses/millimeter squared [4].

    - LAB MEDIA: Figure 4: JoVE Video Editor please emphasize top row of images

    - LAB MEDIA: Figure 4: JoVE Video Editor please emphasize data box in reformation figures graph

    - LAB MEDIA: Figure 4: JoVE Video Editor please emphasize data box in rosette formation graph

    - LAB MEDIA: Figure 4: JoVE Video Editor please emphasize data box in alizarin red graph

  - Here representative images of the corneal endothelium during microscopic evaluation in hypotonic balanced salt solution [1] and after staining with trypan blue and alizarin red S can be observed [2].

    - LAB MEDIA: Figure 5: JoVE Video Editor please emphasize left image

    - LAB MEDIA: Figure 5: JoVE Video Editor please emphasize right image

  - If the split corneal buttons are cultivated with the endothelial side facing down, extensive endothelial cell damage is to be expected [1].

    - LAB MEDIA: Figure 6A

  - Non-split corneal buttons suffer significantly increased endothelial cell loss due to stromal swelling, causing Descemet’s membrane folding over 15 days of cultivation [1], whereas split corneal buttons exhibit a largely preserved corneal endothelium after 15 days of cultivation [2], as evidenced by scattered punctual alizarin red stained areas indicative of single destroyed cells [3].

    - LAB MEDIA: Figure 6B: JoVE Video Editor

    - LAB MEDIA: Figure 6C: JoVE Video Editor

    - LAB MEDIA: Figure 6C: JoVE Video Editor please emphasize red stained dots

# Section - Conclusion

- Conclusion Interview Statements: (Said by you on camera) - All interview statements may be edited for length and clarity.

  - Daniel Wenzel: To minimize cell loss, care should be taken to not touch the corneal endothelium throughout any of the steps of this protocol [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera (Step 3.7.)