60388_screenshot_1

Screenshot commentary for steps 7.3–7.5

7.3 (0:00 – 0:55)

- 0:00 – 0:14 Select track for AuNCs imaging

- 0:14 – 0:17 Select laser 405 nm for AuNCs track

- 0:17 – 0:24 Set power of 405 nm laser to 2 %

- 0:24 – 0:31 Select 405 nm beam splitter for invisible light

- 0:31 – 0:35 Activate fluorescence detector for AuNCs track

- 0:35 – 0:55 Set the wavelength detection interval to 650–760 nm

  - (0:55 – 1:37)

- 0:55 – 1:09 Set resolution to 2048 × 2048 pixels

- 1:09 – 1:18 Set pixel dwell time to 4.12 µs (speed 4)

- 1:18 – 1:23 Set averaging number to 2

- 1:23 – 1:30 Set pinhole size to 1 Airy unit

- 1:30 – 1:37 Change the acquisition mode to photon counting

7:5 (1:37 – 2:19)

- 1:37 – 1:42 Select track for DIC imaging

- 1:42 – 1:49 Select laser 488 nm for DIC track

- 1:49 – 1:57 Set power of 488 laser to 0.7 %

- 1:57 – 2:04 Switch on transmitted light detection

- 2:04 – 2:19 Select 488 nm beam splitter for visible light