Screenshot Summary:

60423_screenshot_1

2.1 Open the Graphic User Interface (GUI) of the device and press the ‘START’ button on laptop PC or tablet. Then select ‘Open’ to use an existing protocol or press ‘New’ to create a new one. 0:02-0:13

60423_screenshot_2

2.2 Choose the ‘Control’ button from left side of the user interface to add new steps. Name the steps and select the valves to be opened or closed. Input centrifugal speed and pump speed, then choose action triggers for each step. The settings for each step are summarised in Table 1. Use the ‘forward’ and ‘backward’ button to navigate through each step. When finished, save the protocol. 0:01-0:53

60423_screenshot_3

3.2 Connect the GUI to the processing device and press the ‘Connect’ button. Then download the saved program. When the green ‘Play’ button lights up, the protocol is ready to start. 0:02-0:20

60423_screenshot_4

4.2 At the end of step 6 (Table 1), the process will pause as air in the now emptied bag triggers the bubble sensor. Press ‘Pause’ to resume the processing if the sensor is triggered by a bubble in the line. Press ‘Play’ to move the process to the next step if the bag is indeed empty. 0:02-0:15