- 60590_screenshot_1

2.5. Within the Measurement and Automation program associated with the digital I/O card, determine the Device ID for the I/O card by selecting the Devices and Interfaces menu. 00:05 – 00:46

2.6. Verify that the I/O detects when the IR beam is broken using the Test Panels… option in the program. Note the channel and port IDs for each set of sensors. 00:47 – 01:33

NOTE: Green indicators will change color when the status of the beam is altered.

- 60590_screenshot_2

3.3. To configure the sensor input so that SAP can detect perch landings and initiate song playbacks, click the Operant Devices tab in the SAP Recorder control window. Check the box Enable Operant Training (NI Card Installed). 00:0 -00:13

3.4. Select the appropriate Device ID (the Ports slider can be set to 3). For each detector, indicate the appropriate port and line from the information collected in step 2.6. 00:14 – 00:28

NOTE: When the port and line are set correctly for each detector, the light will change from yellow to red.

3.5. In the Main window for the SAP recorder, hit Train to activate the sensors for the appropriate channels. 00:29 – 00:39

NOTE: A yellow button should appear.

3.6. Record the name of the bird by selecting the channel number of interest (Press channel 1 or 2 tab on the left side of the screen), and in the Identification and Mode window, type in the bird’s identification in the Name field. 00:43 – 00:59

3.7. To ensure that sound is played to the speakers, manipulate the settings in the Output Selection tab to select the appropriate device connected to the computer (i.e., speakers) and the channel connected to the speaker. 01:00 – 01:23

- 60590_screenshot_3

5.1.2. In Sound Analysis Pro, select the ‘Playbacks’ tab and then the ‘Sounds’ button in the main window. 00:00 – 00:07

5.1.3. Select the audio .wav file to play ‘Silence’ from chamber 1 and chamber 2. 00:08 – 00:17

5.1.4. Go back to ‘Main’ and hit ‘Reset’ on the top of the boxes on the right. 00:18 – 00:23

5.1.5. After the acclimation period, press ‘Start’. 00:26

5.1.6. At the end of the session, press ‘Stop’. Write down the number of triggers at each perch, which is displayed in the blue boxes.  00:27 - 00:44

5.2 Determine song preference.

5.2.1. Repeat the steps in section 5.1, but choose the partner’s song to play from the side chamber with the fewest perch triggers (the non-preferred side), and an unfamiliar song from the side chamber with the most perch triggers (the preferred side).  00:50 – 01:39

5.2.2. Press ‘reset’ prior to starting the 1 h with song playback. 01:20 – 01:22