- 60771_screenshot_1

- 2.1. Launch the software by clicking the Launch icon on the toolbar which opens loggers’ setup window. Enter a name for the launch [1].

  - 2.1.1. SCREEN: Talent launches the software and enters a name. 00:05 – 00:20

- 2.2. Select the Light sensor. Set the measurement to log State from the drop-down list, and choose the state description off and on from the drop-down list [1]. Select the Occupancy sensor. Set the measurement to log State from the drop-down list, and choose the state description unoccupied/occupied from the drop-down list [2].

  - 2.2.1. SCREEN: Talent selects the light sensor, and adjusts settings. 00:21-00:27

  - 2.2.2. SCREEN: Talent selects the occupancy sensor, and adjusts settings. 00:27-00:33

- 2.3. Click the Filters button to enable automatic calculation of additional values. Select the sensor type of choice. Select the type of filter and the interval to use. Edit the Name and click Create New Series. Click Done [1].

  - 2.3.1. SCREEN: Talent adjusts settings. 00:34-01:22

- 2.4. Click the Advanced button to access the sensor properties. Select the Light sensor. Select Set to maximum sensitivity for calibration and click the Save button [1].

  - 2.4.1. SCREEN: Talent adjusts sensor properties. 01:23-01:33

- 2.5. Select the Occupancy sensor. Select a preset timeout value or select Custom and enter a value in minutes and seconds if needed. Click the Save button [1].

  - 2.5.1. SCREEN: Talent selects a present timeout value, and then enters a value. And saves. 01:34-01:41

- 2.6. Select when to launch the logger, depending on the experimental plan: immediately; at intervals which is available when logging runtime; on a specified date and time [1]; or, by manually using the start button [2]. 01:44-01:54

- 2.6.1. SCREEN: Talent points to the available 3 choices. 01:44-01:54

- 2.6.2. Talent points to the start button on the logger. 01:44-01:54

- 2.7. Select when the logger should stop logging: when memory fills; stop at a specified date and time [1]; stop manually [2]; or, never stop which results in the newest data overwriting the oldest [3]. 01:55-2:09

  - 2.7.1. SCREEN: Talent points to the available 3 choices. Important Step 01:55-2:09

  - 2.7.2. Talent points to the stop button on the logger. 01:55-2:09

  - 2.7.3. SCREEN: Talent points to the never stop choice. 01:55-2:09

- 2.8. Click the Start button upon finishing the configuration [1]. Unplug the logger from the computer [2]. 02:10-02:17

  - 2.8.1. SCREEN: Talent clicks on start button. 02:10-02:17

- 60771_screenshot_2

  - 2.1.1. SCREEN: Talent launches the software and unloads the data. 00:03-00:20

  - 2.2. Choose a location and a filename to save the data. Click Save and select the sensors and events to display in a graph and click Plot [1]. Select the series to view on the table data and plot. Click the All or None button to select or deselect all series, or click the checkboxes to select or deselect individual series [2].

  - 2.2.1. SCREEN: Talent saves, and plots a graph. 00:20-00:50

  - 2.2.2. SCREEN: Talent selects the series, and shows the all or none button, and the checkboxes. 00:20-00:50

  - 2.3. Select Export table data from the control panel. Choose destination folder for the export [1]. 00:50-01:12

  - 2.3.1SCREEN: Talent exports the data. 00:50-01:12