60851_Screenshot_1

6.8	Inject sodium formate (0.5 mM in 90% IPA) at the beginning of the sample sequence (described below) to check the instrument calibration 0:00:00 – 0:00:49

6.9 	Set up the instrument sequence table so that solvent and preparative (extraction) blanks are analyzed first; followed by pooled QC samples (6-10) for system conditioning; then the randomized sample list with QC samples run at regular intervals, e.g. every fifth injection, as technical replicates 0:00:50 – 0:02:30. Run two QC samples at the end of the sequence

60851_Screenshot_2

7.1	Check the data quality (internal standard mass accuracy 0:00:00 – 0:00:50 and signal reproducibility) while the sequence is running. To check signal reproducibility, visual inspection of overlaid spectra should suffice 0:00:55 – 0:01:14