Arduino Script for laser control

This script turns on and off laser connected to digital pin 13 when receiving input from the TTL box module attached to pin 5:

// constants that will not change and are used here

// set pin numbers:

const int boxPin = 5;     // number of pin where TTL box is connected - "TTL pin"

const int ledPin =  13;      // the number of the pin where you will connect the laser - "laser pin"

// variables will change:

int boxPinState = 0; // variable for reading the TTL box status

void setup() {

// initialize the laser pin as an output:

pinMode(ledPin, OUTPUT);

// initialize the TTL pin as an input:

pinMode(boxPin, INPUT);

}

void loop() {

// read the state of the pushbutton value:

boxPinState = digitalRead(boxPin);

// check if there is signal from boxes.

// if it is, the buttonState is HIGH:

if (boxPinState == HIGH)

{

digitalWrite(13, HIGH);   // turn the Laser on (HIGH is the voltage level)

delay(10);              // wait for 10 milliseconds

digitalWrite(13, LOW);    // turn the laser off by making the voltage LOW

delay(40);              // wait for 40 milliseconds

;

}

else {

// turn LED off:

digitalWrite(ledPin, LOW);

}

}