Polyethylenimine (PEI) Solution Protocol

- Prepare Borax buffer:

  - Dissolve 14.3 g of sodium tetraborate decahydrate in 950 ml of sterile water.

  - Maintain the solution under constant agitation for 30 min.

  - Adjust pH to 8.3.

  - Add 50 ml of sterile water to the solution for a final solution of 1l.

  - Filter-sterilize the solution using a 0.22 μm filter.

- Prepare PEI solution (20X):

  - Dissolve 2g of PEI in 200 ml of borax buffer.

  - Maintain the solution under constant agitation for 30 min.

  - Add 800 ml of borax buffer to the solution for a final solution of 1l at 2g/l.

  - Filter-sterilize the solution using a 0.22 μm filter.

  - Store at 4°C.

NOTE: Dilute 20X stock PEI solution in sterile water to obtain 1X working PEI solution. Coating can be performed for 30 min at 37°C or overnight at 4°C. Due to PEI toxicity, rinse coating three times with sterile water before cell plating.