# Submission ID #:  60983

# Scriptwriter Name: Anastasia Gomez

# Project Page Link: https://www.jove.com/account/file-uploader?src=18599863

# Title:   Assessing the Age-Specific Phagocytic Ability of Adult Drosophila melanogaster Hemocytes Using an In vivo Phagocytosis Assay

# Authors and Affiliations:

Shonda M. Campbell, Michelle Starz-Gaiano, Jeff Leips

Department of Biology, University of Maryland Baltimore County, Baltimore, MD

# Corresponding Authors:

# Jeff Leips

# Email Addresses for Co-authors:

campb2@umbc.edu

starz@umbc.edu

# leips@umbc.edu

## Author Questionnaire

1. Microscopy: Does your protocol involve video microscopy, such as filming a complex dissection or microinjection technique?  Yes

If Yes, can you record movies/images using your own microscope camera?

No

If No, JoVE will need to record the microscope images using our scope kit (through a camera port or one of the oculars). Please list the make and model of your microscope.

The dissection microscope is a Zeiss SteREO microscope fitted with an Axiocam ERc 5s camera, currently connected to a PC. Alternatively, a different camera could be connected to this port.

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No (this question is just regarding the part of your protocol that will be filmed)

3. Filming location: Will the filming need to take place in multiple locations?   No.

# Introduction

- Introductory Interview Statements

REQUIRED:

  - Shonda Campbell: Our protocol provides an effective and repeatable approach to evaluate and quantify phagocytic events in hemocytes.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Shonda Campbell: The main advantage of this technique is that we can quantify phagocytic events within individual hemocytes, which allows us to detect variations in phagocytic processes among genotypes, in individual flies.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

OPTIONAL:

  - Shonda Campbell: Performing dissections can be challenging. When attempting this technique for the first time, practice dissections as much as possible before carrying out the full procedure.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

# Protocol

- Injection of Flies

  - Begin by preparing glass needles for injection [1]. Fill a 1-millimeter syringe with mineral oil [2] and attach the 30-gauge hypodermic needle provided with the nano-injector [3].

    - WIDE: Establishing shot of talent preparing the needles.

    - Talent filling the syringe with mineral oil. NOTE: Take 1

    - Talent attaching the needle.

  - Insert the 30-gauge needle into the blunt end of a pulled capillary needle and fill it with mineral oil [1]. Slowly remove the 30-gauge needle, making sure that no air bubbles are formed [2].

    - Talent inserting the 30 G needle into the pulled glass needle and filling it with oil.

    - Talent slowly removing the 30 G needle.

  - Remove the collet and place the sealing O-ring with the indentation facing up [1], place the larger O-ring onto the metal plunger [2], then reattach the collet without tightening [3].

    - Talent removing the collet and placing the sealing O-ring.

    - Talent placing the larger O-ring.

    - Talent reattaching the collet.

  - Insert the metal plunger into [added shot] the blunt end of the oil-filled glass needle [1] and gently push it down, inserting it into the larger O-ring [2], then tighten the collect until secure [3].

Added Shot (maybe 2.4.0): Talent breaks off the tip of the needle to create an opening to allow ejections of solution.

    - Talent inserting the metal plunger into the blunt end of the glass needle.

    - Talent pushing the needle into the O-ring.

    - Talent tightening the collet.

  - After filling the needle with fluoro- or pH-sensitive particles, inject the flies in the sternopleural plate of the thorax [1]. The injection is successful if the green dye is going into the fly [2]. Videographer: This step is important!

    - SCOPE: Fly being injected.

    - SCOPE: Green dye going into the fly.

  - Place the injected flies into a new food vial [1], noting the time that the first and last fly was injected [2]. Lay the vial on its side until all flies have recovered to prevent them from getting stuck in the food [3].

    - Talent putting flies into a new vial. NOTE: Take 2

    - Talent writing down the injection times.

    - Talent laying the vial on its side.

- Dorsal Vessel Dissection

  - Orient the fly ventral side up under a dissecting stereo microscope [1]. Then, insert an insect pin into the thorax and another through the most posterior end of the abdomen, near the genitalia [2-TXT].

    - SCOPE: Talent orienting the fly. NOTE: All scope shots inverted!

    - SCOPE: Talent inserting the two pins into the fly. TEXT: Pin up to 10 flies per plate

  - Once all flies have been pinned, use a transfer pipet to add enough dissection media to cover the flies [1]. Remove the head with cuticle scissors and make a horizontal incision directly above the posterior pin in the abdomen, then another at the most anterior end of the abdomen [2].

    - Talent adding dissection media to cover the flies.

    - SCOPE: Talent removing the head and making the two incisions.

  - Make a vertical incision connecting the two horizontal incisions, which will open up the abdominal cavity [1]. Use forceps to remove the internal organs and tissue, making sure to avoid the dorsal vessel [2]. If necessary, use additional pins to secure the cut ends of the cuticle [3], then remove the thorax with the cuticle scissors [4]. Videographer: This step is difficult and important!

    - SCOPE: Talent making the vertical incision and opening the abdominal cavity.

    - SCOPE: Talent removing internal organs, with the dorsal vessel visible (if possible).

    - SCOPE: Talent pinning the cuticle ends.

    - SCOPE: Talent removing the thorax.

- Fixation, Mounting, and Processing

  - Discard the dissection media [1] and replace it with 1 milliliter of 4% PFA, protecting the dissections from light as much as possible [2]. Fix the dissections for 15 minutes at room temperature while rocking at 20 rpm [3], then remove the fixative and add 1 milliliter of 1 X PBST. If desired, stain the dissections with antibodies [4].

    - Talent discarding the dissection media.

    - Talent adding PFA to the plate.

    - Plate on a rocker.

    - Talent removing fixative and adding PBST. NOTE: Take 2

  - After mounting the cuticles on a microscope slide [1], use forceps to orient them 13ventral side up and make sure that the dorsal vessel is visible [2].

    - Talent at the dissection microscope.

    - SCOPE: Talent orienting the cuticle on the slide, with the dorsal vessel visible.

# Results

- Results: Imaging and Quantification of Phagocytic Events

  - Young and aged flies were assessed for age-specific ability to carry out phagocytosis [1] by fluorescent imaging of hemocytes along the dorsal vessel [2]. An antibody against Pericardin was used to ensure that only cells along the dorsal vessel were counted [3].

    - LAB MEDIA: Figure 3 A.

    - LAB MEDIA: Figure 3 A and B. Video Editor: In B, emphasize the hemocytes that the blue arrow is pointing to.

    - LAB MEDIA: Figure 3 A and C.

  - Since fluorescently labeled E. coli particles are 1 micrometer in length and hemocytes are 10 micrometers in diameter [1], only fluorescent events within a 10-micrometer diameter centered on a DAPI-positive nucleus were counted [2]. Image J software was then used to quantify the events [3].

    - LAB MEDIA: Figure 4 A, C, E, and G.

    - LAB MEDIA: Figure 4 B, D, F, and H. Video Editor: These are magnifications of the insets in A, C, E, and G. If possible, transition to these images in a way that makes it obvious that you’re zooming in on the white boxes.

    - LAB MEDIA: Figure 5.

# Conclusion

- Conclusion Interview Statements

  - Shonda Campbell: When attempting this procedure, it is important to visualize that the fly was actually injected and to remember that timing is critical. Completing injections and dissections in a timely manner will help minimize experimental error and possible variations in phagocytic rate.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.5.2 and 3.3.2.

  - Shonda Campbell: This technique will allow researchers to explore a variety of research topics, including questions regarding cellular immunity, different populations of blood cells, or distinguishing phagocytosis from AMP production during a successful immune response.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.