A Sectioning, Coring, and Image Processing Guide for High-Throughput Cortical Bone Sample Procurement and Analysis for Synchrotron Micro-CT

Supplementary Materials

CTAnalyser Tasklist

The following is a CTAnalyser task list created in the Custom Processing menu with the settings used to create the images seen in Figure 9 and the porosity data shown in Table 1.

To collect data on vascular canals:

- Reload the dataset

- Filter the dataset with a Gaussian blur in 3D space using a Round kernel with a radius of 2.

- Threshold: Apply a Global threshold from 0-120.

NOTE: This is a guideline only. Threshold values should be adjusted for each experimental set-up.

- Despeckle: Remove white speckles in 3D space within the range 13 - 2743 voxels. Apply this step to the image.

  - This removes all osteocyte lacunae from the image. These values are specific to a 0.9 µm voxel image.

- Despeckle: Remove black speckles in 2D space less than 15,000 pixels. Apply this step to the image.

- Morphological Operations: Dilate in 3D space with a radius of 10. Apply this step to the image.

- Repeat step 5 but replace 15,000 pixels with 100,000 pixels.

- Morphological Operations: Erode in 3D space with a radius of 10. Apply this step to the image (This is the inverse of Step 6).

- Perform a 3D analysis to gather the Basic values as well as trabecular separation, trabecular thickness, and number of objects.

- Use save bitmaps to save the processed images in a custom subfolder.

- Perform Individual object analysis to gather data specific to each canal.

- Bitwise operations: Clipboard = Region of Interest SUB Image

  - This reads as “Clipboard equals the region of interest without the currently seen image” for each reconstructed slice of the dataset.

  - This will allow for easy removal of the vascular canals in the following steps.

To collect data on osteocyte lacunae:

- Repeat steps 1-3.

- Despeckle: Remove white speckles in 3D space less than 13 voxels. Apply this step to the image.

  - This removes noise from the scan that may be mistakenly counted as osteocyte lacunae.

- Despeckle: Remove white speckles in 3D space greater than 2743 voxels. Apply this step to the image.

- Bitwise operations: Image = Clipboard AND Image

  - This step removes any leftover traces of the vascular canals that weren’t removed in the previous steps, leaving only the osteocyte lacunae.

- Perform 3D analysis on the osteocyte lacunae using the same settings as Step 9.

- Use save bitmaps to save the processed images in a custom subfolder.

- Perform Individual object analysis to gather data specific to each lacuna.

Experiment Settings Employed to Achieve Representative Results

| Table S1. February 2015 Imaging Parameters (Figs. 7A, C, E and 8A-C) |  |
| --- | --- |
| Canadian Light Source beamline utilized | BMIT-ID |
| Beam energy | 31 keV |
| Pre-scan flat-field projections acquired | 10 images |
| Pre-scan dark-field projections acquired | 10 images |
| Frame averaging | 4 frames |
| Rotation step | 0.25° |
| Binning | 2x2 |
| Number of projections per scan | 720 images |
| Total scan rotation | 180° |
| Post-alignment scan rotation step | 15° |
| Post-scan flat field projections acquired | 10 images at 18° |
| Scan time | 75 minutes |
| Voxel size | 1.8 µm |

| Table S2. June 2018 Imaging Parameters (Figs. 7B, D, F and 8D-F) |  |
| --- | --- |
| Canadian Light Source beamline utilized | BMIT-ID |
| Beam energy | 30 keV |
| Pre-scan flat-field projections acquired | 10 images |
| Pre-scan dark-field projections acquired | 10 images |
| Frame averaging | 5 frames |
| Rotation step | 0.20° |
| Binning | N/A |
| Number of projections per scan | 900 images |
| Total scan rotation | 180° |
| Post-alignment scan rotation step | N/A |
| Post-scan flat field projections acquired | N/A |
| Scan time | 85 minutes |
| Voxel size | 0.9 µm |