# Submission ID #:  61086

# Scriptwriter Name: Anastasia Gomez

# Project Page Link: https://www.jove.com/account/file-uploader?src=18631278

# Title:   Preparation of Small RNA Libraries for Sequencing from Early Mouse Embryos

# Authors and Affiliations:

Rachel A. Keuls1, Ronald Parchem2

1Program in Development Disease Models and Therapeutics, Baylor College of Medicine, Houston, TX, USA

2Center for Cell and Gene Therapy, Stem Cells and Regenerative Medicine Center, Department of Neuroscience, Baylor College of Medicine, Houston TX, USA

# Corresponding Authors:

Ronald Parchem

# Ronald.Parchem@bcm.edu

# Email Addresses for Co-authors:

Rachel.Keuls@bcm.edu

## Author Questionnaire

1. Microscopy: Does your protocol involve video microscopy, such as filming a complex dissection or microinjection technique?  Yes

If Yes, can you record movies/images using your own microscope camera?

No

If No, JoVE will need to record the microscope images using our scope kit (through a camera port or one of the oculars). Please list the make and model of your microscope.

Leica stereo M165FC

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

If Yes, we will need you to record using screen recording software to capture the steps. If you use a Mac, QuickTime X also has the ability to record the steps.

3. Filming location: Will the filming need to take place in multiple locations?   Yes

If Yes, how far apart are the locations? Walking distance to flow cytometer, down one floor and down the hallway. (<0.1 mi)

# Introduction

- Introductory Interview Statements

  - Rachel A. Keuls: This protocol aims to simplify the preparation of low input, small-RNA libraries from early embryos to accurately profile the miRNAs that regulate embryonic development.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Rachel A. Keuls: The advantage of this technique is that it avoids the pooling of low input samples and can easily be applied to both non-sorted and sorted cells. We also demonstrate careful embryo staging to avoid variance between replicates.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

OPTIONAL:

  - Rachel A. Keuls: This method can be applied to temporal, genetic and developmental studies, or other applications where input RNA concentrations are low. In the future, it could potentially be used to diagnose developmental disorders in which miRNAs are dysregulated.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Rachel A. Keuls: The most technically challenging part of this protocol is the embryo dissection because each embryo must be dissected, staged, imaged and then dissociated quickly to ensure cell viability.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Ethics Title Card

  - All research and animal care procedures were approved by the Baylor College of Medicine Institutional Animal Care and Use Committee.

# Protocol

- Embryo Dissection

  - To dissect the embryos, rinse the uterus of a pregnant female mouse with PBS [1] and place it in a sterile plastic 10-centimeter dish [2]. Use microdissection scissors to separate each decidua-containing region [3] and peel off the uterine muscle to expose all decidua [4].

    - Talent rinsing the uterus with PBS.

    - Talent placing the uterus in a plastic dish.

    - SCOPE: Talent separating the decidua-containing regions.

    - SCOPE: Talent peeling off the uterine muscle.

  - One by one, remove the decidua from each embryo [1], then remove the yolk sac and save it for genotyping [2]. Move all embryos into a new dish with PBS [3] and take an image of the entire litter [4]. Videographer: This step is difficult and important!

    - SCOPE: Talent removing the decidua from an embryo.

    - SCOPE: Talent removing the yolk sac.

    - Talent transferring embryos to a new dish.

    - SCOPE: Litter being imaged.

  - Then, image each embryo individually and count the number of somites, keeping the magnification and exposure constant between experiments [1]. Videographer: This step is difficult and important!

    - SCREEN: Talent imaging the embryos. Videographer: Film the screen over talent’s shoulder.  NOTE: No screen cap uploaded for this, use the vid footage.

- Embryo Dissociation and Cell Sorting

  - For embryos older than E8.0, decapitate just above the optic placode [1] and transfer the head to a clean well of a 48-well plate [2]. Add 250 microliters of papain to the well and pipette up and down gently [3]. Videographer: This step is important!

    - Talent decapitating the embryo.

    - Talent putting the head in the well of a plate.

    - Talent adding papain to the well and pipetting up and down.

  - Use the microscope to check for clumps [1] and continue pipetting up and down until a single cell suspension is achieved [2]. Then, add 250 microliters of FBS to the cells and filter 500 microliters of the suspension through a 35-micrometer nylon mesh filter [3].

    - Talent at the microscope.

    - SCOPE: Single cell suspension.

    - Talent adding FBS to a well.

    - Talent filtering the cell suspension.

  - Move the filtrate to a new 1.5-milliliter tube [1] and spin it down at 200 x g for 5 minutes [2]. Transfer the supernatant to a new tube [3] and resuspend the cell pellet in 300 microliters of PBS with BSA [4-TXT].

    - Talent transferring the filtrate to a new tube.

    - Talent putting the tube in the centrifuge and closing the lid.

    - Talent removing the supernatant from the tube.

    - Talent resuspending the cell pellet. TEXT: Keep on ice until sorting

  - When ready to sort, filter the cells again through a filter cap tube [1] and add DAPI (pronounced ‘dappee’, like happy) to stain live cells [2]. Use a cell sorter with a 70-micrometer nozzle to sort each sample into 500 microliters of RNA extraction lysis solution [3], then mix and store the sample at -80 degrees Celsius [4].

    - Talent filtering the cells.

    - Talent adding DAPI to the cells.

    - Talent using the cell sorter.

    - Talent putting the sample into the freezer.

- Size Selection

  - Add 5 microliters of 6 X loading dye to each sample and mix by pipetting [1]. Load the samples onto a 6% TBE-polyacrylamide gel, starting with the ladder in the first well and leaving 1 lane between each sample [2]. Run the gel at 150 volts for 30 to 40 minutes in 0.5 X TBE running buffer [1]

    - Talent adding loading dye to a sample and pipetting it up and down.

    - Talent loading the ladder and then a sample into the gel.

  - When the gel has finished running, carefully remove it from the glass plates [1] and place it in the tray from its original packaging, noting the orientation [2]. Remove the running buffer and stain the gel for 15 minutes [3], then image it on a UV transilluminator [4].

    - Talent removing the gel from the glass plates.

    - Talent placing the gel in its original packaging.

    - Talent adding staining solution to the gel.

    - Talent imaging the gel.

  - Identify the band at 150 base pairs and use a clean razor blade to excise it, making sure to avoid the adapter dimer product at 130 base pairs [1-TXT]. Put the gel slices into microcentrifuge tubes and spin them down at the maximum speed for 30 seconds [2]. Videographer: This step is difficult and important!

    - Talent cutting the gel. TEXT: Image gel after cutting

    - Talent putting microcentrifuge tubes in the centrifuge and closing the lid. Videographer: Obtain multiple reusable takes of this shot because it will be reused in 4.5.2.

  - Next, use a p200 tip to crush the gel slice into the smallest bits possible and eject the tip into the tube [1]. Add 300 microliters of elution buffer to each tube, washing the gel bits from the side [2]. Reattach the pipette tip and wash it off before removing it from the tube [3]. Videographer: This step is important!

    - Talent crushing the gel slice with the pipette tip, then ejecting the tip into the tube.

    - Talent adding elution buffer to a tube.

    - Talent reattaching the pipette tip and removing it from the tube.

  - Incubate the samples overnight at 25 degrees Celsius with 1000 rpm shaking [1]. On the next day, spin them down at maximum speed for 10 minutes [2] and transfer the supernatant to an RNase-free 96-well plate [3].

    - Talent putting the samples in the incubator and starting the shaker.

    - Use 4.3.2.

    - Talent transferring supernatant to the plate.

  - Add 50 microliters of cleanup beads to each sample and pipette to mix [1], then add 350 microliters of isopropanol, mix the sample [2], and incubate the plate at room temperature for 10 minutes while rocking [3].

    - Talent adding cleanup beads to a well and mixing it.

    - Talent adding isopropanol to the well and mixing it.

    - Plate on the rocker.

  - After the incubation, pulse spin the plate [1] and magnetize it for 2 minutes [2]. Discard the supernatant and wash the beads with 950 microliters of 80% ethanol for 30 seconds [3-TXT].

    - Talent pulse spinning the plate.

    - Talent putting the plate on a magnetic stand.

    - Talent adding ethanol to the plate. TEXT: Repeat wash 2 X

  - Dry the sample for 3 minutes, then remove all residual liquid at the bottom of the well [1]. Remove the plate from the magnetic stand [2] and resuspend the beads in 13 microliters of the resuspension buffer [3]. Incubate the plate for 2 minutes, then transfer it back to the magnetic stand for 3 minutes [4].

    - ECU: Talent removing liquid from the bottom of the well.

    - Talent removing the plate from the magnetic stand.

    - Talent resuspending the beads.

    - Talent picking the plate up from the lab bench and putting it on the magnetic stand.

  - Transfer 12 microliters of the supernatant to a clean tube [1] and store it overnight at 4 degrees Celsius or long term at -20 degrees Celsius [2].

    - Talent transferring supernatant to a new tube.

    - Talent putting the tube in the fridge or freezer.

# Results

- Results: Small RNA Libraries

  - Embryos at E7.5, E8.5, and E9.5 were harvested [1] and somite staged to resolve the 6-hour time intervals [2]. When principal component analysis was used to group samples based on similarity, it was found that samples cluster by age [3].

    - LAB MEDIA: Figure 1 A.

    - LAB MEDIA: Figure 1 A and B.

    - LAB MEDIA: Figure 1 C. Video Editor: Emphasize that the red, green, and blue dots form their own cluster. Maybe draw three ovals, each enclosing all dots of the same color.

  - Premigratory and migratory neural crest cells were labeled at E8.5 with Wnt1-Cre (pronounce ‘Wint-one-cree’) [1] and at E9.5 with Sox10-Cre (pronounce ‘socks-ten-cree’) [2]. A comparison of the two Cre-drivers confirms that they mark different populations in early mouse embryos [3].

    - LAB MEDIA: Figure 1 D. Video Editor: Emphasize the first image (Wnt1-Cre).

    - LAB MEDIA: Figure 1 D. Video Editor: Emphasize the second image (Sox10-Cre).

    - LAB MEDIA: Figure 1 E. Video Editor: Emphasize that the circle and triangle clusters on the plot, similar to 5.1.3.

  - RNA was quantified with a spectrophotometer and capillary electrophoresis [1]. While the spectrophotometer trace revealed that the RNA contained no contaminating proteins and a high salt concentration [2], it was not sensitive enough to detect changes in RNA concentration with age [3].

    - LAB MEDIA: Figure 2 A.

    - LAB MEDIA: Figure 2 A and B. Video Editor: Emphasize the 260/280 and 260/230 rows in B.

    - LAB MEDIA: Figure 2 C.

  - The capillary electrophoresis trace can be used to estimate concentration and size of the RNA fragments [1]. Peaks at 2000 and 5100 nucleotides are 18s and 28s ribosomal RNA, respectively [2], while the small RNA region is located at about 150 nucleotides [3].

    - LAB MEDIA: Figure 2 D.

    - LAB MEDIA: Figure 2 D. Video Editor: Emphasize the large peaks at ~2000 and ~5100.

    - LAB MEDIA: Figure 2 D. Video Editor: Emphasize the small peak at 156.

  - Gel extraction can be used to isolate the small RNA sequencing libraries away from the adapter dimers [1]. Before excision, a multitude of product sizes are present in each sample [2].

    - LAB MEDIA: Figure 3 A.

    - LAB MEDIA: Figure 3 A. Video Editor: Emphasize the gel lanes numbered 1 – 4.

  - Capillary electrophoresis traces before and after size selection [1] show the improvement in the purity of the 150-base pair library product after gel purification [2].

    - LAB MEDIA: Figure 3 C and D. Video Editor: Label D “After Size Selection”.

    - LAB MEDIA: Figure 3 C and D. Video Editor: Emphasize the peaks at ~145 on both plots.

# Conclusion

- Conclusion Interview Statements

  - Rachel A. Keuls: We have designed this protocol to split an RNA prep from a single sample into both small-RNA sequencing libraries and bulk mRNA sequencing libraries. This will address questions surrounding not only miRNA expression, but also which mRNA targets may be regulated by the expressed miRNAs.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Rachel A. Keuls: This technique enabled us to obtain a comprehensive view of the most highly expressed miRNAs and the regulation of their targets in cranial neural crest cells between gastrulation and neural tube closure.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.