- 61159_screenshot_5

  - 5.1.1 (Open Aivia and open image sequence.) 00:00-00:05

  - 5.1.2 (Select the Neuron Particle Tracking from the Recipes dropdown menu in the Recipe Console.) 00:06-00:10

  - 5.2.1 (Click on the Magnifying Glass icon to turn on preview for the Detection Parameters set. Toggle the display of the Enhanced – Preview channel and use the channel to guide parameter adjustments. Zoom in the image to view the enhanced puncta.) 00:11-00:25

  - 5.2.2 (Click on the right caret ‘>’ next to Detection Parameters to expand the parameter group. Mouse over the bright spots on the image and note the pixel intensity value shown at the bottom of the screen. Set the Min Particle Intensity value based on the observed puncta intensity on the Enhanced – Preview channel. The preview automatically updates and the detected regions are highlighted in blue.) 00:27-00:45

  - 5.2.3 (Both the Detection Parameters and Tracking Parameters can be tuned by adjusting the parameter values individually or select one of the preset buttons.) 00:48-00:50

  - 5.3.1 (Click the Start button to apply the analysis recipe to the entire image.) 00:55-01:45

  - 5.3.2 (Adjust the Object Set Settings to toggle display of individual track elements such as “Objects”, “Tracks”, and “Anchors”. You can also adjust the size of the anchors to facilitate visualization of the puncta.) 01:47-01:58

  - 5.3.3 (Click on the Play button in the time bar to play through the image sequence.) 01:59-02:10

  - 5.3.4 (Click on the Spreadsheet button below the time bar to show the output results for the tracking parameters. The results can be exported by clicking on the Export to Excel button in the Spreadsheet section.) 02:16-02:33

  - 5.4 (Select tracks by rubberband selection, or left-click and select. Select multiple tracks by holding down the Ctrl key on your keyboard while left-clicking the puncta.) 02:37-02:45