Supplemental file

Picture explaining how the different coating have been disposed in the 96-well plate.

Experimental parameters:

To Acquire the images:

- Application: Confluence

- Image Acquisition setting: Auto Exposure

- Configuration: Brightfield

- Acquisition resolution: 1um/pixel

- For the Contrast: auto contrast

- For the Focus set up:

- “hardware autofocus – register auto mode”

- Target focal plane bright focus.

For the Analysis:

- Application: Confluence

- Analysis resolution: 2um/pixel

- Algorithm: Texture

- Intensity threshold: 15

- Precision: High

- Min cluster size: 50 µm^3

Imaging parameters used

| Channel Name | Name | Cube | Contrast Method | Emission Wavelength | Peak Emission | Peak Excitation |
| --- | --- | --- | --- | --- | --- | --- |
| Channel 1 | DAPI | DAPI | FLUO | nm | 461 | 0 nm |
| Channel 2 | RHOAD | RHOAD | FLUO | nm | 578 | 0 nm |

| Channel Name | Lookup table name | Exposure time | Gain | Differential interference contrast | Condenser | Resolution XY | Resolution Z |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel 1 | Blue | 220.02 ms | 0 | - |  | 0.469 µm | 1.281 µm |
| Channel 2 | Red | 795 ms | 0 | - |  | 0.469 µm | 1.281 µm |

| Camera settings |  |
| --- | --- |
| Camera | Hamamatsu-C11440-42U-USB-001428 |
| Format | -- |
| Digitization | 16 bits |
| Gamma | 0.9807789 |
| Quality Mode | 1 MHz |
| Image Flip X Axis | No |
| Image Flip Y Axis | No |
| Black-Value | 0 |
| White-Value | 65535 |
| Shading Correction | OFF |

| Microscope Settings |  |
| --- | --- |
| System Name | LAS X |
| Microscope Model | DMI8 |
| OBjective | HC PL FLUOTAR L 40x/0.60 DRY |
| Order number (Obj.) | 11506201 |
| Numerical aperture | 0.6 |
| Refractive index | 1 |
| Mounting Medium Refraction index |  |
| Z Movement | Z then Lambda |
| Camera Light | 100% |