# Screenshot Summary for JoVE manuscript #61235

- 61235_screenshot_2.1.2

  - Open freeware DICOM viewer and import MR images. 0:00 – 0:16

  - Load the DCE sequence in 4D mode. 0:16–0:21

- 61235_screenshot_2.2-.2.3

  - 2.2

    - Identify proximal right tibia on slice number 4. 0:00–0:05

    - Place a ROI within the bone marrow. 0:05–0:20

    - Open DCE Tool and select “Relative Enhancement”. 0:20–0:29

  - 2.3

    - Enter the baseline range from time points 1 to 5. 0:29–0:32

    - Export the measurements to “DCEraw.txt”. 0:33–0:43

- 61235_screenshot_2.4-2.5

  - 2.4.1

    - Open RStudio and load the provided “DCE-Script.R” file. 0:00–0:13

  - 2.5.1

    - Run the Code. 0:13–0:21

    - Select and copy the output. 0:21–0:28

    - Open the “ImagingFeatures.xlsx” file and paste the output into the respective columns. 0:28–0:43

- 61235_screenshot_2.6

  - 2.6.1

    - Place a second ROI within the back muscles. 0:00–0:06

  - 2.6.2

    - Open DCE Tool and select “Relative Enhancement”. 0:06–0:12

    - Enter the baseline range from time points 1 to 5. 0:12–0:15

    - Export the measurements to “DCEraw.txt”. 0:15–0:27

- 61235_screenshot_2.7

  - 2.7.1

    - Use the “DCE-Script.R” file within RStudio and run the code. 0:00–0:09

    - Select and copy the output. 0:09–0:16

    - Open the “ImagingFeatures.xlsx” file and paste the output into the respective columns. The normalization is performed automatically. 0:16–0:28

- 61235_screenshot_3.1.2-3.4.1

  - 3.1.2

    - Image(s) being imported TEXT: See text for PET/CT imaging details. 0:00–0:59 - this can be fast-forwarded as a lot of time passes during image loading

  - 3.2.1

    - General analysis being clicked, then ROI quantification, Create, and Create ROI from template being clicked. 2:10–0:14

  - 3.3.1

    - ROI being placed, then ROI Target 1 overlay being selected. 2:14–2:40

  - 3.4.1

    - Shot of mean, minimum, and maximum values. 2:40

- 61235_screenshot_3.5.1

  - 3.5.1

    - Max value being divided by injected activity and result being multiplied by animal weight. 0:00–0:28

- 61235_screenshot_4

  - 4.1.

    - Import MR images from day 30 post injection into the freeware DICOM viewer. 0:00–0:14

    - Check whether metastases have grown. Here, we see large metastases within the right hind leg. 0:14–0:30

  - 4.2.

    - Within the “ImagingFeaturex.xlsx” spreadsheet, enter “1” if metastases have developed (which is the case for animal #1), or enter “0” if no metastases are visible. 0:30–0:41

    - NOTE: we did not screenshot the procedure of entering “0”. We would kindly ask the video production team to successively unveil the entire spreadsheet lines for the animals 2-28, e.g., by successively fading these lines in (we provided a complete “ImagingFeatures.xlsx” file that offers data for all animals). This will perfectly show that there are animals with and without metastases. The procedure is always the same.

- 61235_screenshot_5

  - 5.2.1

    - Open the open-source data visualization, machine learning, and data mining toolkit. 0:00–0:03

    - Draw the File subroutine into the workspace and double-click it. 0:03–0:09

    - Select the spreadsheet. 0:09–0:19

  - 5.3.1

    - Click the folder icon and select the spreadsheet. 0:09–0:19

  - 5.4.1

    - Select the “Export” worksheet. 0:19–0:23

    - Assign the “target” attribute to the “Tumor” variable, and the “skip” attribute to the “Animal” variable. 0:23–0:34

  - 5.5.1

    - Draw the Rank subroutine into the workspace. 0:34–0:38

    - Connect the “File” and the “Rank” subroutine. 0:38–0:41

  - 5.6.1

    - Double-click the “Rank” subroutine and select the “Information Gain” algorithm. 0:41–0:50

- 61235_screenshot_6.1-6.3

  - 6.1.2

    - Within RStudio, open the “TrainModel.R” file. 0:00–0:09

  - 6.2.1

    - Select the lines 3-6 within the script. 0:09–0:14

  - 6.3.1

    - Run the selected lines of code. 0:14–0:22

- 61235_screenshot_6.4

  - 6.4.1

    - Select the lines 8–39. 0:00–0:05

    - Run the selected lines. 0:05–0:54

- 61235_screenshot_6.5

  - 6.5.1

    - Select the lines 41–50. 0:00–0:07

    - Run the selected lines. 0:07–0:13

    - Inspect the accuracy measures and ROC curve. 0:13–0:16