Supplementary Protocol

Schematic 1: HAT Assay Reaction

| 10 Minute Pre-incubation (do not add again in complete reaction) | Complete HAT Assay Reaction (includes pre-incubation components) |
| --- | --- |
| 2 L of 5x assay buffer | 2 L of 5x assay buffer |
|  | 3 L of 100 M Acetyl-CoA |
| 1 L of purified p300 (0.19 g/L ) | 1 L of purified p300 (0.19 g/L ) |
|  | 1 L of purified H3.1 (0.2 g/L ) |
| 1 L of anacardic acid (HATi) or DMSO control diluted in 1x assay buffer | 1 L of anacardic acid (HATi) or DMSO control diluted in 1x assay buffer |
| 2 L autoclaved ddH2O | 2 L autoclaved ddH2O |

Schematic 2: ChHAI Assay Experimental Design

| Experimental Groups | Initial Drug Dilutions (Step 2.1.2-2.1.5) | Final Drug Dilutions (Step 2.1.7) |
| --- | --- | --- |
| DMSO | 0.5 L of DMSO to 1 mL of DMEM media | 1.0 L of DMSO to 1 mL of DMEM media |
| A-485 3 M | 0.5 L of DMSO to 1 mL of DMEM media | 0.5 L of DMSO and 0.5 L A-485 (6 mM) to 1 mL of DMEM media |
| A-485 10 M | 0.5 L of DMSO to 1 mL of DMEM media | 0.5 L of DMSO and 0.5 L of A-485 (20 mM) to 1 mL of DMEM media |
| MS-275 3 M | 0.5 L of MS-275 (6 mM) to 1 mL of DMEM media | 0.5 L of DMSO and 0.5 L of MS-275 (6 mM) to 1 mL of DMEM media |
| MS-275 3 M + A-485 3 M | 0.5 L of MS-275 (6 mM) to 1 mL of DMEM media | 0.5 L of A-485 (6 mM) and 0.5 L of MS-275 (6 mM) to 1 mL of DMEM media |
| MS-275 3 M + A-485 10 M | 0.5 L of MS-275 (6 mM) to 1 mL of DMEM media | 0.5 L of A-485 (20 mM) and 0.5 L of MS-275 (6 mM) to 1 mL of DMEM media. |

Schematic 3: ChIP-qPCR Experimental Design

| Experimental Groups | Drug Dilutions (Step 3.2.2-3.2.3) | Immunoprecipitation Groups (Step 3.5.4) |
| --- | --- | --- |
| DMSO | 6 L of DMSO to 12 mL of DMEM media | IP #1: IgG antibody (5-10 µg) |
|  |  | IP #2: H3K27ac antibody (5-10 µg) |
| A-485 3 M | 6 L of A-485 (6 mM) to 12 mL of DMEM media | IP #3: IgG antibody (5-10 µg) |
|  |  | IP #4: H3K27ac antibody (5-10 µg) |