# Submission ID #:  61292

# Scriptwriter Name: Anastasia Gomez

# Project Page Link: https://www.jove.com/account/file-uploader?src=18694108

# Title:   Paper-Based Preconcentration and Isolation of Microvesicles and Exosomes

# Authors and Affiliations:

# Dohwan Lee1,*, Kyung Wook Wee1,*, Hyerin Kim1,*, Sung Il Han1, Seungmin Kwak1,2, Dae Sung Yoon3, Jeong Hoon Lee1

# 1Department of Electrical Engineering, Kwangwoon University, Seoul, Republic of Korea

# 2Korea Institute of Science and Technology (KIST), Seoul, Republic of Korea

# 3School of Biomedical Engineering, Korea University, Seoul, Republic of Korea

# *These authors contributed equally.

# Corresponding Authors:

Dae Sung Yoon	(dsyoon@korea.ac.kr)

# Jeong Hoon Lee	(jhlee@kw.ac.kr)

# Email Addresses for Co-authors:

Dohwan Lee		(ldh8917@gmail.com)

Kyung Wook Wee	(kyungwook.wee@ni.com)

Hyerin Kim		(hr.kim0406@gmail.com)

Sung Il Han		(sihan624@gmail.com)

# Seungmin Kwak	(smkwak17@naver.com)

## Author Questionnaire

1. Microscopy: Does your protocol involve video microscopy, such as filming a complex dissection or microinjection technique?  Yes

If Yes, can you record movies/images using your own microscope camera?

Yes

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   No

# Introduction

- Introductory Interview Statements

REQUIRED:

  - Hyerin Kim: In this protocol, we describe a simple method to fabricate a paper-based device called the Exo-Pad, which is designed for the effective enrichment and isolation of extracellular vesicles.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Hyerin Kim: The Exo-PAD is a simple device that can preconcentrate and isolate extracellular vesicles without the use of any chemicals or bulky machines while showing higher recovery rates than conventional methods.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

# Protocol

- Device Fabrication

  - Begin by defining the region to be printed with printer software [1]. The design should have 12 wax-patterned layers in which the diameters of the circular sample areas gradually narrow from 5 to 2 millimeters [2]. Then, print hydrophobic wax on the designated regions on both sides of the cellulose paper [3].

    - WIDE: Establishing shot of talent at the computer designing the region to be printed

    - LAB MEDIA: Figure 1 A.

    - Talent picking up and looking at the freshly printed design.

  - Place the wax-printed paper in a laboratory oven for 80 seconds at 120 degrees Celsius [1], then cut the wax-printed paper with a cutter to make individual Exo-PAD devices [2]. Drop 5 and 2 microliters of permselective membrane onto the sample areas in the leftmost and rightmost layers, respectively [3]. Videographer: This step is difficult and important!

    - Talent placing the paper in the oven.

    - Talent cutting the paper into individual devices.

    - Talent dropping membrane onto the leftmost and rightmost layers.

  - Place the layers coated with the permselective membrane on a hot plate at 70 degrees Celsius for 30 minutes to evaporate the solvent [1]. Then, seal the outermost surface of the coated layer facing the buffer solution with pressure-sensitive tape, leaving a small hole [2]. Videographer: This step is important!

    - Talent placing the device on a hotplate.

    - Talent sealing the coated layer with tape and leaving a small hole.

  - Fold the Exo-PAD device back and forth along the white lines to convergently connect all sample areas [1].

    - Talent folding the device back and forth.

- Enrichment and Spatial Focusing of Microvesicles and Exosomes

  - Pipette 15 microliters of the microvesicle and exosome sample in the convergent sample areas and wait a few seconds to ensure complete wetting [1-TXT]. Place two acrylic chambers at both ends of the Exo-PAD [2] and clamp it securely with small binder clips to prevent unfolding [3].

    - Talent loading the sample onto the device. TEXT: 3 x 1011 particles/mL in 0.1 X PBS with 0.05% Tween 20

    - Talent placing acrylic chambers on the ends of the Exo-PAD.

    - Talent clamping the Exo-PAD.

  - Fill the chambers with 110 microliters of 0.1 X PBS [1] and insert two silver electrodes [2], then use a current-voltage source measurement system to apply 30 volts to the electrodes for 20 minutes [3]. Videographer: This step is difficult and important!

    - Talent adding PBS to the chambers.

    - Talent inserting the silver electrodes.

    - Talent programming the current-voltage source.

  - To isolate the enriched microvesicles and exosomes, separate the folded Exo-PAD from the acrylic chambers [1] and unfold the device to isolate the enriched particles from the other layers [2]. Punch out the sample areas in layers 8 and 9, where the microvesicles and exosomes are enriched, for downstream analysis [3]. Videographer: This step is important!

    - Talent removing the device from the acrylic chambers.

    - Talent unfolding the device.

    - Talent punching out the appropriate sample areas. Videographer: Obtain multiple usable takes of this shot because it will be reused in 5.1.1.

# Results

- Results: Analysis of Enriched Microvesicles and Exosomes

  - A time-lapse migration assay of fluorescently labeled microvesicles and exosomes was performed in order to optimize the operation time for maximum recovery of the enriched particles [1]. The fluorescence intensities in all sample areas were quantified with ImageJ software [2].

    - LAB MEDIA: Figure 2 A.

    - LAB MEDIA: Figure 2 B.

  - Before the enrichment process, microvesicles and exosomes were dispersed in all sample areas with a gradual decrease in the intensity [1]. After 10 minutes of processing, the particles migrated electrokinetically [2] and an instant preconcentration plug appeared on layer 7 [3].

    - LAB MEDIA: Figure 2 A. Video Editor: Emphasize the 0 min row.

    - LAB MEDIA: Figure 2 A. Video Editor: Emphasize the 10 min row.

    - LAB MEDIA: Figure 2 A. Video Editor: Emphasize the 10 min row and the layer 7 image (7th from left).

  - When the time reached 20 minutes, microvesicles and exosomes [1] were strongly focused on layers 8 and 9 and enriched fivefold [2], based on the fluorescence intensities [3].

    - LAB MEDIA: Figure 2 A. Video Editor: Emphasize the 20 min row.

    - LAB MEDIA: Figure 2 A. Video Editor: Emphasize the 20 min row and the layer 8 and 9 images.

    - LAB MEDIA: Figure 2. Video Editor: Include B here and emphasize the part of the plot with the red background, layers 8 and 9.

  - The integrity of the enriched microvesicles and exosomes in layer 8 was investigated with SEM [1]. Significantly more nondamaged particles were observed after the enrichment process [2]. The size distribution of the enriched microvesicles and exosomes was also analyzed [3].

    - LAB MEDIA: Figure 3 A.

    - LAB MEDIA: Figure 3 A. Video Editor: Emphasize the After image.

    - LAB MEDIA: Figure 3 B.

  - Subsequently, the actual concentration of the enriched microvesicles and exosomes on the sample areas in layers 6, 8, and 10 was analyzed [1]. In layer 8, the microvesicles and exosomes were preconcentrated by a factor of 5.58 [2], but no noticeable preconcentration was observed in the other layers [3].

    - LAB MEDIA: Figure 3 C.

    - LAB MEDIA: Figure 3 C. Video Editor: Emphasize the layer 8 bars.

    - LAB MEDIA: Figure 3 C. Video Editor: Emphasize layer 6 and 10 bars.

# Conclusion

- Conclusion Interview Statements

  - Hyerin Kim: To ensure reproducibility of this procedure, precisely follow the protocol for oven incubation time and temperature during device fabrication and device operation time because those are the optimized conditions.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.2.1 and 3.2.3.

  - Hyerin Kim: The Exo-PAD can also be used to separate and isolate extracellular vesicle from protein-rich samples, such as serum or plasma, by using carbonate buffer.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Hyerin Kim: This protocol can provide an efficient sample preparation tool with preconcentrating and isolating capabilities required for clinical research such as diagnosis, drug delivery, immunotherapy and other therapeutics.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.