# Submission ID #:  61348

# Scriptwriter Name: Anastasia Gomez

# Project Page Link: https://www.jove.com/account/file-uploader?src=18711188

# Title:   Digestion of Whole Mouse Eyes for Multi-Parameter Flow Cytometric Analysis of Mononuclear Phagocytes

# Authors and Affiliations:

# Steven Droho1, Carla M. Cuda2*, Jeremy A. Lavine1*

# 1Department of Ophthalmology, Feinberg School of Medicine, Northwestern University, Chicago, IL, USA

# 2Department of Medicine, Division of Rheumatology, Feinberg School of Medicine, Northwestern University, Chicago, IL, USA

# *These authors contributed equally to the work

# Corresponding Authors:

# Jeremy A Lavine	(jeremy.lavine@northwestern.edu)

# Email Addresses for All Authors:

steven.droho@northwestern.edu

# c-cuda@northwestern.edu

# jeremy.lavine@northwestern.edu

## Author Questionnaire

1. Microscopy: Does your protocol involve video microscopy, such as filming a complex dissection or microinjection technique?  Yes

If Yes, can you record movies/images using your own microscope camera?

No

If your protocol involves microscopy but you are not able to record movies/images with your microscope camera, JoVE will need to use our scope kit (through a camera port or one of the oculars). Please list the make and model of your microscope.

National DC3-420T

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  16

Number of Shots:  43

# Introduction

- Introductory Interview Statements

REQUIRED:

  - Jeremy Lavine: This protocol is designed to digest whole mouse eyes into a single cell suspension for multi-parameter flow cytometry in order to quantitively analyze mononuclear phagocytes, which include monocytes, microglia, macrophages, and dendritic cells.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Jeremy Lavine: The main advantage of this technique is the ability to use quantitative analysis of fluorophore intensity to differentiate closely related cell types that share overlapping cell surface marker expression.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

OPTIONAL:

  - Carla Cuda: Our protocol is optimized for the analysis of macrophage heterogeneity, including both resident and infiltrating populations, in the laser-induced choroidal neovascularization model. However, these concepts and techniques can be equally applied to other models like diabetic retinopathy, experimental autoimmune uveitis, and manifestations of lupus.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Carla Cuda: When attempting this protocol, understanding your cytometer configuration is a critical starting-point, as all cytometers are not configured the exact same way.  Thus, volumes of fluorescently-conjugated antibodies may need to be titrated and fluorophores may need to be adjusted to optimize the protocol based on the cytometer’s configuration.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

# Introduction of Demonstrator on Camera

  - Jeremy A Lavine: Demonstrating the procedure will be Steve Droho, a senior technician from my laboratory.

    - INTERVIEW: Author saying the above.

    - The named demonstrator(s) looks up from workbench or desk or microscope and acknowledges the camera.

Ethics Title Card

  - Procedures involving animal subjects have been approved by the Institutional Animal Care and Use Committee (IACUC) at Northwestern University.

# Protocol

- Digestion of Ocular Tissue

  - Begin by preparing the digestion buffer according to manuscript directions [1]. Using a dissection microscope [2] and 10 X total magnification, remove the remaining conjunctiva and optic nerve from the eyes [3]. Videographer: This step is difficult and important!

    - WIDE: Establishing shot of talent finishing the preparation of dissection buffer.

    - Talent at the microscope.

    - SCOPE: Talent removing conjunctiva and optic nerve from the eye.

  - Move clean eyes to a dry dissecting dish or a small weigh boat [1], then make two perforating wounds with a 90-degree angle between them through the visual axis [2]. Use a syringe with a 30-gauge needle to inject 0.15 to 0.2 milliliters of digestion buffer into the vitreous cavity of each eye [3]. Videographer: This step is important!

    - Talent transferring the eyes to a dissecting dish.

    - Talent making the two stabs through the visual axis.

    - Talent injecting digestion buffer into an eye.

  - Mince the eyes with spring scissors and fine forceps [1-TXT], then dissociate the lens tissue with blunt mechanical disruption to prevent clogging pipette tips in subsequent steps [2]. Rinse the forceps and scissors with 0.75 milliliters of digestion buffer and replace the dish for each sample [3]. Videographer: This step is important!

    - Talent mincing eyes. TEXT: 0.5 mm x 0.5 mm pieces

    - Talent dissociating the lens tissue with forceps.

    - Talent rinsing the forceps and scissors.

  - Use a P1000 pipette to transfer the contents of the dish to a dissociation tube on ice, taking care to not lose any material [1]. Rinse the dish with an additional 1.5 milliliters of digestion buffer [2] and add it to the dissociation tube, bringing the total volume to between 3.25 and 3.5 milliliters [3].

    - Talent transferring the tissue to a dissociation tube.

    - Talent rinsing the dish with digestion buffer.

    - Talent transferring the buffer to the dissociation tube.

  - Dissociate the tissue on an electronic dissociator as described in the text manuscript [1], making sure that all tissue is at the bottom of the inverted dissociation tube and in contact with the digestion buffer [2].

    - Talent using the dissociator. Videographer: Obtain multiple usable takes because this will be reused in 2.6.2.

    - Tube with all tissue in contact with the digestion buffer.

  - Then, incubate the dissociation tube at 37 degrees Celsius and 200 rpm for 30 minutes [1]. Repeat the tissue dissociation two more times, incubating the sample at 37 degrees Celsius and 200 rpm for 30 minutes in between the dissociations [2]. To stop the reaction, add 10 milliliters of cold flow buffer and place the sample tubes on ice [3].

    - Talent putting the tube in the incubator and starting the rotation.

    - Use 2.5.1.

    - Talent adding flow buffer to a tube and placing it on ice.

- Preparation of Cell Suspension

  - To create a single-cell suspension, strain the halted eye digestion reaction through a 40-micrometer filter placed on the top of a 50-milliliter conical centrifuge tube [1]. Use the plunger from a 2.5-milliliter syringe to push any remaining pieces of undigested eye tissue through the filter [2]. Videographer: This step is difficult and important!

    - Talent straining the tissue.

    - Talent pushing remaining tissue through the filter with a plunger.

  - Wash the dissociation tube with 10 milliliters of flow buffer and rinse the filter [1], then use the same plunger to again pass the undigested eye tissue pieces through the filter. Repeat this process with 5 milliliters of flow buffer [2]. Then, wash the dissociation tube and filter with a final 10 milliliters of flow buffer [3]. Videographer: This step is important!

    - Talent washing the dissociation tube and rinsing the filter, with the flow buffer container in the shot.

    - Talent passing the tissue through the filter with a plunger.

    - Talent washing the dissociation tube.

  - Centrifuge the conical tube at 400 x g for 10 minutes at 4 degrees Celsius [1] and decant the supernatant without disturbing the cell pellet [2]. Break up the pellet by flicking the tube against another tube [3], then add 1 milliliter of lysing solution and swirl the tube at room temperature for 30 seconds to lyse the red blood cells [4]. Videographer: This step is important!

    - Talent putting the tubes in the centrifuge and closing the lid. Videographer: Obtain multiple usable takes because this will be reused in 3.4.2.

    - Talent decanting the supernatant. Videographer: Obtain multiple usable takes because this will be reused in 3.4.3.

    - Talent flicking one tube against the other. Videographer: Obtain multiple usable takes because this will be reused in 3.5.1.

    - Talent adding lysing solution to the tube and swirling it.

  - Stop the reaction with 20 milliliters of flow buffer [1] and centrifuge the conical tube at 400 x g for 10 minutes at 4 degrees Celsius [2]. Decant the supernatant without disturbing the cell pellet [3].

    - Talent adding flow buffer to the reaction.

    - Use 3.3.1.

    - Use 3.3.2.

  - Flick the tube against another tube [1], then add 5 milliliters of cold HBSS, repeat the centrifugation, and remove the supernatant [2].

    - Use 3.3.3.

    - Talent adding HBSS to the tube, with the HBSS container in the shot.

- Staining of Cell Suspension for Mononuclear Phagocytes

  - Add 0.5 milliliters of Live-Dead stain to each sample using a P1000 pipette, making sure to dissociate the pellet completely [1]. Transfer the samples to 1.2-milliliter micro titer tubes [2] and incubate them for 15 minutes in the dark [3].

    - Talent resuspending the cells in the Live-Dead solution.

    - Talent transferring the sample to the microtiter tubes.

    - Talent putting the tubes in a dark place.

  - Meanwhile, count a 10-microliter aliquot of the sample with Trypan blue and a hemocytometer [1]. After the incubation, wash each sample by adding 400 microliters of cold Flow buffer [2].

    - Talent using the hemocytometer.

    - Talent adding flow buffer to a sample and washing it.

  - Centrifuge the tubes in a micro titer tube rack at 400 x g for 10 minutes at 4 degrees Celsius [1] and aspirate the supernatant without disturbing the cell pellet [2]. Resuspend the cells completely in 500 microliters of cold Flow buffer, then repeat the centrifugation and remove the supernatant [3].

    - Talent putting the tube rack in the centrifuge and closing the lid.

    - Talent aspirating the supernatant.

    - Talent resuspending the cells.

  - Block up to 5 million living cells in 50 microliters of FC block. Completely dissociate the cell pellet [1] and incubate it at 4 degrees Celsius for 20 minutes [2].

    - Talent adding FC block to the cells.

    - Talent putting the sample in the refrigerator. Videographer: Obtain multiple usable takes because this will be reused in 4.5.2.

  - Then, add 50 microliters of antibody staining solution to each sample, mix it completely [1], and incubate it for another 30 minutes at 4 degrees Celsius [2].

    - Talent adding antibody staining solution to the sample.

    - Use 4.4.2.

# Results

- Results: Mononuclear Phagocyte Populations in the Mouse Eye

  - FSC-A versus SSC-A properties for all analyzed events from two eyes of a single mouse are shown here [1]. Bead counts were cleaned and confirmed by plotting PE over APC-Cy7 (‘A-P-C-sigh-7’), creating a tight cluster of PE-positive and APC-Cy7-negative events [2].

    - LAB MEDIA: Figure 2 A.

    - LAB MEDIA: Figure 2 B.

  - Next, singlets and live cells were identified from all events [1]. Singlets were positively correlated in FSC-H versus FSC-A while doublet and triplet cells had greater FSC-A than FSC-H [2]. Live cells are FSC-A positive and Live-Dead stain negative [3].

    - LAB MEDIA: Figure 2 C and D.

    - LAB MEDIA: Figure 2 C.

    - LAB MEDIA: Figure 2 D

  - The initial gating strategy for the delineation of mononuclear phagocytes from live, singlet cells is shown here. Live singlets were visualized using a CD45 versus CD11b plot [1]. The absence of CD45-positive cells in the CD45 FMO confirmed the gate selection [2].

    - LAB MEDIA: Figure 3. Video Editor: Emphasize the first plot in A, B, and C.

    - LAB MEDIA: Figure 3. Video Editor: Emphasize the last plot in C.

  - Next, neutrophils, eosinophils, B-cells, NK cells, and T-cells were excluded by plotting CD45-positive live singlets with Lineage gate versus CD11b [1]. CD45-dim cells were differentiated from CD44-high cells [2].

    - LAB MEDIA: Figure 3. Video Editor: Emphasize the middle plots in A, B, and C.

    - LAB MEDIA: Figure 3. Video Editor: Emphasize the right plots in A and B.

  - Macrophage subsets were also identified. Microglia were delineated in the CD45-dim cells with CD64+MHCIILow (pronounce ‘C-D-64-positive-M-H-C-2-low’) staining [1]. MHCII- (‘M-H-C-2’) macrophages were identified as CD64+MHCII- (‘C-D-64-positive-M-H-C-2-negative’) [2]. CD11c- (‘C-D-eleven-negative’) and CD11c+ (‘C-D-eleven-positive’) macrophages were demonstrated as CD64+CD11c- ( ‘C-D-64-positive-C-D-11-negative’) and CD64+CD11c+ (‘C-D-64-positive-C-D-11-positive’) in the MHCII+ (‘M-H-C-2-positive) cells [3].

    - LAB MEDIA: Figure 4. Video Editor: Emphasize left panel of A, B, and C.

    - LAB MEDIA: Figure 4. Video Editor: Emphasize middle left panel of A and B.

    - LAB MEDIA: Figure 4. Video Editor: Emphasize middle right panel of A, B, and C.

  - It was found that laser treatment increased the amount of MHCII- (pronounce ‘M-H-C-2-negative’), CD11c- (pronounce ‘C-D-eleven-negative’), and CD11c+ (pronounce ‘C-D-eleven-positive’) macrophages [1]. Dendritic cell counts were also upregulated by laser treatment [2], while microglia and monocyte counts were not affected. Systemic perfusion had no effect on macrophage numbers [3].

    - LAB MEDIA: Figure 5. Video Editor: Emphasize A, B, and C.

    - LAB MEDIA: Figure 5. Video Editor: Emphasize F.

    - LAB MEDIA: Figure 5. Video Editor: Emphasize D and E.

# Conclusion

- Conclusion Interview Statements

  - Jeremy Lavine: Following this procedure, fluorescence-activated cell sorting, rather than analysis alone, allows for characterization of macrophage heterogeneity and its impact upon function via transcriptomic or proteomic studies.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.