TITLE:

Minimizing Hypoxia in Hippocampal Slices from Adult and Aging Mice

AUTHORS AND AFFILIATIONS:

Maja Djurišić1

1Department of Biology, Neurobiology, and Bio-X, Stanford University, Stanford, CA, USA

Corresponding author:

Maja Djurišić	(djurisic@stanford.edu)

KEYWORDS:

Hippocampal slices, adult, aging, hypoxia, ATP, NMDG, CA1, patch-clamp, field-recordings

SUMMARY:

This is a protocol for acute slice preparation from adult and aging mouse hippocampi that takes advantage of transcardial perfusion and slice cutting with ice-cold NMDG-aCSF to reduce hypoxic damage to the tissue. The resulting slices stay healthy over many hours, and are suitable for long-term patch-clamp and field-recordings.

ABSTRACT:

Acute hippocampal slices have enabled generations of neuroscientists to explore synaptic, neuronal, and circuit properties in detail and with high fidelity. Exploration of LTP and LTD mechanisms, single neuron dendritic computation, and experience-dependent changes in circuitry, would not have been possible without this classical preparation. However, with a few exceptions, most basic research using acute hippocampal slices has been performed using slices from rodents of relatively young ages, ~P20-P40, even though synaptic and intrinsic excitability mechanisms have a long developmental tail that reaches past P60. The main appeal of using young hippocampal slices is preservation of neuronal health aided by higher tolerance to hypoxic damage. However, there is a need to understand neuronal function at more mature stages of development, further accentuated by the development of various animal models of neurodegenerative diseases that require an aging brain preparation. Here we describe a modification to an acute hippocampal slice preparation that reliably delivers healthy slices from adult and aging mouse hippocampi. The protocol’s critical steps are transcardial perfusion and cutting with ice-cold sodium-free NMDG-aSCF. Together, these steps attenuate the hypoxia-induced drop in ATP upon decapitation, as well as cytotoxic edema caused by passive sodium fluxes. We demonstrate how to cut transversal slices of hippocampus plus cortex using a vibrating microtome. Acute hippocampal slices obtained in this way are reliably healthy over many hours of recording, and are appropriate for both field-recordings and targeted patch-clamp recordings, including targeting of fluorescently labeled neurons.

INTRODUCTION:

The advent of mammalian acute brain slice preparations facilitated experiments at the cellular and synaptic level that were previously possible only in invertebrate preparations like Aplysia1. Development of acute hippocampal slices was of particular significance, as it is a structure responsible for working memory and context formation, and has a specialized tri-synaptic circuitry that is amenable to easy physiological manipulation. However, the vast majority of acute brain slices are still prepared from relatively young mice and rats, as it is easier to preserve healthy neurons and circuits, and the slices remain viable for longer periods of time2-4. Here, we introduce modifications to standard slicing protocols that result in increased viability of acute hippocampal slices from adult and aging mice.

The major impediment to the long-term ex vivo viability of mammalian brain parenchyma is the initial hypoxic damage that occurs rapidly once blood flow to the brain stops following decapitation. Loss of oxygen results in fast metabolic consumption of major energy resources in the brain with the loss of phospho-creatine (P-creatine) being the most rapid, followed by glucose, adenosine triphosphate (ATP), and glycogen4. Preservation of ATP is of particular importance for the long-term health of brain slices, as ATP is needed to maintain the membrane potential via the Na-K ATPase, and consequently the neural activity5,6. The ATP level in the adult rodent brain is ~2.5 mM, and it drops precipitously within 20 s of decapitation to reach a basal steady state (~0.5 mM) at around 1 min post-decapitation4,7,8. In young animals, it takes longer to observe the same drop in ATP (~2 min); with phenobarbital anesthesia it is further slowed to 4 min4. These considerations show that preventing loss of ATP and other energy resources is a necessary strategy to prevent hypoxic damage to the brain and in turn to maintain the health of brain slices over longer periods of time, especially in adult animals.

Low temperatures slow down the metabolism. Consequently, it has been demonstrated that modest hypothermia protects brain energy reserves: in young animals, lowering body temperature by six degrees, from 37 °C to 31 °C, preserves ATP levels to around 80% of normal levels over 4 h of controlled hypoxia9. P-creatine levels are similarly preserved, as well as the overall phosphorylation potential9. This suggests that lowering body temperature prior to decapitation could be neuroprotective, as near-normal levels of ATP could be maintained through the slice cutting and slice recovery periods.

To the degree that an ATP drop cannot be completely prevented upon decapitation, a partially impaired function of the Na-K ATPase is expected, followed by depolarization via passive sodium influx. As the passive sodium influx is followed by water entry into cells, it causes cytotoxic edema and eventually pyknosis. In adult rats, replacing Na+ ions with sucrose in slice-cutting solutions has been a successful strategy to alleviate the burden of cytotoxic edema10,11. More recently, methylated organic cations that decrease sodium channel permeability12 have shown to offer more effective protection than sucrose, especially in slices from adult mice, with N-methyl-D-glucamine (NMDG) being most widely applicable across different ages and brain regions13-16.

Numerous brain-slicing protocols involve using cold temperatures only during the slice-cutting step, sometimes in combination with Na+ ion replacement strategy16,17. In young animals, these protocols appear to offer sufficient neuroprotection since the brains can be extracted quickly after decapitation because the skull is still thin and easy to remove3. However, this strategy does not produce healthy slices from adult animals. Over time, a number of laboratories studying adult rodents have introduced transcardial perfusion with an ice-cold solution to decrease the animal’s body temperature, and therefore hypoxic damage to the brain, prior to decapitation. This procedure was successfully applied across different brain regions: cerebellar slices18, midbrain slices19, neocortical slices11,20, perirhinal cortex21, rat hippocampus10,22,23, olfactory bulb24, ventral striatum25, visual cortex26.

In spite of the advantages offered by transcardial perfusion and Na+ ion replacement in preparing slices from rat and in some brain regions in mice, mouse hippocampus remains one of the most challenging areas to protect from hypoxia13,20. To date, one of the most common approaches to slicing hippocampus from aging mice and mouse models of neurodegeneration involves the classical fast slicing of the isolated hippocampi27. In the protocol described here, we minimize the loss of ATP in the adult brain by introducing hypothermia prior to decapitation by transcardially perfusing the animal with ice-cold Na+- free NMDG-aCSF. Slices are then cut in ice-cold Na+-free NMDG-aCSF. With this enhanced protocol we obtain acute hippocampal slices from adult and aging mice that are healthy for up to 10 h after slicing and are appropriate for long-term field-recordings and patch-clamp studies.

PROTOCOL:

The protocol is carried out in accordance with the Guide for the Care and Use of Laboratory Animals of the National Institutes of Health and approved by the Stanford University Institutional Animal Care and Use Committee. Methods are also in accordance with the Policies of the Society for Neuroscience on the Use of Animals and Humans in Neuroscience Research.

NOTE: All mice were maintained in a pathogen-free environment. Wild-type mice on mixed C57Bl/ 6 x SV/ 129J genetic background were used here, unless otherwise noted.

1. Setup

1.1. Prepare 1 L of 1x aCSF using the following reagent (in mM): 125 NaCl, 26 NaHCO3, 2.3 KCl, 1.26 KH2PO4, 1.3 Mg2SO4·7H2O, 2.5 CaCl2, 25 glucose (Table 1). Use this solution for the recovery chamber and subsequent recordings.

1.1.1. Store aCSF as a 10x stock solution that contains NaCl, NaHCO3, KCl, and KH2PO4. Store Mg2SO4·7H2O and CaCl2 as 1 M stock solutions. Prepare the working solution on the day of experiment from the above stock solutions, and add glucose before adjusting the final volume with 18 MΩ water.

NOTE: Depending on the brain region or mouse strain, chilled aCSF can be also used for transcardial perfusion with success11,15,26.

1.2. Prepare 300 mL of NMDG-aCSF using the follow reagents (in mM): 135 NMDG, 1 KCl, 1.2 KH2PO4, 1.5 MgCl2, 0.5 CaCl2, 20 choline bicarbonate, 10 glucose (Table 1). This volume of NMDG-aCSF is sufficient for both transcardial perfusion and cutting steps.

1.2.1. Store NMDG-aCSF as a 3x stock solution at 4 °C. Prepare working solution on the day of the experiment; add choline bicarbonate and glucose before adjusting the final volume with 18 MΩ water. Bubble the solution with 95%O2/5%CO2 to buffer it, and saturate with oxygen.

NOTE: NMDG stock starts as highly alkaline, and requires concentrated hydrochloric acid (HCl) to adjust pH to ~7.4. Addition of hydrochloric acid should be slow once below pH 8, as even a small surplus of HCl can precipitously acidify the solution to pH 3. This adjustment should be done prior to adding divalent cations. This NMDG-aCSF recipe is sodium-free. Previously published NMDG recipes use 30 mM NaHCO3 for buffering, which results in 30 mM sodium present in NMDG-aCSF13,20.

1.3. Set the vibrating microtome cutting tray and mounting disk to -20 °C.

1.4. Prepare the recovery chamber.

1.4.1. Fill the recovery chamber to just above the slice-holding mesh, keep it on the bench at room temperature, and bubble.

NOTE: The slice recovery chamber used here is very similar to the classical chambers described by previously by Edwards and Konnerth3. aCSF is kept in a glass beaker (400 mL) that holds a round acrylic frame with black nylon mesh glued to the bottom. The acrylic frame is suspended in the middle of the beaker via an acrylic hook resting over the edge of the beaker. The glass bubbler is inserted all the way to the bottom. The design allows for oxygenation of both sides of slices. Bubbling also provides constant mixing of aCSF in the recovery chamber. The black mesh provides a high contrast against the off-white slices, which are then easier to see.

1.5. Prepare the transcardial perfusion and cutting solution.

1.5.1. Chill the entire 300 mL of NMDG-aCSF in a freezer, until ice crystals start to form on the surface and the walls of the bottle. DO NOT over-freeze!

1.5.2. Place the bottle with chilled NMDG-aCSF on ice and bubble. The solution should be between 0‒2 °C.

NOTE: Slushy (NMDG-) aCSF implies having most of the solution as liquid, while a small fraction of slushy ice present will keep the solution near 0 °C throughout perfusion and cutting. Care should be taken to keep away ice crystals from the brain during cutting.

1.6. Prepare the tissue-mounting disk.

1.6.1. Take the disk out of the freezer, wipe it dry if needed.

1.6.2. Cut out a block of 5% agar about the size of a mouse brain from the previously prepared agar plate, and glue it in the center of the disk using a thin layer cyanoacrylate glue. Place the disk with glued agar on ice, and cover with paper towels until ready to use.

1.6.3.1. For 5% agar plate, dissolve 5 g of agar in 100 mL of 18 MΩ water by microwaving, and pour into a clean petri dish. Keep at 4 °C.

1.7. Take the cutting tray out of the freezer, place it into the microtome, surround it with ice, and load the blade.

2. Transcardial perfusion and brain extraction

2.1. Overdose mice via an intraperitoneal injection of rodent cocktail. Check for the anesthesia depth by checking the pain reflex (toe pinch); a mouse should not exhibit the reflex once it reaches deep anesthesia.

NOTE: Rodent cocktail recipe: Ketamine HCl (66 mg/mL), xylazine HCl (6.6 mg/mL), acepromazine maleate (0.1 mg/mL), 18 MΩ water to final volume. Dose: 0.4 mg/g body weight, 0.04 mg/g body weight, 6 x 10-4 mg/g body weight for ketamine, xylazine, and acepromazine, respectively.

2.2. Set-up the peristaltic pump for transcardial perfusion. Insert one side of the pump tubing into the bottle with iced NMDG-aCSF. Fit the other side of the tubing with the 27 G needle that will be inserted into the left ventricle.

2.3. Set the pump speed at approximately 3.5 mL/min. At this speed, the outflow of NMDG-aCSF is a fast drip, not continuous flow.

NOTE: Perfusion by gravity is a good substitute for the peristaltic pump perfusion, as long as the same approximate flow-rate can be achieved. A perfusion at high flow-rate will result in burst blood vessels in the brain. A telltale sign of an overly fast perfusion and increased pressure in blood vessels, is solution coming out of the nose of the animal.

2.4. Perfusion

2.4.1. Place the properly anesthetized mouse on its back on a diaper. Using paper tape, tape down its front and hind legs so that the chest and abdomen are exposed.

2.4.2. Cut out a large patch of the skin atop of the chest, going from below the sternum to the throat; this should provide a large working area. Grab the sternum with forceps, lift gently, and start cutting through the rib cage on both sides until the chest cavity is exposed.

2.4.3. Cut through the diaphragm in order to expose the chest cavity. The flap of the rib cage should be left attached via a thin piece of muscle. It should be possible to set it on a side without having it fall back onto the exposed chest cavity. Check that the heart is still beating. Ensure that most of the liver is visible.

2.4.4. Insert the 27 G needle into the left ventricle; the mouse’s left ventricle looks lighter in color than the right. Locate the dark red-colored right atrium. Cut through the right atrium with small scissors; the blood should start flowing out.

2.4.5. Start the pump that has been preset to the correct flow speed. If everything was done correctly, the liver should change color from red to brown soon after the start of perfusion. Monitor the liver color to determine the length of the perfusion; the liver should turn pale brown.

2.4.6. Run the pump for few more minutes. If using rectal thermometer, the body temperature should fall down to 28‒29 °C, and animal’s nose should be cold to the touch

2.5. Brain extraction.

NOTE: For this step, have the following dissection tools ready: decapitation scissors, small scissors with straight or angled blade, scalpel and #10 blade, single edge blade, #3 forceps, spatula with one side bent to 90°, a spatula, a “60°” tool (Figure 1A, B), and a small soft brush.

2.5.1. Decapitate the mouse with large decapitation scissors. Using a scalpel with #10 blade, cut open the skin on top of the skull. With small angled scissors, cut the skull at the midline. Next, using the #3 forceps, pry away the right and left halves of the skull, being careful to take the dura away with it. The brain should be exposed now.

NOTE: If the dura detaches from the skull, it will stay over the brain and has to be removed separately. The edges of the remaining dura can tighten and slice deep through the brain, potentially damaging the brain region of interest.

2.5.2. Remove the brain by scooping it out with a small spatula. Drop the brain into the NMDG-aCSF solution placed in a separate small beaker on ice. Leave it there for up to a minute.

NOTE: In addition to hypothermia, exposure to the ice-cold saline firms up the brain, which is necessary for even cutting. The entire procedure from decapitation to the brain extraction should be under 30 s.

3. Slicing

3.1. Take the brain out of the NMDG-aCSF and place it on a piece of filter paper.

3.2. Cut and remove a 60° wedge of tissue using a “60°” tool centered at the midline from the rostral end of the forebrain. Use this as the mounting surface as it provides the proper angle for hippocampal slices as discussed below (Figure 1A, B).

NOTE: Two single-edged blades held together via a home-made holder make an easy-to-use tool for making the 60° cut (Figure 1A).

3.3. Separate hemispheres down the midline with the scalpel.

3.4. Glue the hemispheres onto the mounting disk as follows. Take the mounting disk that has been on ice until now. If needed, wipe it dry again. Glue each hemisphere in front of the agar block, cut side down.

3.5. Ensure that the ventral side of the both hemispheres are touching the agar block. The agar block provides additional support during cutting and is essential for even slices. Ensure that the dorsal sides of both hemispheres are facing the blade. When glued on the cut side, each hemisphere is oriented relative to the blade in a way that ensures transverse slices of dorsal hippocampus in situ (Figure 1B).

3.5. Submerge the disk with hemispheres into a cutting chamber containing ice-cold carbogenated NMDG-aCSF.

3.6. Cut 400 µm sections. Cutting should be done in less than 10 min. Different microtomes will require different settings to achieve this time. A total of 8‒10 slices from the dorsal hippocampus region will be obtained.

4. Recovery

4.1. Transfer slices to a recovery chamber containing carbogenated aCSF at room temperature using disposable transfer pipettes with cut-off tips (Figure 1C).

NOTE: It is highly recommended to fortify recovery chamber-aSCF with 5 mM Na-ascorbate and 3 mM Na-pyruvate. Pyruvate is an energy substrate shown to boost the production of ATP in slices28, while Na-ascorbate is a free-radical quencher29.

4.2. Incubate at room temperature (22‒24 °C) for approximately 2 h before recording (up to 4 h).

NOTE: A longer incubation at room temperature results in healthier slices for longer periods of time, relative to the standard incubation at 37 °C for 30 min followed by room temperature incubation. Rewarming at 37 °C can introduce cytotoxic edema13.

REPRESENTATIVE RESULTS:

We applied the above protocol to generate hippocampus slices from CamKIIa-Cre+; PirB+/+ mice on a mixed genetic background C57Bl/ 6 x SV/ 129J, at P > 120. Large numbers of pyramidal cells in the CA1 field (Figure 2A) and subiculum (Figure 2B) appear in low contrast when observed under infrared differential contrast microscopy (IR-DIC), a hallmark of healthy cells in a slice preparation. With this preparation, a high rate of giga-ohm seals (>90%) is routinely achieved when targeting the healthiest cells approximately 20‒50 microns beneath the surface. For this success rate, it is important to use a high NA objective for IR-DIC, such as a 60x water-immersion objective, in order to achieve adequate visualization of pyramidal cells at this depth (Figure 2A, B).

Patch-clamp recordings from single neurons are easily achievable using this hippocampal slice preparation even in mice over six months of age. In Figure 2C, using mEPSC recordings in CA1 neurons, we demonstrate that induction of chemical NMDA-LTD with 3 min bath application of 20 μM NMDA results in lower mEPSC frequency in CA1 cells when assessed 60 min post-NMDA treatment. This finding suggests that NMDA-LTD causes activity-dependent pruning of synapses in CA1 in older mice (results adapted from Djurisic et al.15). Change in mEPSC amplitude was not detected. During mEPSC recordings, CA1 cells were also filled with biocytin. Figure 2D reveals an intact dendritic arbor and healthy cell habitus of filled CA1 pyramidal neurons. A robust distribution of the fluorescent dye throughout the cell allows for routine evaluation of dendritic spine properties under different conditions.

Using field-recordings, we readily observe long-term potentiation (LTP) of CA3-CA1 synapses of ~170% in slices from adult mice, suggesting maintenance of signaling cascades needed for LTP (Figure 2E, F). Network connectivity needed for a robust field excitatory postsynaptic potential (fEPSP) signal is also preserved (Figure 2E). The ability to assess synaptic plasticity in hippocampal slices from adult or aging mice is especially relevant for mouse models of neurodegenerative diseases as their hallmark synaptic dysfunction develops later in life.

Together, our results demonstrate that an acute hippocampal preparation from adult and aging mice allows assessment of synaptic function at the level of both single cells and population of cells routinely, as long as transcardial perfusion and ice-cold NMDG-based solutions are used to minimize hypoxic damage.

FIGURE LEGENDS:

Figure 1: Cutting method and recovery of transversal hippocampal slices from adult and aging mice. (A) A “60°” tool used for removing the 60° wedge from the rostral end of the brain, centered on the midline. (B) Positioning the hemispheres for cutting of transversal slices. An illustration of a 60° cut is shown on the left and the upper right panels; positioning of the two hemispheres on the surface with glue in manner shown in the lower right panel ensures a perpendicular orientation of dorsal hippocampus relative to the blade. This orientation results in transversal slices of hippocampus. Yellow structures are a 3D rendering of hippocampi within the rodent brain (gray). This illustration is adapted from SynapseWeb30. All the views are of the dorsal side of the brain. (C) Transversal hippocampal slices cut from a 4-month-old mouse in a recovery chamber.

Figure 2: Patch-clamp and synaptic plasticity measurements in hippocampal slices from mice at P120‒180. (A) Example IR-DIC micrographs of CA1 region. (B) Example IR-DIC micrographs of subiculum. In both A and B, images are taken 3 h after cutting with a 60x water-immersion objective. Calibration bar is 10 µm. (C) The effect of chemical NMDA-LTD on mEPSCs in CA1 neurons from P120‒180 mice. Upper panel are example traces of mEPSCs recorded at baseline and after NMDA-LTD pulse. Lower left panel: NMDA-LTD resulted in lower mEPSC frequency. Lower right panel: mEPSC amplitude after NMDA-LTD is not detected. N = 17 cells at baseline and n = 15 cells for NMDA-LTD, 6 mice. P = 0.004, t-test. This figure has been modified from Djurisic et al.15. (D) Example of biocytin filled CA1 pyramidal cell. (E) Example of fEPSP signal from CA1 stratum radiatum after Schaffer collateral stimulation. The gray trace is obtained during baseline recording, and the black trace is observed 20 min after tetanic stimulation. Each trace is an average of 30 consecutive traces. (F) Cumulative average of long-term potentiation of CA3-CA1 synapses after 4 trains of 100 Hz induction; n = 23 slices from 8 WT mice at approximately P90.

Table 1: Media formulations.

DISCUSSION:

The protocol described here demonstrates that hippocampal slices obtained from adult and aging mice can remain healthy and viable for many hours after cutting. The protocol is appropriate for patch-clamp recordings, as well as long-lasting field-recordings in the CA1 regions.

There are two critical steps in this protocol. First step is the transcardial perfusion step with an ice-cold solution. Fast clearance of blood is signaled by rapid change of liver color. Extracted brain should be off-white in color. If the brain remains pink, it means that systemic blood was not replaced with the cold aCSF, and the drop in body temperature was not achieved. This could be caused by improper placement of the needle into the heart, or because the heart was punctured through. Brains from poorly perfused mice should not be used for slicing. The second critical step is the use of NMDG as a sodium ion substitute. A well-known earlier variation of the protocol that successfully uses sucrose as substitute for Na+ in rat brains10,31 does not produce sufficiently healthy hippocampal slices in mice (also see Ting et al.13). The use of NMDG as a sodium ion substitute is critical for mouse hippocampal slices.

While we get reliably good hippocampal slices for electrophysiology using the described protocol, the hippocampal preparation from adult and aging animals remains challenging. Its difficulty also changes with different mouse lines and genetic backgrounds. Potential modifications to consider are NMDG-aCSF and recovery-aCSF additives like taurine, D-serine, or N-acetylcysteine (NAC) that could augment neuronal and synaptic function13,29, increase oxygenation29, as well as Cl- ion substitutions during perfusion and cutting32. Use of an interface recovery chamber is another way to maintain increased oxygenation, which is especially relevant for long-term plasticity field-recordings. The described recovery chamber (Figure 1C) is modifiable for that purpose (e.g., a culture insert dish that is moistened from below by aSCF, could substitute the submerged mesh). The slice-cutting step using sapphire or ceramic blades instead of steel ones, in combination with microtomes designed to minimize the vertical vibration (e.g., zero-z in Leica VT1200S), is another modifiable step. It could further improve the quality of the slice surface as the sharp blades decrease tissue compression exacerbated by heavy myelination of white matter around hippocampus.

This protocol is applicable to any brain region of interest, with appropriate modifications in cutting angles. Different brain regions are sensitive to hypoxia to different degrees. Thus, the stringency of the slice preparation protocol will matter. In fact, a protocol that uses NMDG-aSCF has been reported for adult mouse neocortex and striatum slice preparations13,20. This set of protocols uses an NMDG-aCSF recipe that is not sodium-free (it contains 30 mM NaHCO3)20, a potentially critical factor for regions as susceptible to hypoxia as hippocampus. Moreover, transcardial perfusion has been conducted with NMDG-aCSF at room temperature13. As discussed above, without lowering the body temperature via cold transcardial perfusion, the ATP levels will drop to about 20% of normal value within a minute4,7. This loss of ATP could be hard to recover in brains from aging animals. It follows then that re-introduction of Na+ ions with regular aCSF during the recovery step will result in more cytotoxic edema.

This protocol is applicable to the vast array of transgenic mouse lines modeling neurodegenerative diseases. Moreover, the protocol could be further modified to brain slices from other mammalian model species, as well. Together, this protocol could serve as a basis for a standardized preparation for acute hippocampal slices from aging animals, and thus facilitate comparisons across studies in the context of disease mechanisms.

ACKNOWLEDGMENTS:

I thank Dr. Carla J. Shatz for advice and support, and Dr. Barbara K. Brott and Michelle K. Drews for critically reading the manuscript. The work is supported by NIH EY02858 and the Mathers Charitable Foundation grants to CJS.

DISCLOSURES:

The author has nothing to disclose.

REFERENCES:

1	Glanzman, D. L. The cellular mechanisms of learning in Aplysia: of blind men and elephants. Biological Bulletin. 210 (3), 271-279 (2006).

2	Aitken, P. G. et al. Preparative methods for brain slices: a discussion. Journal of Neuroscince Methods. 59 (1), 139-149 (1995).

3	Edwards, F. A., Konnerth, A. Patch-clamping cells in sliced tissue preparations. Methods in Enzymology. 207 (13), 208-222 (1992).

4	Lowry, O. H., Passonneau, J. V., Hasselberger, F. X., Schulz, D. W. Effect of Ischemia on Known Substrates and Cofactors of the Glycolytic Pathway in Brain. Journal of Biological Chemistry. 239 18-30 (1964).

5	Lipton, P., Whittingham, T. S. The effect of hypoxia on evoked potentials in the in vitro hippocampus. Journal of Physiology. 287, 427-438 (1979).

6	Lipton, P., Whittingham, T. S. Reduced ATP concentration as a basis for synaptic transmission failure during hypoxia in the in vitro guinea-pig hippocampus. Journal of Physiology. 325 (1), 51-65 (1982).

7	Mandel P, H. S. Free nucleotides of the brain in various mammals. Journal of Neurochemistry. 8, 116-125 (1961).

8	Andjus, R. K., Dzakula, Z., Markley, J. L., Macura, S. Brain energetics and tolerance to anoxia in deep hypothermia. Annals of the New York Academy of Sciences. 1048, 10-35 (2005).

9	Williams, G. D., Dardzinski, B. J., Buckalew, A. R., Smith, M. B. Modest hypothermia preserves cerebral energy metabolism during hypoxia-ischemia and correlates with brain damage: a 31P nuclear magnetic resonance study in unanesthetized neonatal rats. Pediatric Research. 42 (5), 700-708 (1997).

10	Gasparini, S., Losonczy, A., Chen, X., Johnston, D., Magee, J. C. Associative pairing enhances action potential back-propagation in radial oblique branches of CA1 pyramidal neurons. Journal of Physiology. 580 (3), 787-800 (2007).

11	Thomson, A. M., Bannister, A. P. Release-independent depression at pyramidal inputs onto specific cell targets: dual recordings in slices of rat cortex. Journal of Physiology. 519 (1), 57-70 (1999).

12	Hille, B. The permeability of the sodium channel to organic cations in myelinated nerve. Journal of General Physiology. 58 (6), 599-619 (1971).

13	Ting, J., Daigle, T., Chen, Q., and Feng, G. in Patch-Clamp Methods and Protocols Vol. 1183 (ed M. Martina, Taverna, S.) Ch. 14, 221-242 (Springer, 2014).

14	Jiang, X. et al. Principles of connectivity among morphologically defined cell types in adult neocortex. Science. 350 (6264), 1-10 (2015).

15	Djurisic, M., Brott, B. K., Saw, N. L., Shamloo, M., Shatz, C. J. Activity-dependent modulation of hippocampal synaptic plasticity via PirB and endocannabinoids. Molecular Psychiatry. 24 (8), 1206-1219 (2019).

16	Djurisic, M. et al. PirB regulates a structural substrate for cortical plasticity. Proceedings of the National Academy of Sciences U S A. 110 (51), 20771-20776 (2013).

17	Vidal, G. S., Djurisic, M., Brown, K., Sapp, R. W., Shatz, C. J. Cell-Autonomous Regulation of Dendritic Spine Density by PirB. eNeuro. 3 (5), 1-15 (2016).

18	Blot, A., Barbour, B. Ultra-rapid axon-axon ephaptic inhibition of cerebellar Purkinje cells by the pinceau. Nature Neuroscience. 17 (2), 289-295 (2014).

19	Lammel, S., Ion, D. I., Roeper, J., Malenka, R. C. Projection-specific modulation of dopamine neuron synapses by aversive and rewarding stimuli. Neuron. 70 (5), 855-862 (2011).

20	Ting, J. T. et al. Preparation of Acute Brain Slices Using an Optimized N-Methyl-D-glucamine Protective Recovery Method. Journal of Visual Experiments. (132), e53825, doi:10.3791/53825 (2018).

21	Moyer, J. R., Jr., Brown, T. H. Methods for whole-cell recording from visually preselected neurons of perirhinal cortex in brain slices from young and aging rats. Journal of Neuroscience Methods. 86 (1), 35-54 (1998).

22	Losonczy, A., Magee, J. C. Integrative properties of radial oblique dendrites in hippocampal CA1 pyramidal neurons. Neuron. 50 (2), 291-307 (2006).

23	Frick, A., Magee, J., Johnston, D. LTP is accompanied by an enhanced local excitability of pyramidal neuron dendrites. Nature Neuroscience. 7 (2), 126-135 (2004).

24	Alvarado-Martinez, R., Salgado-Puga, K., Pena-Ortega, F. Amyloid beta inhibits olfactory bulb activity and the ability to smell. PLoS One. 8 (9), e75745 (2013).

25	Brooks, J. M., O'Donnell, P. Kappa Opioid Receptors Mediate Heterosynaptic Suppression of Hippocampal Inputs in the Rat Ventral Striatum. Journal of Neuroscience. 37 (30), 7140-7148 (2017).

26	Goel, A., Lee, H. K. Persistence of experience-induced homeostatic synaptic plasticity through adulthood in superficial layers of mouse visual cortex. Journal of Neuroscience. 27 (25), 6692-6700 (2007).

27	Mathis, D. M., Furman, J. L., Norris, C. M. Preparation of acute hippocampal slices from rats and transgenic mice for the study of synaptic alterations during aging and amyloid pathology. Journal of Visual Experiments. (49), e2330, doi:10.3791/2330 (2011).

28	Izumi, Y., Zorumski, C. F. Neuroprotective effects of pyruvate following NMDA-mediated excitotoxic insults in hippocampal slices. Neuroscience Letters. 478 (3), 131-135 (2010).

29	Hajos, N., Mody, I. Establishing a physiological environment for visualized in vitro brain slice recordings by increasing oxygen supply and modifying aCSF content. Journal of Neuroscience Methods. 183 (2), 107-113 (2009).

30	Fiala, J. C., Spacek J. https://synapseweb.clm.utexas.edu/hippocampus-rat, (1999).

31	Combe, C. L., Canavier, C. C., Gasparini, S. Intrinsic Mechanisms of Frequency Selectivity in the Proximal Dendrites of CA1 Pyramidal Neurons. Journal of Neuroscience. 38 (38), 8110-8127 (2018).

32	Rothman, S. M. The neurotoxicity of excitatory amino acids is produced by passive chloride influx. Journal of Neuroscience. 5 (6), 1483-1489 (1985).