# Submission ID #: 61405

# Scriptwriter Name: Bridget Colvin

Project Page Link: https://www.jove.com/account/file-uploader?src=18728573

Title: Modified Methods for Loading of High-Throughput DNA Extraction Plates Reduce Potential for Contamination

Authors and Affiliations: Gordon F. Custer1,2* and Reilly R. Dibner3,4*

*These authors contributed equally

1 Department of Ecosystem Science and Management, University of Wyoming

2 Program in Ecology, University of Wyoming

3 EPSCoR-IDEA, University of Wyoming

4 Department of Botany, University of Wyoming

# Corresponding Author:

# Gordon F. Custer

# gcuster@uwyo.edu

# Co-authors:

# rdibner@uwyo.edu

## Author Questionnaire

1. Microscopy: Does your protocol involve video microscopy, such as filming a complex dissection or microinjection technique?  N

2. Software: Does the part of your protocol being filmed demonstrate software usage?  N

3. Filming location: Will the filming need to take place in multiple locations (greater than walking distance)?   N

Protocol Length

Number of Shots: 46

# Introduction

- Introductory Interview Statements

REQUIRED:

  - Gordon Custer: This protocol provides a technique for increasing the efficiency of 96-well DNA extraction plate loading while simultaneously decreasing the risk of sample cross contamination [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

REQUIRED:

  - Reilly Dibner: This technique reduces the chances for contamination during the critical first step of DNA extraction through the individual addition of the samples to each well in the 96-well plate [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

OPTIONAL:

  - Gordon Custer: This methodology can be applied to any of the diverse fields of microbiome research [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

# Protocol

- Laboratory Bench and Tool Preparation

  - Before beginning an experiment, mist the bench top with 70% ethanol [1].

    - WIDE: Talent spraying benchtop, with ethanol container visible in frame

  - After wiping, let the bench air dry [1] before spraying the bench with 10% bleach [2].

    - Talent wiping bench

    - Talent spraying bench with bleach, with bleach container visible in frame

  - After wiping and air drying [1], dip the micro-scoopula, spatula, and curved surgical scissors into 95% ethanol [2] and expose the tools to a flame [3].

    - Talent wiping bench

    - Talent dipping tool(s) into ethanol, with ethanol container visible in frame

    - Talent exposing tool(s) to flame

  - Then dip each tool in 10% bleach [1] and allow the tools to air dry [2-TXT].

    - Talent dipping tool in bleach, with bleach container visible in frame

    - Talent placing tool(s) to air dry TEXT: Re-sterilize tools between samples

- Sub-Sampling and Sample Preparation

  - Prior to sub-sampling, ethanol-sterilize gloves [1] and thoroughly homogenize the soil samples [2].

    - WIDE: Talent sterilizing gloves, with ethanol container visible in frame

    - Talent homogenize sample(s) and returns sample(s) to cooler to store on ice.

  - Next, assign a sample ID with well location to each tube [3.5.2]. Fill 95 labeled 2-milliliter centrifuge tubes with one sample per tube until each tube is approximately half full [1].

3.5.2 Talent writing ID’s in notebook. NOTE: This shot was moved from 3.5

    - Talent adding sample to tube

  - The 96th tube should be used as an extraction blank [1].

    - Shot of 96th tube labeled as extraction blank

  - Place the sub-sample tubes on ice [1] and label 96, sterile, 200-microliter, flat-capped PCR tubes according to the well labels for a 96-well plate [2].

    - Talent placing sample tube(s) onto ice

    - Talent labeling tube(s)

  - Then place the labeled 200-microliter tubes in order in a 96-well rack [1] [2].

    - Talent placing tube(s) into rack

    - Talent writing IDs onto tubes or into notebook or similar.

- Plate Preparation

  - To prepare a 96-well plate for the experiment, remove the cover from the plate [1] and place the cover into a sterile plastic bag [2].

    - WIDE: Talent removing lid from plate

    - Talent placing plate into bag

  - Seal the bag to prevent contamination [1] and cover the plate with a precut piece of pierceable sealing film [2].

    - Talent sealing plate

    - Talent covering plate with film

  - Then use a rubber roller to firmly adhere the seal to the plate [1] and store the plate at 4 degrees Celsius [2].

    - Seal being rolled

    - Talent placing plate into refrigerator

- Sub-Sample Transfer

  - To transfer the sub-samples to the plate, place 24 of the 2-milliliter sub-sample tubes into an ice block for cold storage [1] and use the sample name and well location sheet to select the appropriate corresponding 200-microliter flat-capped tubes [2].

    - WIDE: Talent placing tube(s) into ice block

    - Talent selecting 200-microliter tube(s), with sheet visible in frame

  - Vortex the sub-samples one at a time for 5 seconds per sample to ensure homogenization [1] and load approximately 200 microliters of each sample into the appropriate corresponding PCR tube [2].

    - Sample being vortexed Videographer: Important step

    - Talent loading sample into tube Videographer: Important step

  - When all 24 of the samples have been loaded, use a bleach-soaked paper wipe to clean the outside of one PCR tube [1] and invert and tap the tube on the bench to move the sample to the top of the tube [2].

    - Talent wiping tube(s) Videographer: Important step

    - Talent inverting/tapping tube Videographer: Important step

  - Using the flame-sterilized and bleached scissors, clip the bottom of the PCR tube to create an opening for the sample [1] and pass the tube over the plate with the cut end facing up until the appropriate well has been reached [2].

    - Tube being clipped Videographer: Important step

    - Tube being passed over plate to well Videographer: Important step

  - Tilt the plate slightly to facilitate puncturing of the precut pierceable sealing film [1] and, with the tube directly above the correct well, quickly but carefully invert the plate so that the cut tip fits into the well [2].

    - Plate being tilted Videographer: Important/difficult step

    - Tube above well, then plate being inverted Videographer: Important/difficult step

  - Use a sterilized tool to tap the top of the tube until all of the soil has fallen from the tube into the well [1] and leave the tube in the well with the lid closed [2].

    - Tube being tapped Videographer: Important step

    - Shot of closed tube in well Videographer: Important step

  - When all of the samples have been loaded in the same manner, remove one 200-microliter flat-capped PCR tube [1] and add 750 microliters of bead solution to the sample well [2].

    - Tube being removed Videographer: Important step

    - Bead solution being added to well, with solution container visible in frame Videographer: Important step

  - Place the 200-microliter flat-capped PCR tube back into the well, pushing it all the way down into the well [1], and use a sharpie to mark the top of the tube to indicate that the well has been loaded [2].

    - Tube being pushed down into well

    - Tube lid being marked

  - When all of the wells have been loaded with beads, return the 24 sample tubes to minus 20-degree Celsius storage [1] and add 24 new sub-samples tubes to the ice block to allow the next set of samples to be loaded into the 96-well plate as just demonstrated [2].

    - Talent placing samples at -20 °C

    - Talent adding tube(s) to ice block

  - When all 95 of the wells have been loaded with sample and bead solution [1], add bead solution only to the 96th well [2]. Remove all tubes, passing them over covered wells only [Added shot], and carefully remove the pierceable film from the plate [3].

    - Shot of plate with tubes in 95 wells

    - Bead solution being added to 96th well

Add new shot: Tubes being removed (before film removed)

    - Film being removed

  - Then carefully transfer the plate cover from the plastic bag to the plate [1] and place the plate at minus 20 degrees Celsius until the planned extraction [2].

    - Talent covering plate with lid

    - Talent placing plate at -20 °C

## Protocol Script Questions

A. Which steps from the protocol are the most important for viewers to see?

5.2.-5.7.

B. What is the single most difficult aspect of this procedure and what do you do to ensure success?

5.5 Make sure the 200-microliter tube is upright when passed over the plate. Carefully tilt the plate and quickly insert the tube into the correct well.

# Results

- Results: Representative Blank Well DNA Concentration

  - A comparison of plate loading methods shows that the demonstrated method results in the lowest DNA concentration within the blank wells [1].

    - LAB MEDIA: Figure 2 Video Editor: please emphasize Custer Dibner data box

  - Using this method, the DNA concentration was significantly lower [1] than when using the method proposed by McPherson et al [2], although the DNA concentrations by this method were not statistically different than the Qiagen default method [3].

    - LAB MEDIA: Figure 2

    - LAB MEDIA: Figure 2 Video Editor: please emphasize McPherson et al. data box

    - LAB MEDIA: Figure 2 Video Editor: please emphasize Qiagen data box

  - All three methods produce mean DNA concentrations under 2 nanograms/microliter [1], although only this new method produces wells with no measurable DNA concentration [2].

    - LAB MEDIA: Figure 2 Video Editor: please draw dotted horizontal line across figure from 2 ng/microliter

    - LAB MEDIA: Figure 2 Video Editor: please circle bottom SD line for Custer Dibner box

# Conclusion

- Conclusion Interview Statements

  - Reilly Dibner: To minimize the potential for spillage, keep the 200-microliter tube upright when moving it over the plate, tilt the plate, and insert the tube into the correct well [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera (5.5)