# Submission ID #: 61450

# Scriptwriter Name: Bridget Colvin

Project Page Link: https://www.jove.com/account/file-uploader?src=18742298

Title: Injection of Hydrogel Biomaterial Scaffolds to The Brain After Stroke

Authors and Affiliations: Katrina L Wilson1, S. Thomas Carmichael2, and Tatiana Segura1,3,4.

1Department of Biomedical Engineering, Duke University

2Department of Neurology, University of California Los Angeles

3Department of Neurology, Duke University

4Department of Dermatology, Duke University

# Corresponding Authors:

# Tatiana Segura

# Tatiana.segura@duke.edu

# S. Thomas Carmichael

# scarmichael@mednet.ucla.edu

# All Authors:

katrina.wilson@duke.edu

# scarmichael@mednet.ucla.edu

# Tatiana.segura@duke.edu

## Author Questionnaire

1. Microscopy: Does your protocol demonstrate the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or similar?  N

2. Software: Does the part of your protocol being filmed demonstrate software usage?  N

3. Filming location: Will the filming need to take place in multiple locations (greater than walking distance)?   No, S. Thomas Carmichael will provide video.

Protocol Length

Number of Shots: 49

# Introduction

- Introductory Interview Statements

REQUIRED:

  - Tatiana Segura: The reproducible injection of hydrogel scaffolds into a rodent ischemic stroke model allows for the study of how materials and other delivered therapeutics affect brain repair, regeneration, and behavioral improvement.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera	Suggested b-roll: 3.8.1.

  - Tatiana Segura: The main advantages of using the photothrombotic stroke model to study brain-material interactions are the reproducible location and size, which allow injection of materials into the stroke core without imaging.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

  - S. Thomas Carmichael: The phototrombotic stroke model is designed to be a relevant model of ischemic stroke in a region of the brain, the motor cortex, that is commonly affected by stroke in humans and to provide a relevant behavioral read out for stroke, which is motor control of the limb so as to evaluate a candidate therapeutic like a biopolymer hydrogel.

    - Added: INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera NOTE: Authors recorded this on their own, it’s uploaded to AWS

Ethics Title Card

  - Procedures involving animal subjects have been approved by the Institutional Animal Care and Use Committee (IACUC) at Duke University.

# Protocol

- Photothrombotic Stroke Model

# NOTE: Authors mentioned that some shots were merged, please check videographer’s log (I didn’t receive any videographer notes for this one

  - Confirm a lack of response to toe pinch in an anesthetized mouse [1-TXT], then weigh the mouse to determine how much Rose Bengal dye to inject [2] and secure it to a stereotaxic device equipped with a laser [3-TXT]. Videographer: This step is difficult!

    - WIDE: Talent pinching toe Videographer: More Talent than mouse in shot TEXT: Anesthesia: 4% -> 1.5% isoflurane

    - Talent placing mouse onto balance Videographer: More Talent than mouse in shot

    - Talent securing mouse to device Videographer: More Talent than mouse in shot TEXT: See text for device and laser preparation details

  - After shaving and disinfecting the surgical area, use forceps to tent the skin between the ears and eyes [1] and insert a blade into the raised skin to make a lateral, 1 to 2-centimeter midline incision in the skin [2].

    - Skin being tented

    - Incision being made

  - Separate the skin [1] and press the forceps lightly onto the parietal bones to locate the bregma [2].

    - Skin being separated

    - Bones being pressed

  - Use a surgical marker to label the bregma with a dot [1] and, wearing laser safety goggles, move the laser over the skull of the mouse [2], fixing the stereotaxic device at a 90-degree angle [3]. Videographer: This step is important!

    - Bregma being marked

    - Laser being moved over skull

    - Device being fixed at 90° angle

  - Select the 10-milliwatt pre-set [1] and turn on the laser [2].

    - Talent selecting pre-set

    - Talent turning on laser

  - Adjust the x- and y-axis until the laser is directly over the bregma [1].

    - Talent adjusting axes

  - Reset the x and y on the digital display console [1] and move the laser 1.8 millimeters left of the bregma [2].

    - Talent resetting x and y

    - Talent moving laser

  - Bring the laser as close to the skull as possible without letting the laser touch the skull [1] and move the laser 2.2 millimeters left of the bregma to ensure that it can move without any obstructions [2].

    - Laser being moved closed to skull

    - Laser being moved left of bregma

  - When the laser is in position, turn it off [1] and use a disposable 29-gauge needle to intraperitoneally inject the appropriate volume of Rose Bengal dye [2].

    - Talent turning off laser

    - Dye being injected

  - Immediately start a timer to allow the dye to circulate systemically for 7 minutes [1].

    - Talent starting timer

  - While the dye is circulating, select the pre-set for 40 milliwatts without turning on the laser [1].

    - Talent selecting pre-set

  - When the timer goes off, turn on the laser [1] and set the timer for 10 minutes [2].

    - Talent, wearing goggles, turning on laser

    - Talent setting timer

  - When the timer goes off again, move the laser to 2.2 millimeters left of the bregma [1] and set another 10-minute timer [2].

    - Laser being moved

    - Talent setting timer

  - When the second 10-minute timer has gone off, change the laser back to 10 milliwatts [1] and move it to 2 millimeters left of the bregma [2].

    - Talent changing laser to 10 mW

    - Laser being moved

  - Mark this spot with the surgical marker [1] and turn off the laser [2].

    - Spot being marked

    - Talent turning off laser

  - Lift the laser [1] and unlock it from the 90-degree angle so it can be moved away from the mouse [2].

    - Talent lifting laser Videographer: Can combine 3.16.1. and 3.16.2. as appropriate

    - Talent unlocking laser Videographer: Can combine 3.16.1. and 3.16.2. as appropriate

  - Use cotton to apply sterile saline to the surgical area as necessary [1] and drill the 2-millimeter mark in small bursts perpendicular to the surface of the skull [2-TXT]. Videographer: This step is important!

    - Saline being applied

    - Mark being drilled TEXT: Remove excess blood with cotton

  - Place a small wipe next to the mouse [1] and use the forceps to pull the skin close together and upward [2].

    - Wipe being placed

    - Skin being pulled

  - Dab any large drops of surgical glue at the end of the tip on the wipe [1] and sparingly apply surgical glue to the skin [2].

    - Glue being dabbed

    - Glue being applied

  - Hold skin together with forceps for 10 seconds [1], then continue to apply glue until no additional openings are visible in the skin [2].

    - Skin being held

    - Glue being applied

- Hydrogel Injection

  - For hydrogel injection into the stroke region, attach injection pumps to the stereotaxic device [1] and set the flow rate to 1 microliter per minute and the volume to 4 microliters [2].

    - WIDE: Talent attaching pump(s) to device

    - Talent setting flow rate and/or volume

  - Reopen the incision over the brain [1] and use saline-soaked cotton to clean any debris from the skull [2]. A white-yellow circle of tissue and the burr hole from the drill will be visible [3-TXT].

    - Incision being opened

    - Tissue/debris being wiped

    - Shot of tissue/burr hole TEXT: If no dead tissue visible, no stroke occurred

  - If the burr hole is not centered above the stroke, re-drill to center it [1] and lock the injection pump at 90 degrees over the mouse [2].

    - Talent attaching bit to drill

    - Pump being locked over mouse

  - Clean the glass syringe with sterile saline [1] and remove the needle [2].

    - Syringe being cleaned

    - Syringe being disassembled

  - Use a 25-microliter positive displacement pipette to backload the syringe with 10 microliters of hydrogel [1] and use the plunger to push the gel all the way to the front of the syringe [2].

    - Talent loading syringe with hydrogel

    - Gel being pushed to front of syringe

  - Reassemble the syringe [1] and continue pushing until the gel visibly exudes from the needle [2].

    - Needle being attached to syringe

    - Gel being pushed

  - Load the syringe onto the pump [1-TXT] and move the pump in the x and y directions to orient the syringe over the burr hole [2].

    - Talent loading syringe TEXT: See text for syringe position adjustment details

    - Syringe being oriented

  - Move the pump in the z direction until the needle touches the top of the hole [1] and reset the z on the digital display console [2]. Videographer: This step is important!

    - Needle being moved to burr hole

    - Z being reset

  - Move the syringe 0.75 millimeters in the z direction [1-TXT] and press Start on the pump console to inject 4-6 microliters of the hydrogel at a 1 microliter per minute flow rate [2-TXT]. Videographer: This step is difficult and important!

    - Syringe being moved TEXT: If syringe resistance or bending, re-drill hole

    - Start being pressed TEXT: Optional: Use stereomicroscope for injection

  - When all of the hydrogel has been delivered, set a 5-minute timer to allow the gel to begin crosslinking [1].

    - Talent setting timer, with stereotaxis device visible in frame as possible Videographer: More Talent than mouse in shot

  - After 5 minutes, turn the z knob to slowly pull up the syringe [1]. When the needle is far enough away from the skull, unlock and remove the pump [2].

    - Knob being turned/needle being lifted

    - Talent unlocking/removing pump

  - Then close the skin over the skull [1] and place the mouse in a clean cage with monitoring until it has recovered from the anesthesia [2].

    - Suture/glue being placed

    - Talent placing mouse into cage Videographer: More Talent than mouse in shot

- Sample Collection

  - At the appropriate experimental endpoint, use blunt thin forceps [1] to carefully remove pieces of the skull to expose the brain [2-TXT], using small surgical scissors to cut away the bone over the stroke site as necessary [3]. Videographer: This step is difficult and important!

    - WIDE: Talent selecting forceps

    - Skull pieces being removed TEXT: Euthanasia: anesthesia + decapitation

    - Skull pieces being cut

  - After harvesting, place the brain in 10-15 milliliters of 4% paraformaldehyde overnight for 16-24 hours [1] before placing the tissue into 30% sucrose [2].

    - Talent placing brain in PFA, with PFA container visible in frame

    - Brain being placed into sucrose, with sucrose container visible in frame

  - The brain will be ready for cryosection when it sinks to the bottom of the tube [1-TXT].

    - Shot of brain at bottom of tube TEXT: i.e., 2-3 d

# Results

- Results: Representative Hydrogel Biomaterial Scaffold Injection and Analysis

  - Five days after stroke induction [1], the infarct can be visualized during surgery [2] and by TTC (T-T-C) and immunohistochemically stained slides imaging [3-TXT].

    - LAB MEDIA: Figures 1B and 2B

    - LAB MEDIA: Figures 1B and 2B Video Editor: please emphasize tissue within dashed circle in Figure 1B

    - LAB MEDIA: Figures 1B and 2B Video Editor: please emphasize Figure 2B TEXT: TTC: 2,3,5-triphenyltetrazolium chloride

  - An increase in the laser diameter with a 2x lens [1] leads to a visual increase in the stroke lesion that spans over 2.5 millimeters [2] versus a single 1-millimeter section using the laser only [3].

    - LAB MEDIA: Figure 2C

    - LAB MEDIA: Figure 2C Video Editor: please emphasize Stroke 2x lens images

    - LAB MEDIA: Figure 2C Video Editor: please emphasize Stroke no lens images

  - Hyaluronic acid-based hydrogel microparticle injection into the stroke lesion five days after stroke [1] results in the formation of microannealed particles with a porous scaffold that facilitates cell migration into the stroke core [2].

    - LAB MEDIA: Figure 4B

    - LAB MEDIA: Figure 4B Video Editor: please emphasize Hydrogel images

  - Hyaluronic hydrogel injections that match the modulus of the cortex [1] lead to a significant decrease in the reactive astrocytes within the peri-infarct [2] as well as inside the microannealed particle gel [3].

    - LAB MEDIA: Figures 5C-5E Video Editor: please emphasize HA-MAP images in Figure 5C

    - LAB MEDIA: Figures 5C-5E Video Editor: please emphasize HA-MAP (infarct) data cluster in Figures 5D and 5E

    - LAB MEDIA: Figures 5C-5E Video Editor: please emphasize HA-MAP data cluster in Figures 5D and 5E

  - Furthermore, microglia that were also within the hydrogel express pro-repair M2 markers [1], suggesting that microannealed particle hydrogel can reduce astrocyte reactivity [2] and promote a more anti-inflammatory environment with pro-recovery astrocytes and microglia in just two days following injection [3].

    - LAB MEDIA: Figures 5F-5H Video Editor: please emphasize bottom row merged images

    - LAB MEDIA: Figures 5F-5H Video Editor: please add/emphasize asterisks in Figure 5H

    - LAB MEDIA: Figures 5F-5H

# Conclusion

- Conclusion Interview Statements

  - Tatiana Segura: The scaffolds-tissue can be retrieved and used for molecular, cellular or tissue level analyses such as scRNA-seq, flowcytometry, or clarity. Also, behavioral analysis can be performed to assess behavioral improvement.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera   Suggested b-roll: 4.2.2.

  - S. Thomas Carmichael: this approach delivers a candidate therapeutic, a biopolymer hydrogel, into the stroke cavity, which is a potential cavity in stroke and area of dead tissue that can accept a transplant. Additionally, this cavity is adjacent to the area of the brain that undergoes the most plasticity after stroke, the peri-infarct tissue. So a candidate therapeutic like a biopolymer hydrogel can deliver cells or small molecules to the part of the brain that is really the target for recovery in stroke.

    - Added: INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera    NOTE: Authors recorded this on their own, it’s uploaded to AWS