# Submission ID #: 61566

# Scriptwriter Name: Bridget Colvin

Project Page Link: https://www.jove.com/account/file-uploader?src=18777678

Title: Dynamic Measurement and Imaging of Capillaries, Arterioles, and Pericytes in Mouse Heart

Authors and Affiliations: Guiling Zhao1, Humberto C. Joca1, and W. Jonathan Lederer1

1Laboratory of Molecular Cardiology, Center for Biomedical Engineering and Technology and Department of Physiology, University of Maryland School of Medicine

# Corresponding Author:

# Guiling Zhao

# gzhao@som.umaryland.edu

# Co-authors:

jlederer@som.umaryland.edu

# hjoca@som.umaryland.edu

## Author Questionnaire

1. Microscopy: Does your protocol demonstrate the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or similar?  Y, Dissecting microscopes: Olympus, Japan, SZX12 and Nikon

2. Software: Does the part of your protocol being filmed demonstrate software usage?  Y

3. Filming location: Will the filming need to take place in multiple locations (greater than walking distance)?   N

Protocol Length

Number of Shots: 51

# Introduction

- Introductory Interview Statements

REQUIRED:

  - Guiling Zhao: This method can be used to study coronary microcirculation in living murine heart tissue and vascular tree components by ex vivo monitoring of the arterial perfusion pressure and flow [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

REQUIRED:

  - Humberto C. Joca: This method provides a useful platform for studying the function of pericytes and reagents on microcirculation within the heart by simultaneously measuring the vascular diameter and arterial luminal pressure [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Ethics Title Card

  - Procedures involving animal subjects have been approved by the Institutional Animal Care and Use Committee (IACUC) at University of Maryland School of Medicine.

# Protocol

- Cannula Preparation

  - To prepare a cannula for the experiment, use a micropipette puller to produce two cannulae with long, thin tips from a single clean, borosilicate glass tube [2].

    - WIDE: Talent pulling pipette TEXT: 047" X .040" 6-8" Length

  - Use a fine pair of scissors and a dissecting microscope to cut the tip of each cannula to a final tip diameter of 100-150 micrometers [1] and fire polish the tips to slightly round the sharp edges [2].

    - SCOPE: Cannula tip being cut Videographer: Important step

    - Tip being polished Videographer: Important step

  - Use a platinum wire positioned on the side of the shaft approximately 2 millimeters from the tip to bend the cannula along the shaft to an about 45-degree angle [1] and insert the open end of the cannula into the cannula holder  [2].

    - Shaft being bent with wire Videographer: Important step

    - Cannula being inserted into cannula holder

  - Then tighten the fitting [1] and use a 5-milliliter syringe to flush the cannula and tubing with Tyrode’s solution. Mount the cannula holder onto micromanipulation of a pressure myograph chamber [2]. NOTE: The added VO text was originally numbered 2.5 but there seem to be no shots associated with it, so I moved it to this step.

    - Fitting being tightened

    - Talent flushing cannula and tubing, with Tyrode’s solution container visible in frame

- Mouse Heart Extraction

  - To collect a mouse heart for the experiment, first intraperitoneally inject 500 microliters of heparin into an anesthetized, 8 to 16-week-old C57BL-6 mouse [1-TXT].

    - WIDE: Talent injecting mouse Videographer: More Talent than mouse in shot TEXT: Heparin

  - After 10 minutes, place the mouse onto a heated bed in the supine position [1] and stabilize the paws with labeling tape [2].

    - Talent mouse onto bed NOTE: 3.2.1 and 3.2.2 in one shot Videographer: More Talent than mouse in shot

    - Paw being taped

  - Use forceps and surgical scissors to make an incision in the abdominal skin above the diaphragm [1] and cut the diaphragm and the sternum [2] to allow dissection of the heart from the thoracic cavity [3-TXT].

    - Incision being made NOTE: 3.3.1 - 3.4.1 in one shot

    - Diaphragm and/or sternum being cut

    - Heart being dissected TEXT: Cut as close as possible to dorsal thoracic wall

  - Place the heart into ice-cold Tyrode’s solution containing 300-millimolar BDM (B-D-M) [1-TXT] and remove any connective tissues from the heart as necessary [2].

    - Talent placing heart into solution, with BDM container visible in frame TEXT: BDM: 2,3-butanedione monoxime

    - Tissue being removed

  - Then place the heart into a pre-chilled dissecting chamber filled with fresh Tyrode’s solution supplemented with 30-millimolar BDM [2].

    - Talent placing heart into chamber, with solution container visible in frame

- Septal Artery Preparation and Cannulation

  - To prepare the septal artery for cannulation, turn on the servo pump [1] and set the servo controller pressure to “flow” mode [2].

    - WIDE: Talent turning on pump NOTE: 4.1.1 and 4.1.2 in one shot

    - Talent setting pressure to flow

  - Pin the heart onto the PDMS, taking care to avoid damage to the middle area of the tissue [1-TXT], and place the tissue under a dissecting microscope [2].

    - Heart being pinned TEXT: PDMS: polydimethylsiloxane NOTE: 4.2.1 and 4.2.2 maybe in one shot

    - Microscope arm being placed above dish

  - Remove both the right and left atria [1] and cut open the right ventricle [2].

    - SCOPE: At least one atrium being removed NOTE: Scope shots are inverted

    - SCOPE: Ventricle being opened

  - Remove the right ventricular free wall [1] and expose the septal artery [2].

    - SCOPE: Wall being removed Videographer: Important step

    - SCOPE: Artery being exposed Videographer: Important step

  - Use a 30-micrometer-diameter nylon thread to tie a loose knot around the septal artery [1].

    - SCOPE: Thread being placed/knot being tied Videographer: Important step

  - Use fine scissors to remove the left ventricular free wall [1] and transfer the papillary muscle preparation to the experimental chamber [2].

    - SCOPE: Wall being removed Videographer: Important step

    - Talent placing tissue into experimental chamber under microscope

  - Use a micromanipulator to insert a cannula into the septal artery [1] and secure the cannula with the suture [2].

    - SCOPE: Artery being cannulated Videographer: Important step

    - SCOPE: Suture being tightened   Videographer: Important/difficult step

  - Then use pins to secure the papillary muscle to the chamber floor with the tip of the cannula parallel to the arterial wall so that a clear view of the vasculature can be obtained [1] and use the syringe to gradually flush the solution through the cannula [2].

    - SCOPE: Muscle being pinned to chamber Videographer: Important/difficult step

    - SCOPE: Solution being expelled Videographer: Important step

- Preparation Stabilization

  - To stabilize the tissue preparation, turn on a peristaltic pump [1] and continuously superfuse the preparation with pre-gassed physiological saline solution at a 3-4 milliliters/minute flow rate [2].

    - WIDE: Talent turning on pump

    - Solution being perfused

  - Turn on the temperature controller for the superfusion solution [1] and adjust the temperature of the bath superfusate to 35-37 degrees Celsius [2].

    - Talent turning on controller

    - Talent measuring bath temperature

  - Connect the cannula to the servo pump [1] and adjust the flow to set the pressure to approximately 10 millimeters of mercury [2-TXT].

    - Talent connecting cannula to pump

    - Talent adjusting flow TEXT: Monitor flow and pressure throughout experiment

  - Let the superfusion run for about 10 minutes [1].

    - Talent setting timer

  - Then increase the flow of the luminal solution to set the artery pressure to approximately 60 millimeters of mercury [1] and allow the sample to stabilize for 30-60 minutes with monitoring [2].

    - Talent increasing flow

    - Talent setting timer, with setup visible in frame

- Fluorescence-Tagged Wheat-Germ Agglutinin (WGA) Loading and Confocal Arteriole and Capillary Imaging

  - To load the sample with wheat germ agglutinin, perfuse the preparation with 5 milliliters of Tyrode’s solution supplemented with 100 micrograms of Alexa Fluor 488 (four-eighty-eight)-conjugated wheat germ agglutinin [1].

    - WIDE: Talent switching perfusate, with WGA container visible in frame

  - After 30 minutes, change the perfusate back to normal Tyrode’s solution [1] and turn on the confocal microscope system [2].

    - Talent changing perfusate to Tyrode’s solution, with solution container visible in frame

    - Talent turning on system

  - Use the microscope at a low power of magnification to locate the septal artery in transmitted light mode [1-TXT] and begin imaging with the spinning disk confocal [2].

    - Talent at microscope, locating artery TEXT: Follow cannula position to locate septal artery NOTE: Scope shot also available

    - Talent opening software imaging, with monitor visible in frame

  - Select a 40x objective magnification and the 488-nanometer excitation laser [1] and adjust the laser intensity and the sampling rate [3].

    - Talent selecting objective

    - SCREEN: video 2: 00:00-00:08

  - To define the imaging range, set the top and bottom imaging positions for the microscope and set the step size and z-stack imaging parameters [1].

    - SCREEN: video 3: 00:04-00:24 Video Editor: please speed up

  - Then select a folder for storing the image and click Run to begin recording the images [1].

    - SCREEN: video 3: 00:25-00:44 Video Editor: please speed up

## Protocol Script Questions

A. Which steps from the protocol are the most important for viewers to see?

2.2., 2.3., 4.4.-4.8.

B. What is the single most difficult aspect of this procedure and what do you do to ensure success?

4.7.2., 4.8.1

# Results

- Results: Representative Capillary and Arteriole Imaging and Luminal Pressure and Atrial Diameter Assessment

  - When a fluorescence vascular marker is perfused into the vascular lumen [1], it is possible to visualize whole vascular trees using high-speed confocal microscopy [2].

    - LAB MEDIA: Figure 5 left image

    - LAB MEDIA: Figure 5 left image Video Editor: please emphasize vessels in image

  - Further magnification enables imaging of the capillary in detail [1].

    - LAB MEDIA: Figure 5 left image Video Editor: please emphasize/zoom into dashed box and show Figure 5 right image as magnification of image in dashed box

  - When pinacidil, an ATP-sensitive potassium channel agonist, is served from the lumen, the diameter of the arterioles increases [1].

    - LAB MEDIA: Video 1

  - When the vasoconstrictor endothelin-1 is applied from the lumen [1], the diameter of the arteriole decreases [2]. When the flow set is constant, the luminal pressure increases [3].

    - LAB MEDIA: Figure 6 Video Editor: please emphasize ET-1 image in Figure 6A

    - LAB MEDIA: Figure 6 Video Editor: please emphasize orange data line in Figure 6B

    - LAB MEDIA: Figure 6 Video Editor: please emphasize black data line in Figure 6C

  - In these images of papillary tissue from an NG2-DsRed (N-G-two-D-S-red)-expressing transgenic mouse [1], both the capillary [2] and pericytes can be visualized within the papillary muscle tissue [3] under conditions that better mimic the physiology of live animals [4].

    - LAB MEDIA: Figure 7

    - LAB MEDIA: Figure 7 Video Editor: please emphasize green signal in right Merge image

    - LAB MEDIA: Figure 7 Video Editor: please emphasize red signal in right Merge image

    - LAB MEDIA: Figure 7

# Conclusion

- Conclusion Interview Statements

  - Humberto C. Joca: It is important that no bubbles are present within the tubing at any point during the procedure [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera Suggested B-roll: 2.5

  - Guiling Zhao: Combining this method with optic imaging techniques, the preparation can be used to study microcirculation in the heart and intercellular communications between different cell types using fluorescence-tagged transgenic mice [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera