61583_screenshot_1

3.3. (Open the analysis software program named as “EKG”.) 00:00 – 00:10

3.3.1. (Click Setup | Channel Settings to set the Sample Rate to 2 k/s, the Range to 20 mV, and the Input Amplifier to 200 Hz of Low Pass. Click “OK”.) 00:11 – 00:40

3.3.2. (Click ECG Analysis | ECG Settings to choose “Mouse” in the Preset of Detection and Analysis settings. Click “OK”.) 00:41 – 00:50

3.3.3. (In the Averaging panel, choose to concatenate N (e.g., 60 s) consecutive cardiac cycles into a single average signal for Averaging View and Table View.) 00:51 - 00:55

3.3.4. (In the QTc panel, select “Bazett” method. Click “OK”.) 00:56 – 01:05

4.7. (Begin recording the ECG by clicking the “Start” button in the Chart View.) 01:06 – 01:11

*** We do not think that this 4.7. step needs a screen shot, but in case, we included it here. So depending on your opinion, please remove this shot if it is not necessary. ***